RAS Hub Functional Specification

# LAN Manager Remote Access Services 2.0

Connectivity & Systems Integration Group

Authors: Thomas J. Dimitri

Revision History

Revision	Date			Author		Comment

0.1		March 25, 1992		tommyd	Original

0.2		May 8, 1992		tommyd	Review Changes

Table of Contents

µ

Introduction	3

Overview	4

Meat of the Functional Specification	5

RasHub to RasMan Interface	5

RasHub to MAC Interface	8

How Endpoint Handles are Assigned	10

Simple IP and IPX routing	11

# Introduction

Description

The RAS Hub (RasHub) is an NDIS 3.0 device driver layered between protocols and MACs. The RasHub routes frames from MACs below it to protocols above it and route frames sent by the protocol to the proper MAC (and possibly that MACs corresponding endpoint or port).  The RasHub is not designed to work with just the AsyncMAC, it is designed to work with any NDIS 3.0 ethernet driver.  The RasHub also provides external Device IO Control interfaces for enumeration of the protocols above the RasHub, routing capabilities, statistics, and the ability to send and receive raw ethernet frames.  In general, the RasHub acts as an n-to-m multiplexor with a few extra pieces of functionality thrown in.  The RasHub also solves the 'binding' problem.  That is, most protocols and MACs only have to bind once with the RasHub even the MACs support several endpoints.  Endpoints are discriminated by liberal use of the ethernet 6 byte SRC field.

Organization of Document

This documents is ordered in the following format....

Intended Audience

The intended audience for this document is anyone who wishes to understand the NT RAS subsystem, theRasHub APIs, or how RasHub works.  Some knowledge of how the AsyncMAC works or how an NDIS 3.0 MAC may be written can be derived from this document as well.

Goals

The RasHub should be able to handle an unlimited number of endpoints if resources are unlimited.  The architecture should support simultaneous thread execution so that performance improves on a MultiProcessor platform (i.e. the architecture should not have one thread waiting on multiple events and thus serializing events at the thread level).  The RasHub should not expect the MAC to follow anything but the NDIS 3.0 guidelines - nor should assume anything special about the protocols above it.  Performance should be equal to or better than that of RAS 1.0 on OS/2 (with the same hardware).  The RasHub should port to "Cougar" with not much effort as all the code should be written in 'C' and conform to the NDIS 3.0 specification.

Related Components and Documents

The NT RAS Archiecture document should be read before reading this functional specification.  The NDIS 3.0 document describes the upper level interface AND lower level interface of the RasHub.  The AsyncMAC, RasMan DLL functional specifications are all related to the RasHub.

Document Conventions

The syntax ...

char[16]	NameOfProtocol  	is used in the document to mean...

char	NameOfProtocol[16]

# Overview

The RasHub is an NDIS 3.0 MAC driver wedged between protocols (NetBiosFrames, TCP/IP, IPX) and MACs (AsyncMAC, X.25 MACs, Ethernet MACs).  MACs only have one bind to the RasHub.  Protocols which can route such as TCP/IP and IPX also only have one binding.  Protocols which require gateways, such as NBF, require a binding per endpoint.

The RasHub has three major interfaces.  One is a device driver IOCtl interface to allow the RasManager to get hub information, set routes, and transceive data.  The second is the NDIS interface with the protocols.  The third is the NDIS interface with the MACs.

Through these interfaces the RasHub is able to route frames properly - whether it be to RasMan, to a MAC, or to a protocol.  The RasHub does not have any threads that it uses.  It uses the threads that pass through it.  However, it is not stateless.  It is prepared to work at DPC level for any Ndis call.

µ §

A protocol may send a frame to any MACs endpoint and a MAC may pass up a frame to any protocol.  This is what is meant by RasHub being an n-to-m multiplexor.  It accomplishes by playing wth the ethernet 6 byte SRC and DEST fields.  Please see the CDD for more information on how this is done.

In the figure above, there is one NDIS binding from the AsyncMAC to the RasHub.  One binding to the Ethernet MAC.  One binding to TCP/IP.  But multiple bindings to NetBEUI.

# Meat of the Functional Specification

### RasHub to RasMan Interface

RasHub to RasMan IOCtl APIs

RasHubEnum(OUT HubEnumBuffer, IN BufferSize)

RasHubRoute(IN EndpointHandle, IN ProtocolHandle);

RasHubGetStatistics(IN EndpointHandle, OUT ptr to NdisStatistcsBuffer);

RasHubMediaEnum(IN EndpointHandle, OUT ptr to MediaEnumBuffer);

RasHubProtocolEnum(OUT ProtocolEnumBuffer, IN BufferSize);

RasHubSendPacket(IN EndpointHandle, IN Packet, IN PacketFlags, IN Event);

RasHubRcvPacket(IN EndpointHandle, OUT Packet, OUT PacketFlags, IN Event);

HubEnumBuffer - a buffer which describes protocol bind time information.  It looks somthing like this but not exactly -- see structure below.

| Endpoint Handle | 6 byte Address | Link Speed | Quality Of Service | Mac Owner | Route type | Protocol Handle route | Bytes xmit and rcvd | Frames xmit and rcvd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | HUB001 | 9600 | V.42bis | AsyncMAC | <1500 | AsyBEUI | 3445 | 6 |
| 2 | HUB002 | 2400 | modem | AsyncMAC | IP | TCP/IP | 6777 | 5 |
| 3 | HUB003 | ---- | ----- | AsyncMAC | ----- | ----- | ----- | ----- |
| 4 | HUB004 | ---- | ----- | AsyncMAC | ----- | ----- | ----- | ----- |
| 5 | 334567 | 57000 | x.25 | EiconMAC | <1500 | AsyBEUI | 63454 | 54 |
| 6 | ----- | ----- | ----- | EiconMAC | ----- | ----- | ----- | ----- |
| 7 | 08c6fe311232 | 1000000 | ethernet | Elnkii | IPX | IPX | 4554 | 8 |
| 8 | ----- | ----- | ----- | Elnkii | ----- | ----- | ----- | ----- |
| 9 | ----- | ----- | ----- | Elnkii | ----- | ----- | ----- | ----- |

HubEnumBuffer struct {

ULONG		EndpointHandle;

BYTE[6]	IEEEAddress;

ULONG		LinkSpeed;

ULONG		QualityOfService;

CHAR[16]	MacName;

ULONG		MacBindingHandle;

ULONG		MediumType {Async, Ethernet, Token-ring};

ULONG		NumberOfRoutes;

USHORT	EthernetType;

ULONG		ProtocolRoutedTo;

ULONG		BytesXmitted;

ULONG		BytesRcvd;

ULONG		FramesXmitted;

ULONG		FrameRcvd;

USHORT	EthernetType;

ULONG		ProtocolRoutedTo;

ULONG		BytesXmitted;

ULONG		BytesRcvd;

ULONG		FramesXmitted;

ULONG		FrameRcvd;

USHORT	EthernetType;

ULONG		ProtocolRoutedTo;

ULONG		BytesXmitted;

ULONG		BytesRcvd;

ULONG		FramesXmitted;

ULONG		FrameRcvd;

}

NdisStatisticsBuffer - A structure containing specific information about the endpoints status.  This information is per endpoint (not per protocol route).  It is maintained by the MACs.  If the MAC is not of type Async, the information is for that whole MAC.  This is the best I can do with the NdisProtocol.  NOTE: HubEnumBuffer will report bytes and frames rcvd/xmitted on a per route basis if this information needs to be extracted.

NdisStatisticsBuffer struct {

ULONG			FramesXmittedOK;

ULONG			FramesRcvdOK;

ULONG			FramesXmittedBad;

ULONG			FramesRcvdBad;

ULONG			FramesMissedNoBuffer;

ULONG ASYNC_ONLY	SerialOverrunErrors;

ULONG ASYNC_ONLY	CRCErrors;

ULONG ASYNC_ONLY	TimeoutErrors;

ULONG	ASYNC_ONLY	FramingErrors;

ULONG	ASYNC_ONLY	BufferOverrunErrors;

ULONG	ASYNC_ONLY	BytesTransmittedUncompressed;

ULONG	ASYNC_ONLY	BytesReceivedUncompressed;

ULONG	ASYNC_ONLY	BytesTransmittedCompressed;

ULONG	ASYNC_ONLY	BytesReceivedCompressed;

}

MediaEnumBuffer - a pointer to a structure which describes the media DLL and the number of endpoints that media has.  This information is for RasManager only.

| Media DLL Name | Number of Endpoints |
| --- | --- |
| "SERIAL" | 4 |
| "SNA" | 3 |

MediaEnumBuffer struct {

USHORT	NumberOfMediaDevices;

CHAR[16]	MediaDllName;

USHORT	NumberOfEndpoints;

}

ProtocolEnumBuffer - A structure containing information about the protocols which have bound to the RasHub.  The lana number is derived from the Adapter Name which is prepared by setup.

| Protocol Handle | Protocol Type | Adapter Name | Ndis Protocol Handle | Binding Context |
| --- | --- | --- | --- | --- |
| 1 | AsyBEUI | "RAS01" | 3 | 3c84 |
| 2 | AsyBEUI | "RAS02" | 4 | 4cf5 |
| 3 | AsyBEUI | "RAS03" | 5 | 4de3 |
| 4 | AsyBEUI | "RAS04" | 6 | 5ff4 |
| 4 | IPX | "RASIPX" | 7 | 6145 |
| 5 | IP | "RASIP" | 8 | 7c67 |

ProtocolEnumBuffer struct {

ULONG		ProtocolHandle;

ULONG		ProtocolType;

CHAR[32]	AdapterName;

ULONG		NdisProtocolHandle;

ULONG		NdisBindingContext;

}

PacketFlags- a ULONG with just one bit set or not set to BROADCAST.  If set, the packet received was a broadcast frame, or it should be sent broadcasted.  Broadcast reception and transmission for ethernet type MACs will probably be done with a special multicast address (to avoid broadcast filter problems).  All other broadcasts will be ignored.

Packet - a pointer to the RAW buffer to send or receive.  This buffer DOES NOT contain any ethernet headers.  It simply contains data portion of the packet.  Use PacketFlags to determine if the packet is to be sent broadcast or directed and to determine if the packet received was sent broadcast or directed.

NOTE: The RasHub will only buffer ONE packet.  If another frame comes in, it will drop it so that a RasHubRcvPacket always gets the least recent packet sent.

NOTE: For MACs which do not support the ASYNC_MEDIUM type, the endpoint is 'dynamically' determined.  That is, the first frame which comes in for that MACs endpoint establishes the end to end communication address.  This scheme may not work, if so, I'll have to change it.

LinkSpeed is a 32 bit unsigned integer in bits per second.  This is the RAW speed not the optimal speed.  The LinkSpeed is passed up to real NDIS protocols so they can dynamically adjust their timeouts.

QualityOfService is a 32 bit unsigned integer.  Currently NDIS must better define what it wants from this field.  I suspect something like 0-raw mode, 1-V.42(bis), 2-X.25

### RasHub to MAC Interface

RasHub calls NdisRegisterProtocol to get the NdisProtocolHandle which it then uses when calling NdisOpenAdapter.  Then for each adapter it calls NdisOpenAdapter and passes in the NdisProtocolHandle and the ProtocolBindingContext (which the protocol generates).

RasHub is also a MAC.  It must call NdisRegisterMac to register the MAC to the NDIS interface and get back the NdisMacHandle.  Then call NdisRegisterAdapter once to get  the NdisAdapterHandle.  RasHub will be called with MacOpenAdapter when a protocol such as NBF wants to bind to it where it will be passed the NdisBindingContext.

NOTE: To support other MAC mediums as opposed to "async" and "ethernet", the RasHub must make additional MAC and adapter registrations so that it can register itself to look like a token-ring or FDDI MAC (so that the protocols aboce the RasHub can operate properly).  The current version of RasHub does not support these other "medium" types due to complexity and lack of need.

NdisIndicateStatus(NdisBindingContext, NDIS_STATUS_LINE_CHANGE, NDIS_STATUS_CONNECT) is used to signal 'ASYNC_MEDIUM' aware protocols that a connection has just occured.  The protocol should then ask for the QualityOfService and the LinkSpeed with the call below.

NdisQueryGlobalStatistics(MacAdapterContext, UINT Oid {ASYNC_MEDIUM | STATISTICS | OPTIONAL})

NdisIndicateStatus(NdisBindingContext, NDIS_STATUS_LINE_CHANGE, NDIS_STATUS_DISCONNECT) is used to signal 'ASYNC_MEDIUM' aware protocols that a disconnection has just occured and to 'tear down the link'.

The AsyncMAC to RasHub interface is limited by the NDIS interface, however, theNDIS interface has been extended to give RAS more functionality.  In general when the protocol wants to send a frame, RasHub issues an NdisSend.  When the frame has been completely sent, and NdisSendComplete is issued from the AsyncMAC to RasHub.  When a frame comes in, the AsyncMAC sends an NdisIndicateReceive frame to RasHub and when RasHub is done processing it, it issues an NdisIndicateReceiveComplete.  Typically, this is what happens when a connection exists, however when the AsyncMAC   signals the RasHub when connections and disconnections occur.

The following NDIS calls are used to pass information back and forth from the AsyncMAC to the RasHub.

Used to Get Serial Line Statistics

MacQueryGlobalStatistics(MacAdapterContext, NdisRequest {QueryInformation});

NdisCompleteQueryStatistics(NdisAdapterHandle, NdisRequest {QueryInformation});

Used to Request Connection Information

NdisRequest(&Status, NdisBindingHandle, RequestType{Query & Set Information});

NdisCompleteRequest(NdisBindingContext, Status);

Used to Indicate Line Status has Changed

NdisIndicateStatus(NdisBindingContext, GeneralStatus, SpecificStatus);

NdisIndicateStatusComplete(NdisBindingContext);

For instance if the RasHub wants serial statistics, it will call MacQueryGlobalStatistics with the appropriate fields set.  In turn, the AsyncMAC will return serial line statistics (such as timeouts, framing errors, overruns, byte transmitted).  When an AsyncMacOpen or AsyncMacClose is called, the AsyncMAC sends an NdisIndicateStatus to RasHub.  If the line is connected (i.e. AsyncMacOpen) then RasHub calls back with NdisRequest to get the LinkSpeed and the QualityOfConnection.  The structures used to pass information is not yet defined (waiting on Adam Barr's implementation of AsyBEUI).  The NdisRequest fields looks like this...

typedef struct	NdisRequest {

UINT		RequestType;

INFO_FIELD	Data;

}

typedef union	INFO_FIELD {

SET_INFORMATION,

QUERY_INFORMATION

};

typedef struct	SET_INFORMATION {

UINT		Oid;

PCHAR		InformationBuffer;

ULONG		InformationBufferLength;

};

typedef struct	QUERY_INFORMATION {

UINT		Oid;

PCHAR		InformationBuffer;

ULONG		InformationBufferLength;

};

For RAS, the Oid field would be changed to indicate a specific RAS request - such as a telling how many endpoints the AsyncMAC has.   Remember, an endpoint can be another ethernet card on the LAN, a COM port, an X.25 port, an SNA LU 6.1 endpoint, the D line in ISDN, or any various communications paths which include only one machine.

At binding time, the RasHub driver tells the AsyncMAC how many endpoints it must prepare to support.  At this point the AsyncMAC can go ahead and allocate non-paged pool structures for all the endpoints.  When RasHub binds to the AsyncMAC, the AsyncMAC uses a special type of medium.  If this special ASYNC_MEDIUM is used, then the RasHub uses NdisRequest with SetInformation to tell the AsyncMAC how many endpoints it has.  Once again these structures have yet to be completely defined.µ	§

# How Endpoint Handles are Assigned

When RasHub loads, it reads information from the Registry.  The RasHub registry section is written by the SETUP program.  The RasHub registry information may look like this...

[RasHub]

Binding=AsyncMAC

Media=serial

Endpoints=10

Media=x.25

Endpoints=4

Binding=SNA

Media=SNA

Endpoints=3

The RasHub will assign endpoint in order of bindings, a typical handle assignment for this would like....

µ §

NOTE: The AsyncMAC matches the Endpoint Handle with the serial driver port number from information passed by RasMan to the AsyncMAC.  The RasHub cannot determine how many different serial drivers the AsyncMAC has.

# Simple IP and IPX routing

Simple IP and IPX could be performed by using special "routable only" protocol stacks.  These "routable only" stacks would simply take frames, swap the necessary fields around and pass them back down through RasHub.  In this scenario, RasHub could perform as a very router.  Perhaps even cheaper and faster than commerical hardware routers.  The diagram below shows the frame path.

µ §

As it turns out, it looks like multiple bindings will exist from the RasHub to the IP and IPX protocols.  That is, each binding will generate "subnet" so that true routing can be done.  "True" routine implies that client 1 can dial-in and see client 2 (who has also dialed in) becuase the IP or IPX stack will route it.