# :

Ben Goetter
NBU Development, Thor
User Interface Architecture

Revision 0.0

# 1	Overview

## 2	Definition of component

Memory management strategies for our excellent THOR UI.

## 3	References

LM 3.0 UI C++ Coding Style Guidelines.  Ben Goetter.

## 4	Design overview

### Problem description

The user interface is based on Windows 3.0, Enhanced 386 mode; approximately half of the components are implemented in Visual BASIC, while the remainder are native-coded Windows DLLs which extend the baseline user shells.  Coding will take place in VBASIC, Glockenspiel C++ 2.0, and Microsoft C 6.0a, with the majority of the coding being done in C++.

Clients of Thor UI memory management need to create and destroy objects at will, to amass large pools of such objects (large meaning "having storage which totals greater than 64K"), and to manipulate objects directly with pointers.  In short, they need to engage in traditional C++ memory management, bypassing the traditional Windows double indirection.

While most clients will need simple new/delete functionality, which operates within a pool global to the application, some clients want theirclasses to maintain their storage in a heap separate from that used in the rest of the app.  Also, some classes must support a Resize method to change their size; this surpasses the simple new/delete model.  Such variations need to be selectable from within the client class's implementation.

### Design objectives

Ease of use: the memory management scheme will be easily accessible by the client, and should not impose odious restrictions on user code.

Ease of maintenance: the scheme will be easily tunable and extensible, and will encourage the instrumentation of client code to facilitate this tuning.

Performance: the scheme won't significantly degrade performance beyond those limits already imposed by the pre-existing architecture.  That is, it will not suffer when compared with a vanilla Visual BASIC application of equal dimension.

### Design constraints

Glockenspiel C++ 2.0 only supports the Microsoft C "Large" memory model.

Windows 3.0 applications may have only a single writable data segment, if the application is to support multiple instances of itself correctly.

All extension code called from VBASIC must reside in a dynamic-link library.  Likewise, extensions to the shell must reside in a DLL.

All DLLs must support multiple clients and multiple instances of clients.  A Windows DLL has but a single data segment, to which it switches whenever a client calls into it; thus it must take care to maintain only true global state in this data segment, and to remain re-entrant.

# 5	Development tools, platforms, and environment

All as mandated elsewhere at tiresome length.

# 6	Detailed description of operation

## 7	Environment

## 8	Multiple instances

## 9	Synchronization/locking

## 10	Alternatives considered and tradeoffs

## 11	Global optimisation issues

# 12	Modular decomposition and tasking

## 13	Process/thread/module structure

A Thor user-interface application consists of three logical sections: the user-interface layer APPUI, which contains the Windows message loop; the tasking layer APPTASK, into which the UI layer calls, and which performs the network manipulation; and the core layer UICORE, which contains the utility code common to all Thor UI components.  These UI applications come in two varieties: stand-alone application, and shell extension.  Stand-alone applications keep their message loop in a Thunder application which performs all user-interface activity.  Shell extensions keep their message loop in their client application, since they are extension services called by another app; they may contain window and dialog definitions of their own, where the facilities provided by the client do not suffice.

Thor UI memory management is DLL-side, since the components using its services will always be DLLs: the stand-alone applications keep their message loop in a Thunder (VBASIC) application, which will use the native VBASIC memory management facilities, while the shell-integration applications do not have control over the "application" component proper.  It must therefore take special care to remain re-entrant.

This diagram illustrates two Thor applications, one of which has two instances open (perhaps focused on different domains).

App instance #1
DS(1), SS(1)

...

App instance #n
DS(n), SS(n)

Instance of another app
DS(q), SS(q)

Tasking DLL for App
DS(alpha), SS(x:1..n)

Another tasking DLL
DS(gamma), SS(x)

UICore kernel DLL: DS(beta), SS(x)

Example application DLL structure

All open instances of an application share a single copy of that app's tasking DLL; all Thor applications in the system share one copy of the UICore kernel DLL.  Each module has a unique DS, but shares its SS with the DLLs from which it requests services ("down", in the figure above).

All of the code described in this document lies in UICORE (the kernel DLL); every "client" mentioned is an APPTASK (tasking DLL) component for some Thor application.

## 14	Segment structure

Every Thor application DLL, both tasking and kernel, has a single writeable data segment _DATA; any number of read-only, constant data segments x5_CONST; at least one dynamically allocated segment for the per-instance heap; and any number of code segments x_CODE.

The writable data segments, both static and dynamic, are fixed.  This is acceptable, since Thor only runs on the 386 Enhanced mode of Windows.

## 15	Object structure

The UICORE module will contain one CLIENT object, plus a HEAP object per application instance (each referenced by a CLIENTREC within the CLIENT).  Each heap will contain at least one SUBHEAP.

# 16	Major data structures and functions

## 17	Giant heaps (HEAP)

We require tasks to be able to allocate greater than 64Kbytes of data apiece.  Given that a Windows "local" heap is limited to 64Kbytes maximum size, and given that a task should not have to partition its dynamically-allocated storage, the memory manager forms a "Giant heap" for each requesting task from multiple lesser heaps.

### Private data

A giant heap initially consists of a singleton collection of subheaps.  As each subheap is exhausted, a new subheap is added to the giant heap in order to satisfy the client task's allocation request.  Iterations over the collection are ordered by the age of the subheap, with the oldest subheaps appearing first; this will yield good behavior where allocations follow roughly a LIFO pattern, so that, upon all recent allocations being freed, the newer subheaps can themselves be released back to the system.

COLL_OF(SUBHEAP) collshp;

A subheap gathers a handle to an allocated data segment, which has been initialized for suballocation as a local heap, together with some associated information about its status within the giant heap.  Each subheap keeps a record of the smallest allocation request against it which failed, so as to let further attempts to allocate objects as large elide that subheap; this field is reset upon any item being returned to the subheap, since a compaction (or rather, a merging of adjacent blocks - compaction is unlikely, since all blocks are locked down) might then make sufficient space available.  Each also maintains a count of outstanding allocations, so that at cleanup time the giantheap can return any empty subheaps back to global memory.

HANDLE hDS;		// handle to data segment (selector)
USHORT cbFailed;	// smallest request against heap which failed
USHORT cUses;	// number of allocs active

### Private methods

From the collection, locate the first such subheap which can satisfy the outstanding request, and from it allocate a block.  It iterates over the collection, ignoring any subheap which has failed a request of the specified size or less, and attempting allocation against the others, until it either successfully allocates the block or else has visited each subheap without success.  Return a pointer to the allocated memory, or NULL upon failure.

PBYTE HEAP::AllocFromCollection( USHORT cbRequested );

Add a new subheap to the collection; from it, allocate a block.  Return a pointer to the allocated memory, or NULL upon failure.

PBYTE HEAP::AllocFromNewSubheap( USHORT cbRequested );

Given a pointer to a memory block, determine the subheap of its origin.  Return NULL upon failure.

SUBHEAP * HEAP::SubheapOfOrigin( PBYTE pbDarwin );

Attempt to allocate a block from a subheap; if the allocation fails, mark the subheap as having failed, and return NULL; otherwise, increment its use counter, and return a pointer to the allocated memory.

PBYTE SUBHEAP::GetBlock( USHORT cbRequested );

Return a block to a subheap, clearing any Failed flag if set, and deleting its use counter.

VOID SUBHEAP::PutBlock( PBYTE pbVictim );

Create a new subheap.

SUBHEAP::SUBHEAP( USHORT cbInitial );

### Public methods

Create a new giant heap.

HEAP::HEAP( USHORT cbInitial = CB_DEFAULT_HEAP_INIT );

Destroy the giant heap.

HEAP::~HEAP();

Given the desired size of the memory block, return a pointer to a newly allocated block of at least that size, or NULL if sufficient memory could not be found.

PBYTE HEAP::Allocate( USHORT cbRequested );

Given a pointer to a memory block and its new desired size, resize that block and return a pointer to the newly sized block, or NULL if the resize failed.

PBYTE HEAP::Reallocate(PBYTE pbOriginal, USHORT cbRequested );

Given a pointer to a memory block, return it to the heap.

VOID HEAP::Deallocate( PBYTE pbThing );

Expunge empty subheaps from the collection.  Iterate over the collection; delete each empty heap found; compact nonempty heaps.  This should be called upon return from some alloc-intensive activity in the app.

VOID HEAP::Expunge();

## 18	Task association (CLIENT)

Each client task will expect to have its own heap.  Since the DLL will be shared between every client task, it cannot simply keep the heap's identifying handle in its DS; rather, it must somehow associate the heap with the task, so that upon entry into the DLL it can determine which heap the client needs.

### Private data

A client record associates a single client task with its heap.

HANDLE hTask;
HEAP * php;

The task/heap association-list contains a record for every client task of the DLL.  It is indexed on the task handle, which comes from the GetCurrentTask() API.

COLL_OF(CLIENTREC) collclr;

### Public methods

Register a new client, creating a heap for it.

HEAP * CLIENT::Register();

Deregister a terminating client and destroy its heap.

VOID CLIENT::Deregister();

Return a pointer to this client's heap.

HEAP * CLIENT::QueryHeap();

## 19	Global heap allocation (::)

The global allocation functions new and delete work on the client's global heap, mapping to Allocate and Deallocate methods, respectively.

extern void *     ::operator new( size_t size );
extern void       ::operator delete( void * p );

## 20	Resizable object (BUFFER)

A buffer is a piece of memory which the client can resize at request.

### Private data

A buffer consists of a pointer to the actual block of memory and a count of the bytes therein.  The class keeps these ivars private, providing access through public access methods.

BYTE * pbData;	// data in the buffer
USHORT cbData;	// size of allocated memory area

### Public methods

The object's constructor takes the desired size of the buffer as its only argument.  It allocates the actual data area from the application global heap.

BUFFER::BUFFER( USHORT cbRequested );

Return the number of bytes in the buffer.

USHORT BUFFER::QueryCount();

Return a pointer to the data in the buffer.

BYTE * BUFFER::QueryData();

Resize the buffer.  Returns a pointer to the data in the buffer, which may have moved.

BYTE * BUFFER::Resize( USHORT cbRequested );

## 21	Resizable large object (BIGBUFFER)

A  BIGBUFFER is simply a buffer which the client expects will grow to great size over time.  Unlike a BUFFER or other allocated object, it lives in its own piece of global memory.  It therefore uses the Windows GMEM APIs directly, instead of the HEAP objects described above.

A BIGBUFFER is currently limited to 64K in order to allow a conventional far-pointer to address its memory.

### Private data

The object consists of a pointer to the actual block of global memory and a count of the bytes therein.  As does BUFFER, the class uses public methods to provide access to these.

BYTE * pbData;	// data in the buffer
USHORT cbData;	// size of allocated memory area

### Public methods

The object's constructor takes the desired size of the buffer as its only argument.  It allocates the actual data area from the system's global memory region.

BIGBUFFER::BIGBUFFER( USHORT cbRequested );

Return the number of bytes in the buffer.

USHORT BIGBUFFER::QueryCount();

Return a pointer to the data in the buffer.

BYTE * BIGBUFFER::QueryData();

Resize the buffer.  Returns a pointer to the data in the buffer, which may have moved.

BYTE * BIGBUFFER::Resize( USHORT cbRequested );

## 22	Private heap allocation (HAS_PRIVATEHEAP)

The design for private heaps remains unfinished; I'm not certain that we need them, and other areas of the design are more pressing anyway.  Should somebody prove its need, I may complete the design.

A class may use a private heap for its allocations.  Such a class

A client makes a class use a private heap by making it inherit from HAS_PRIVATEHEAP.

### Private data

Any instance of a private heap will contain its own task-heap associator (CLIENT) as a static object.

static CLIENT clientAssociator;

# 23	Internal interfaces

## 24	Inter-component

The tasking DLL (probably a TASKLIB object) must perform the registration (CLIENT::Register) and deregistration.  Details later.

## 25	Intra-component

# 26	External interfaces and dependencies

## 27	Exported interfaces

extern void * ::operator new( size_t size );
extern void   ::operator delete( void * p );

class BUFFER: public BASE
{
private:
    BYTE *  pbData;
    USHORT  cbData;
public:
    BUFFER( USHORT cbRequested );
    ~BUFFER();

    USHORT QueryCount() const;
    BYTE * QueryData() const;

    BYTE * Resize( USHORT cbRequested );
};

class BIGBUFFER: public BUFFER
{
public:
    BIGBUFFER( USHORT cbRequested );
    ~BIGBUFFER();

    BYTE * Resize( USHORT cbRequested );
};

## 28	Imported interfaces

class TASK;

## 29	Dependencies

class TASK;  // maybe...

# 30	Performance issues

## 31	Performance aims

## 32	Size

# 33	Work effort estimates

## 34	Major task list

# 35	Testing considerations

## 36	Instrumentation, test hooks

## 37	Desirable test utilities

## 38	Unit tests

## 39	Expected problem areas

### System test

### Integration test

### Boundary conditions

### Memory related

### System/subsystem constraints

40	Others

This is from the requirements document.  At present, no portion of the memory management design explicitly requires enhanced mode, though the two proposed alternative implementations of the HEAP object each do depend on it.

This gives us lots of margin!

Hence, no Thor application may have more than 64K of static, non-CONST data.

Alternate strategies are possible.  A giant heap could consist of all of global memory, handing out pieces from GlobalAlloc calls upon request.  Under Windows 3.0, however, global selectors are a precious resource, since every task shares the same descriptor table; having each allocated object occupy a separate global segment would quickly exhaust all available selectors.  A second possibility involves allocating global memory heaps of greater than 64Kbytes, suballocating them with huge pointers, and returning suballocated pieces, possibly casting the huge pointer result to a far pointer; however, this would rule out the use of the local heap manager as a suballocator, and so would necessitate the writing of a new such package.  A third possibility, available only under 386 Enhanced Mode, involves using WIN32MEM to allocate large pieces of 32-bit memory as a giant heap, returning suballocations which have been aliased to 16:16 pointers; this too would necessitate writing a package for suballocation.

Should allocations not be LIFO, this collection might be better indexed on the size of its last failure, so that initial candidates for allocation could be easily located.

This will set DS to SUBHEAP::hDS, so that the Local memory manager will work.  It will restore DS to that of the DLL before returning.  All functions which call the l.m.m. must temporarily set DS thus in one way or another.

The TASK object will call this before it terminates, so actual clients should never have to bother with it, unless they wish to shrink the heap while they still have control.

Alternately, we could just use the SS of the client task as its identifier, as does the Laser Demi-layer.

Obviously, a client shouldn't keep too many of these around at once.  Use a BUFFER instead where possible.