# Microsoft Windows NT 3.1

# Remote Access Service

# Wide Area Networking

Revision 0.3 May 1993

Table of Contents

µ1 Overview	3

2 Architectural Overview	4

2.1 RAS Wide Area Subsystem	4

3 Implementing DLLs and NDIS drivers for ISDN adapters	5

3.1 Implementing Device/Media DLLs	5

3.2 Implementing NDIS 3.0 MAC Driver	6

4 Implementing Drivers for Serial Adapters and X.25 Adapters	6

5 Installation Considerations	7

5.1 Installing ISDN Drivers	7

5.1.1 ISDN Channel Aggregation on Workstations	8

5.1.2 ISDN Channel Aggregation on Servers	8

5.1.3 ISDN Installation Details	9

5.2 Installing Serial and X.25 Drivers	10

6 Media DLL Specification	11

6.1 RAS Manager and Media DLLs	12

6.2 Functional Role	13

6.3 Media DLL APIs	14

6.4 Installation	32

7 Device DLL Specification	32

7.1 RAS Manager and Device DLLs	33

7.2 Functional Role	34

7.3 Device DLL APIs	35

7.4 Installation	45

# 1 Overview

Windows NT Remote Access Service enables remote workstations to dial in to the LAN (Local Area Network) and transparently access resources on the LAN. The Windows NT Remote Access Service architecture is designed to enable easy plug-in of different Wide Area Networking cards.  Windows NT 3.1 Remote Access supports three types of Wide Area networks:

1.	PSTN (Public Switched Telephone Networks)

2.	X.25 Networks.

3.	ISDN (Integrated Services Digital Networks)

This document provides an overview of the Windows NT Remote Access service WAN (Wide Area Network) architecture and identifies key components that IHVs (Independent Hardware Vendors) need to develop in order to enable their products to work with Windows NT.

µ §

Figure 1.1

# 2 Architectural Overview

The Remote Access Service can be categorized into three main groups:

·	Remote Access processes:  Phone book interface,  Server

·	Remote Access APIs: Dial up programming interfaces for mobile work group applications.

·	Wide Area Subsystem:  Remote Access DLLs, NDIS drivers, Serial drivers.

This document focuses on the Wide Area subsystem for Remote Access

## 2.1 RAS Wide Area Subsystem

The main goal of the RAS Wide Area subsystem is to provide an extensible framework so that Serial, ISDN and X.25 drivers can be easily added to Windows NT.  Remote Access sees the Wide Area network as a combination of two things:

1.	A type of Port (Media)

2.	One or more devices connected to the Port.

By using a concept of a port and multiple devices connected to that port (one after the other), Remote Access is able to execute multi-stage dial up.  For example, in the case of workstation that needs to connect to the LAN via a dial up X.25 PAD, Remote Access sees the Wide Area Network as consisting of 1 Port and 2 devices (one modem and one X.25 PAD).  The connection sequence would consist of the following steps:

1.	Open the Serial Port (media)

2.	Communicate with modem(device 1).  Initialize the modem and dial

3.	Communicate with X.25 PAD (device 2). Provide X.25 PAD with necessary parameters to traverse the X.25 network and connect to the Remote Access server.

µ §

Figure 2.1

Note:  For ISDN cards, the distinction between the Port and the first device is blurred.  In other words, an ISDN card provides an ISDN port as well as an ISDN device attached to that port.  In this case, you may choose to combine the media and device DLLs into one unit (DLL), rather than provide two separate DLLs.  This is OK as long as the interface functionality for the combined module is preserved.

# 3 Implementing DLLs and NDIS drivers for ISDN adapters

Due to the advanced functionality of ISDN cards (e.g. channel aggregation) ISDN vendors need to write Remote Access DLLs and NDIS drivers.

µ §

Figure 3.1

## 3.1 Implementing Device/Media DLLs

Refer to the Functional Specifications (Chapters 6 and 7 of this document) for the Device and Media DLLs for a detailed description of the Device and Media DLL interface. Header files RASMAN.H, DEVICE.H and MEDIA.H are needed for implementing this interface. In addition header file ISDN.H contains defines specific to ISDN.

## 3.2 Implementing NDIS 3.0 MAC Driver

Refer to the NDIS 3.0 documentation for implementing the MAC driver.

Note: The address format passed up in the NDIS_WAN_LINE_UP structure when WAN_LINE_UP indication is given must contain the Remote address (6 bytes), Local address (6 bytes) and the endpoint (2 bytes). The endpoint is passed to the DLLs in the PortConnectComplete() call. The structure therefore looks like this:

typedef struct _NDIS_WAN_LINE_UP {

ULONG 	LinkSpeed;

ULONG 	MaximumTotalSize;

NDIS_WAN_QUALITY Quality;

USHORT	SendWindow;

UCHAR 	RemoteAddress[6];

UCHAR 	LocalAddress[6];

USHORT	Endpoint;

} NDIS_WAN_LINE_UP, *PNDIS_WAN_LINE_UP;

In addition, the Remote and Local addresses must be unique for every endpoint.

# 4 Implementing Drivers for Serial Adapters and X.25 Adapters

Remote Access provides a serial NDIS driver (AsyncMac) as well as the necessary media and device drivers for serial interface connection management.  The Remote Access uses the Windows NT serial driver interface for connection management over PSTN and X.25 networks.

µ §

Figure 4.1

Special implications for X.25 cards:

·	Each virtual circuit on the X.25 card appears as a Serial Port (with an X.25 PAD attached to it).

·	The connection plane must correctly emulate serial port signals such as DTR, DCD, CTS, RTS, and DSR.

·	The data plane has no performance penalty.  Once a connection has been established, Remote Access reads and writes from the serial driver in 1500 byte data blocks.  Data transfer is not implemented one character at a time.

·	X.25 vendors will need to make an entry for their PAD in the PAD.INF file. This contains the command/response script required to make a connection through the X25 PAD.  See the Remote Access Administrator Guide for details on the PAD.INF file.

# 5 Installation Considerations

It is the responsibility of the hardware vendor to install the appropriate device drivers and DLLs.  The following sections describe the installation configuration expected for Serial, X.25 and ISDN adapters.

Installation of WAN adapters for Remote Access occurs in two phases:

1.	User installs WAN adapter using vendor specific setup program

2.	User configures Remote Access to use the installed adapter.

The Remote Access configuration utility reads the Windows NT HKEY_LOCAL_MACHINE\HARDWARE\DEVICEMAP registery key in order to find available ports.

Note:

In order to make your ports immediately visible to the Remote Access setup program, you should write your port list to the HKEY_LOCAL_MACHINE\HARDWARE\DEVICEMAP registry key as soon as port configuration is completed.

Furthermore, it is strongly recommended that your setup program prevent port name contention by reading the HKEY_LOCAL_MACHINE\HARDWARE\DEVICEMAP registry key prior to port configuration.  Your setup program can then prevent the user from selecting a port name that is already in use.  For example, if COM1 is already registered in the devicemap key, do not allow the user to select the name "COM1" for subsequent ports.

Remote Access setup writes RAS media specific configuration information to Windows NT Registry.  This information allows the media and device DLLs to determine the following:

·	Number of available ports that have been configured for use by Remote Access

·	Exactly what functionality has been configured for each port (Dial out only, Receive calls only, Dial out and receive calls)

## 5.1 Installing ISDN Drivers

This section provides information on how to install an ISDN driver. A brief description of how channel aggregation is implemented with Remote Access is provided to give you some context for how ISDN ports should be installed.

### 5.1.1 ISDN Channel Aggregation on Workstations

The objective of the channel aggregation implementation for workstations is to allow power users to implement channel aggregation, while keeping an easy-to-use interface for non-power users.

In order to address the needs of users who need large bandwidth communications, one or more ISDN BRI (Basic Rate Interface) cards may be configured for channel aggregation. In other words, the ISDN vendor may choose to allow a user to install 2 cards and allow channel aggregation across the two cards (to give a total of 4 channels). However, in order to keep the user interface simple, the ISDN setup program must allow the user to configure these two cards such that they appear as one port to the Remote Access Service.

The ISDN driver is further responsible for implementing fallback as necessary. The ISDN adapter may choose to allow fallback to occur on a per channel basis. For example, allow fallback sequence that might result in a 64K Digital and 56K Voice circuit aggregated together.

The Remote Access workstations user interface will allow the user to select the following ISDN parameters:

| Parameter | Values | Default |
| --- | --- | --- |
| Line Type | 64K Digital, 56K Digital, 56K Voice | 64K Digital |
| Fallback | Yes, No | Yes |
| Enable Compression | Yes, No | Yes |
| Number of Channels | 1, ... n | 1 |
| Phone Number | <number> | N/A |

For channel aggregation on networks where a unique telephone number is required per channel, the user may select multiple phone numbers by inserting a colon (:) between each number.  For example, if the user wishes to aggregate 4 channels (across two ISDN adapters), and the user's ISDN network does not allow for one telephone number across channels, then the user might input four numbers as follows: v:x:y:z, where v,x,y, and z are the unique phone numbers for each channel.

It is the responsibility of the ISDN DLLs to parse this number (via colon separators) and assign the various telephone numbers to each channel.

### 5.1.2 ISDN Channel Aggregation on Servers

The objective of channel aggregation on the server side is to allow the Remote Access server to accommodate as many ports as possible, while allowing for dynamically aggregating some of those ports as needed by power users.

In order to allow as many ports as possible to be serviced by the Remote Access server, the ISDN setup program must allow the user to configure individual channels as unique ports. In other words, the ISDN setup program should allow a user to install 2 cards and configure them such that each BRI channel appears as a unique RAS port (resulting in 4 ports).

In order to allow for dynamic channel aggregation as requested by workstations, the ISDN driver must provide a state that enables the Remote Access server to determine when a port has been aggregated with one of more other ports (and is no longer available for a new incoming call).

The Remote Access setup program will allow the user to select the following ISDN parameters for each ISDN port that the RAS setup program detects in the Windows NT device map:

| Parameter | Values | Default |
| --- | --- | --- |
| Line Type | 64K Digital, 56K Digital, 56K Voice | 64K Digital |
| Fallback | Yes, No | Yes |
| Enable Compression | Yes, No | Yes |

Notice that unlike the case of the RAS workstation user interface, the RAS setup program does not allow the user to select the number of channels.  This is because the setup program configuration screen is primarily used for server side port configuration, and as described above, server side configuration assumes only one port per channel (with dynamic aggregation as needed).  Server side channel aggregation is not static as is the case with workstation channel aggregation.

### 5.1.3 ISDN Installation Details

Install as ISDNn - ISDNm.  Deposit information in Windows NT registry.  You will need to write a configuration utility that installs your driver.  Microsoft recommends that you package your configuration utility as a DLL which can be invoked from the Network control panel via a setup INF file that you write. Users can install your hardware and drivers by starting the Network Control Panel, clicking the "Add Adapter" button and installing your driver.

Windows NT Registry:

Information stored here should point to the appropriate ISDN DLLs, the names of the ISDN ports, and the Name of the ISDN NDIS driver.

Remote Access will add a parameter to each ISDN subsection of the Remote Access setup section identifying whether a particular ISDN port has been selected to be used by RAS.  This allows the ISDN DLLs to determine how many ports to present to the RAS Server or client software.

Remote Access setup expects the per port information in the registry as defined below:

HKEY_LOCAL_MACHINE\HARDWARE\ DEVICEMAP\ ISDNPORTS

Port0

Name  		REG_SZ   ISDN1

Manufacturer    	REG_SZ   ISDN Card Name

Driver    		REG_SZ	Driver Name

Type		REG_SZ	Media Type            (This should be the same as the corresponding DLL name)

.

.

Portn

Name  		REG_SZ   ISDNn+1

Manufacturer    	REG_SZ   ISDN Card Name

Driver    		REG_SZ	Driver Name

Type		REG_SZ	Dll Name	              (This should be the same as the corresponding DLL name)

After the user completes configuring Remote Access, the following entries will be available in the registry for the ISDN drivers to use:

HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\RAS\MEDIA\ISDN

ISDN1

Usage          	REG_SZ   	Client/Server/ClientAndServer

LineType     	REG_SZ   	64K Digital/56K Digital/56K Voice

FallBack      	REG_SZ    	Yes/No

Compression	REG_SZ  	Yes/No

.

ISDNn

Usage          	REG_SZ   	Client/Server/ClientAndServer

LineType     	REG_SZ   	64K Digital/56K Digital/56K Voice

FallBack      	REG_SZ    	Yes/No

Compression	REG_SZ  	Yes/No

## 5.2 Installing Serial and X.25 Drivers

Install as COMn - COMm.  Deposit information in Windows NT registry.  You will need to write a configuration utility that installs your driver.  Microsoft recommends that you package your configuration utility as a DLL which can be invoked from the Network control panel via a setup INF file that you write. Your customers can install your hardware and drivers by starting the Network Control Panel, clicking the "Add Adapter" button and installing your driver.

Windows NT Registry

Indicate what COM ports the driver exposes to the system (e.g. COM 10)

If necessary, merge [X.25 card Name] script section with Remote Access PAD.INF script file located in (windows NT directory)\system32\ras.  See the Remote Access Service Administrators Guide for details on how to write a PAD.INF script.

# 6 Media DLL Specification

The Media DLLs are used by the RAS Manager component to allow connection establishment over different Media: Serial, X.25, ISDN, etc. The knowledge of configuring, opening, closing, of Ports of a certain Media is encapsulated within the Media DLL for that Media. The Media DLLs are attached to at the time RAS Manager loads; at this time RAS Manager gets all the entry points in the DLLs. All Port related RAS Manager APIs translate to some Media DLL API. The Media DLLs also hide Media specific requirements from RAS Manager; for example, for Win32 ISDN connections, any threads required will live inside ISDN.DLL without any knowledge of RAS Manager.

This chapter describes where the MEDIA.DLL component fits in the RAS Architecture. MEDIA.DLL functional definition, exported APIs, and the data structures relevant to these APIs are described.

Goals

The main motivation for having independent Media DLLs is to contain all the knowledge relevant to making connections over a Media in a Media DLL. New Media types can be added to RAS by writing new Media DLLs for them.

Relevance

This chapter will be of interest to developers who want to write Media DLLs to support new media types with RAS. An understanding of APIs, DLLs, and Synchronization mechanisms for Win32 is necessary for this document.

## 6.1 RAS Manager and Media DLLs

Figure 6.1 describes how Media DLLs fit into the RAS Manager architecture. RAS Manager APIs are independent of the actual Media that the connection takes place over. Based on the Media type of the Port being used, RAS Manager simply routes the request to the appropriate Media DLL.

µ §

Figure 6.1

µ

§

## 6.2 Functional Role

The different functions performed by Media DLLs are:

1.	Exporting the specified APIs to be used by RAS Manager.

2.	Parsing any Information files relevant to the Media.

3. 	Configuring the Ports.

4. 	Communicating with the Media specific MACs.

1. Exporting APIs.

RAS Manager uses Media DLLs through the APIs exported. Each Media DLL must conform to the exact specification of the APIs. In this case the APIs must return as soon as possible; for the Disconnect case (which may take long) the DLL should associate an event (or callback) and return as soon as possible.

2. Parsing Media specific Information files.

Media DLLs are responsible for parsing any vendor specific configuration files that are needed for the Media type. Through this, the structure of the files, and knowledge of Media parameters are localized to the Media DLL itself.

3. Configuring Ports.

The Media DLLs are responsible for configuring ports belonging to that Media using the appropriate APIs (e.g., Win32 Comm APIs for Serial Ports)

4. Communicating with the Media specific MACs.

The communication between the Media DLL and a MAC for the corresponding Media is encapsulated in the Media DLL itself. This includes passing the Port handles to the MAC so that it can write directly to the Port etc. Other mechanisms (e.g., MAC passing frames into User mode so that it can be sent using vendor specific APIs) are also hidden inside the Media DLL.

## 6.3 Media DLL APIs

DWORD	PortEnum (

IN		BYTE *		pBuffer,

BOTH		WORD *	pwSize,

OUT		WORD *	pwNumPorts

)

Parameters:

pBuffer	- Pointer to buffer to receive array of PortMediaInfo structures.

pwSize	- Size of the buffer provided. On return, size of information returned.

pwNumPorts	- Number of PortMediaInfo structures returned.

Return Value:

SUCCESS

ERR_EMPTY_INI_FILE

ERR_BUFFER_TOO_SMALL

ERROR_READING_SECTIONNAME

ERROR_READING_DEVICETYPE

ERROR_READING_DEVICENAME

ERROR_READING_USAGE

ERROR_BAD_USAGE_IN_INI_FILE

Description:

This API is used to get information about all the ports configured for the Media. If the buffer is not big enough, the size of the buffer required is returned. The call is synchronous.

DWORD PortOpen (

IN		CHAR *		pszPortName,

OUT		HANDLE *	phIOPort,

IN		HANDLE	hNotify

)

Parameters:

pszPortName	- Pointer to name of port to open.

phIOPort		- File handle for the Port returned on successful opening of port.

hNotify		- Event handle used to notify user that the link has dropped or that there has

been a hardware failure of sorts.

Return Value:

SUCCESS

ERROR_PORT_NOT_CONFIGURED

ERROR_DEVICE_NOT_READY

Description:

This API is used to open a port. The name of the Port is passed in with the call; on return the file handle associated with the open port is returned, if successful. The hNotify event is used until the port is closed to notify a link dropping or a hardware/power failure.

DWORD PortClose (

IN		HANDLE	hIOPort

)

Parameters:

hIOPort	- Port handle to close.

Return Value:

SUCCESS

Win32 error codes returned by GetLastError()

Description:

This API is used to close a port. The handle of the Port is passed in with the call.

DWORD PortGetInfo (

IN		HANDLE	hIOPort,

IN		CHAR *		pzsPortName,

OUT		BYTE *		pBuffer,

BOTH		WORD *	pwSize

)

Parameters:

hIOPort 	- Port handle.

pszPortName- Name of the port ("COM1", etc).

pBuffer	- Pointer to Buffer receiving the PORTINFO structure.

pwSize	- Size of buffer supplied. Or size of buffer required if buffer too small.

Return Value:

SUCCESS

ERR_BUFFER_TOO_SMALL

Description:

This API is used to get information about the Port specified. The caller can specify either the port handle for which the information is wanted - or the port name. The API call is synchronous. All the parameters relevant to the Port are returned in the RASMAN_PORTINFO structure. If buffer is too small, an error is returned.

For example: The parameters for the Serial Media are listed below.  The type column is the value in the P_Type field of the RAS_PARAMS struct in the RASMAN_PORTINFO struct.

|  | Parameters Returned | Type |
| --- | --- | --- |
|  | Connect Baud Rate | String |
|  | Carrier Baud Rate | String |
|  | Maximum Carrier Baud Rate | String |
|  | Data Bits | Number |
|  | Parity | Number |
|  | Stop Bits | Number |
|  | Hardware Flow Control Flag | Number |
|  | Error Control Flag | Number |
|  | Port Open Flag | Number |
|  | DefaultOff | String |

DWORD PortSetInfo (

IN		HANDLE			hIOPort,

IN		RASMAN_PORTINFO *	pInfo

)

Parameters:

hIOPort	- Port Handle.

pInfo	- RASMAN_PORTINFO structure with parameters to set.

Return Value:

SUCCESS

ERR_INVALID_PARAM_SPECIFIED

Win32 error codes returned by GetLastError()

Description:

This API is used to set information for a Port. The API call is synchronous. This API may be called many times - each time it will change the Port information to what is specified in the RASMAN_PORTINFO structure. Note that the caller need not specify every parameter relevant to the device - only the parameters specified in the supplied RASMAN_PORTINFO structure are changed.

For example, The parameters for Serial media which may be set by PortSetInfo are listed below.  The type column is the value in the P_Type field of the RAS_PARAMS struct in the RASMAN_PORTINFO struct. Attempting to set any other parameters will cause the ERR_INVALID_PARAM_SPECIFIED error to be returned.

|  | Settable Parameters | Type |
| --- | --- | --- |
|  | Connect Baud Rate | String or Number |
|  | Carrier Baud Rate | String or Number |
|  | Data Bits | Number |
|  | Parity | Number |
|  | Stop Bits | Number |
|  | Hardware Flow Control Flag | Number |
|  | Error Control Flag | Number |

DWORD PortTestSignalState (

IN		HANDLE		hIOPort,

OUT		DWORD *		pdwDeviceState

)

Parameters:

hIOPort		- Handle of the Port for which to get signal state.

pdwDeviceState	- Pointer to DWORD to receive signal state bit flag.

Return Value:

SUCCESS

Description:

This API is used to get information on the connection status.  The table below lists the bit fields which are currently defined.  The Serial Line column gives RS-232 lines which map to the bit fields in the case of serial port devices.

|  | Bit Field Name | Value | Serial Line |
| --- | --- | --- | --- |
|  | SS_HARDWAREFAILURE | 0x00000001 | DSR |
|  | SS_LINKDROPPED | 0x00000002 | DCD |

DWORD	PortDisconnect (

IN 	HANDLE		hIOPort

)

Parameters:

hIOPort	- Port Handle for the port to be disconnected.

Return Value:

SUCCESS

PENDING

ERR_PORT_NOT_OPEN

Description:

This API is used to Disconnect a Port. If this API returns SUCCESS then it is assumed that the port was disconnected synchronously. However, if the port takes a long time disconnecting then this API should return asynchronously with return code PENDING. When the disconnect is complete the hNotify event (passed in PortOpen()) must be cleared.

DWORD	PortConnect (

IN 	HANDLE		hIOPort,

IN	BOOL			bListenOnNullDevice,

IN	HANDLE		hRasEndpoint,

OUT	RASMAN_MACFEATURES *macinfo

)

Parameters:

hIOPort		-Handle of Port.

bWaitForConnection   - Whether to return PENDING or FAILURE if the connection does not already 			     exist.

hRasEndPoint	-Handle to be passed to (Async) MAC driver.

macinfo		-Compression mode of (Async) MAC driver.

Return Value:

SUCCESS

ERROR_PORT_NOT_OPEN

ERROR_NO_CONNECTION

Description:

This API is used to inform the Media DLL that a connection has occurred on the Port. The Media DLL should use this call to pass over the Port handles to the MAC so that it can start talking to the Ports directly. At this point the endpoint on which to communicate should also be passed down to the MAC. The API call is synchronous.

Note: Most vendors may only implement bWaiForConnection == FALSE case.

DWORD	PortCompressionSetInfo (

IN		HANDLE	hIOPort,

IN		MAC_FEATURES *Info						)

Parameters:

hIOPort		- Handle of Port on which to set the Compression level.

Info			- The MAC/Compression information to be set for the Port.

Return Value:

SUCCESS

Description:

This API is used to inform the Media DLL (and the MAC) about the level of compression (in the MAC)  to be used for that Port. If the level requested is not valid, an error is returned.

DWORD	PortSend (

IN		HANDLE	hIOPort,

IN		BYTE *	pBuffer,

IN		DWORD	dwSize,

IN		HANDLE	hNotify

)

Parameters:

hIOPort	- Handle of port to send on.

pBuffer	- Used to pass in the information to be sent.

dwSize	- Size of buffer above.

hNotify	- Event handle to be signaled when send is complete.

Return Value:

SUCCESS

PENDING

Win32 error codes returned by GetLastError()

Description:

This API is used to communicate on a port on which a connection has been established, but before control of the port is handed off to the MAC (by a call to PortConnect).  The API is basically asynchronous and will normally return PENDING and hNotify will be signaled when the send is complete.  However, if the send completes immediately (due to buffering, for example) PortSend will return SUCCESS.

DWORD	PortReceive (

IN		HANDLE	hIOPort,

IN		BYTE *	pBuffer,

IN		DWORD	dwSize,

IN		DWORD	dwTimeOut,

IN		HANDLE	hNotify

)

Parameters:

hIOPort	- Handle of port from which to receive.

pBuffer	- Used to receive the information.

dwSize	- Size of buffer above.

dwTimeOut	- Time out in milliseconds for receive operation.

hNotify	- Event handle to be signaled when receive is complete.

Return Value:

SUCCESS

PENDING

Description:

This API is used to communicate on a port on which a connection has been established, but before control of the port is handed off to the MAC (by a call to PortConnect).  The API is basically asynchronous and will normally return PENDING and hNotify will be signaled when the send is complete.  However, if the receive completes immediately (due to buffering, for example) PortReceive will return SUCCESS.

DWORD	PortGetStatistics (

IN	HANDLE			hIOPort,

OUT	RAS_STATISTICS *	pStats

)

Parameters:

hIOPort	- Handle of Port for which to get statistics.

pStats	- Pointer to a RAS_STATISTICS to receive statistics information.

Return Value:

SUCCESS

Description:

This API is used get statistics from the MAC attached to the port.  The counts reported as statistics will be the number of events since the most recent call to PortClearStatistics.  If PortGetStatistics is called without first calling PortClearStatistics the counts reported will be from the call to PortOpen.

DWORD	PortClearStatistics (

IN	HANDLE			hIOPort

)

Parameters:

hIOPort	- Handle of Port for which to get statistics.

Return Value:

SUCCESS

Description:

This API is used clear the current record of statistics from the MAC attached to the port.  PortClearStatistics is used to mark the beginning of the period for which the caller wants statistics.  PortGetStatistics is called at the end of the period.

Data Structures

The data structures relevant to Device DLLs are:

Note: typedef <type> FOO FOO is implicit.

struct DEVICE {

CHAR 	D_Name 	[MAX_DEVICE_NAME] ;

}

struct PORTINFO {

WORD	DI_NumOfParams ;

PARAMS	DI_Params[1] ;

}

struct PARAMS {

CHAR		P_Key		[MAX_PARAM_KEY_SIZE] ;

FORMAT	P_Type ;

BYTE		P_Attributes;

VALUE 	P_Value ;

}

union VALUE {

DWORD	Number ;

struct	{

DWORD 	Length ;

PCHAR	Data ;

}	String ;

}

enum FORMAT {

STRING,

DWORD

}

// Attributes: Can be a combination of:

#define	ADMIN_PARAMETER		0x01

#define	MANDATORY_PARAMETER	0x02

enum STATUS {

OPEN,

CLOSED

}

struct PortMediaInfo {

CHAR		PMI_Name [MAX_PORT_NAME] ;

USAGE	PMI_Usage ;

CHAR		PMI_DeviceType [MAX_DEVICETYPE_NAME] ;

CHAR		PMI_DeviceName [MAX_DEVICE_NAME] ;

}

## 6.4 Installation

Media DLLs should be installed for NT RAS Client and for the NT Remote Access Server in a %NTROOT%\SYSTEM32 directory.

# 7 Device DLL Specification

The Device DLLs are used by the RAS Manager component to allow establishment of connection through different devices. It is important to see the difference between the Device DLL and the Media DLLs. While establishing a connection over any Media (Serial, Parallel, X.25, ISDN) there are intermediary devices that need to be "conversed" with - for example a Modem, PAD, Authentication device etc. The knowledge of conversing with a device is contained in the Device DLL. Since there are several devices of interest there may be more than one Device DLL involved in making a connection. RAS Manager uses a standard set of APIs exported by the Device DLL in order to make a connection. Device DLLs may be attached to, and detached from as and when RAS Manager desires.

This chapter first describes where the DEVICE.DLL component fits in the RAS Architecture. The MEDIA.DLL functional definition, exported APIs, and the data structures relevant to these APIs are described.

Goals

The main motivation for having independent Device DLLs is to contain all the knowledge relevant to making connections over a device in the corresponding Device DLL. Through standard interface APIs,  DLLs for any Device must appear the same to the RAS Manager.

Relevance

This chapter will be of interest to developers who want to write Device DLLs to support new devices with RAS. An understanding of APIs, DLLs, and Synchronization mechanisms for Win32 is necessary for this document.

## 7.1 RAS Manager and Device DLLs

Figure 7.1 describes how Device DLLs fit into the RAS Manager architecture.  RAS Manager APIs are independent of the Devices that may be used to establish a connection. When a Device is requested to be connected through by the UIs and Supervisor, RAS Manager tries to see if the corresponding Device DLL is already loaded, if it is not loaded then it loads it. After that all APIs wishing to use that Device are translated to the Device DLL APIs.

At any given time, RAS Manager may be attached to several Device DLLs - depending on the connection requests.  Each Device DLL must talk directly through the Port in order to establish a connection through the device.

µ §

Figure 7.1

## 7.2 Functional Role

The different functions performed by Device DLLs are:

1.	Exporting the specified APIs to be used by RAS Manager.

2.	Parsing any Information files relevant to the device.

3. 	Communicating with the device.

4. 	Storing and setting information per port using a device.

1. Exporting APIs.

RAS Manager uses Device DLLs through the APIs exported. Each Device DLL must conform to the exact specification of the APIs. Since certain device operations may take significant time, some APIs are asynchronous.  In this case the APIs must return as soon as possible, the Device DLLs are called again and again (either through Polling or through an Event that must be signaled by the Device DLL) until the requested operation is completed.  A polling mechanism is used in order to conserve the use of  Threads.

2. Parsing Device specific Information files.

Device DLLs are responsible for parsing any vendor specific configuration files that are needed for the Device type. Through this the structure of the files, and knowledge of device parameters are localized to the Device DLL itself.

3. Communicating with the Device.

In order to make connections the Device DLLs send and receive strings to the corresponding devices. Since most of connect and listen operations require a long time, the Device DLLs maintain their own states internally so that they can be called again and again until the operation requested has been completed.

4. Device Information.

The Device DLLs must retrieve the device specific information and store it internally on a per port basis so that it can be used for establishing a connection. The DLLs should also store the device information that is set by the RAS Manager clients.

## 7.3 Device DLL APIs

DWORD	DeviceGetInfo (

IN		HANDLE	hIOPort,

IN		char *		pszDeviceType,

IN		char *		pszDeviceName,

OUT		BYTE *		pInfo,

BOTH		WORD *	pwSize

)

Parameters:

hIOPort		- File handle for the Port to which connecting.

pszDeviceType	- Name of the Device Type.

pszDeviceName	- Name of the Device.

pInfo		- Pointer to buffer which will contain the information returned.

pwSize		- Size of the buffer provided. On return, size of information returned.

Return Value:

SUCCESS

ERROR_DEVICENAME_NOT_FOUND

ERROR_BUFFER_TOO_SMALL

Description:

This API is used to get information about a device from the Device DLL. The call is synchronous. If the supplied buffer is too small the API completes with an error and the required size is specified. The information returned is in the form of a DEVICEINFO structure. The Device DLL must parse the appropriate files and retrieve this information. The PCB structure pointer is provided so that the Device DLL can use port specific information if it needs to.

If this API is called after connecting through the device, the information will reflect any connection related information (for example, if connection took place at V.42 instead of V.42bis as requested, the information returned will reflect this).  If this API is called immediately after a call to DeviceDone(), the information returned will be default configuration values.

DWORD	DeviceSetInfo (

IN		HANDLE		hIOPort,

IN		char *			pszDeviceType,

IN 		char *			pszDeviceName,

IN		DEVICEINFO*		pInfo

)

Parameters:

hIOPort		- File handle for the Port to which connecting.

pszDeviceType	- Name of the Device Type.

pszDeviceName	- Name of the Device.

pInfo		- Pointer to DEVICEINFO structure containing information to set..

Return Value:

SUCCESS

ERROR_DEVICENAME_NOT_FOUND

ERROR_BUFFER_TOO_SMALL

ERROR_WRONG_KEY_SPECIFIED

Description:

This API is used to set information for a device of the specified Device Type. The API call is synchronous. The Information supplied is set for the device in question. This API may be called many times - each time it will change the device information to what is specified in the DEVICEINFO structure. Note that the caller need not specify every parameter relevant to the device - only the parameters specified in the DEVICEINFO structure are changed.

If any of the parameters specified in the DEVICEINFO field in not supported by the device the ERROR_WRONG_KEY_SPECIFIED error is returned.

DWORD	DeviceConnect (

IN	HANDLE		hIOPort,

IN 	char *			pszDeviceType,

IN 	char *			pszDeviceName,

IN	HANDLE		hNotifier

)

Parameters:

hIOPort		- File handle for the Port to which connecting.

pszDeviceType	- Name of the Device Type.

pszDeviceName	- Name of the Device.

hNotifier	- Handle to event used to request RAS Manager's attention.

Return Value:

PENDING

ERROR_DEVICENAME_NOT_FOUND

ERROR_UNRECOGNIZED_RESPONSE

Device Specific Return Codes

Description:

This API is called to initiate a connection attempt. Note that the device DLL must not hold on to the thread until connection is complete; it should initiate the connection process and return. Subsequently, RAS Manager will call the DLL with DeviceWork() API when the event specified is cleared,

The DLL should use the I/O APIs asynchronously (associating the supplied event with I/O completion request). This guarantees that when the DLL requires CPU it gets it.

If initiation of the connection is successful, the API should return PENDING, else it should return the relevant error code.

DWORD	DeviceListen (

IN	HANDLE		hIOPort,

IN 	char *			pszDeviceType,

IN 	char *			pszDeviceName,

IN	HANDLE		hNotifier

)

Parameters:

hIOPort		- File handle for the Port to which listening.

pszDeviceType	- Name of the Device Type.

pszDeviceName	- Name of the Device.

hNotifier	- Handle to event used to request RAS Manager's attention.

Return Value:

PENDING

ERROR_DEVICENAME_NOT_FOUND

ERROR_UNRECOGNIZED_RESPONSE

Device Specific Return Codes

Description:

This API is called to initiate a listen on a Port. Note that the device DLL must not hold on to the thread until connection is complete; it should initiate the listening process and return. Subsequently, RAS Manager will call the DLL with DeviceWork() API when the event specified is cleared.

The DLL should use the I/O APIs asynchronously (associating the supplied event with I/O completion request). This guarantees that when the DLL requires CPU it gets it.

If initiation of the connection is successful, the API should return PENDING, else it should return the relevant error code.

DWORD	DeviceEnum (

IN	char *			pszDeviceType,

OUT	WORD *		pcEntries,

IN	BYTE *			pBuffer,

BOTH	WORD *		pwSize

)

Parameters:

pszDeviceType	- Name of the Device Type.

pcEntries		- Number of DEVICE structures returned in the buffer.

pBuffer		- Pointer to buffer which will contain the DEVICE structures returned.

pwSize		- Size of the buffer provided. On return, size of information returned.

Return Value:

SUCCESS

ERROR_BUFFER_TOO_SMALL

Description:

This API is used to retrieve a list of devices available for a device type. An array of  DEVICE structures is returned  in the buffer supplied. The API call is synchronous.

VOID	DeviceDone (

IN 	HANDLE		hIOPort

)

Parameters:

hIOPort	- Pointer to the PortControlBlock of the calling port.

Return Value:

Nothing

Description:

This API is used to inform the Device DLL that the attempt to connect/listen is completed (terminated, succeeded, etc.)  This may be used by the Device DLL to do any clean-up work for that connection/listen attempt. The API call is synchronous.

DWORD	DeviceWork (

IN 	HANDLE		hIOPort,

IN	HANDLE		hNotifier

)

Parameters:

hPort	- Pointer to the PortControlBlock of the calling port.

hNotifier	- Handle to event used to request RAS Manager's attention.

Return Value:

SUCCESS

PENDING

ERROR_UNRECOGNIZED_RESPONSE

Device specific return codes.

Description:

This API is used to call the Device DLL so that it can further process the connection/listening attempt. The API will be called when the event specified in the (DeviceConnect() or DeviceListen() APIs is signaled. The DLL should initiate the next step in the connection attempt asynchronously with the supplied event and return with the PENDING return code. If the operation in progress is complete it should return SUCCESS or the relevant return code.

Data Structures

The data structures relevant to Device DLLs are:

Note: typedef <type> FOO FOO is implicit.

struct DEVICE {

CHAR 	D_Name 	[MAX_DEVICE_NAME] ;

}

struct DEVICEINFO {

WORD	DI_NumOfParams ;

PARAMS	DI_DeviceParams[1] ;

}

struct PARAMS {

CHAR		P_Key		[MAX_PARAM_KEY_SIZE] ;

FORMAT	P_Type ;

BYTE		P_Attributes;

VALUE 	P_Value ;	[MAX_PARAM_VALUE_SIZE] ;

}

union VALUE {

ULONG	Number ;

CHAR 	String	[MAX_VALUE_STRING_SIZE] ;

}

enum FORMAT {

STRING,

ULONG

}

// Attributes: Can be a combination of:

#define	ADMIN_PARAMETER		0x01

#define	MANDATORY_PARAMETER	0x02

## 7.4 Installation

Device DLLs should be installed for NT RAS Client and for the NT Remote Access Server in the %NTROOT%\system32 directory.