# Windows NT Remote Access

# RAS Server Supervisor

# Component Design Document

## Revision History

| Revision | Date | Author | Comment |
| --- | --- | --- | --- |
| 1.0 | 03/18 | Stefan Solomon | Created |

## 1.	Audience

### 1.1.	Prerequisites

### 1.2.	References

NT RAS Functional Requirements.

NT RAS Architectural Specification Document.

Win32 API Specification.

NT Service Control Specification.

## 2. 	The Problem

### 2.1.	Purpose

The  Supervisor  is the central component of the RAS server. It monitors the connections,  and synchronizes the authentication, NetBIOS gateway , line signals and service control requests.

The Supervisor is a  process which starts as a service  on NT LM. It posts listen on every dialin device present and monitors the lines. When a line gets connected, it invokes the authentication procedures for the remote client and requests the NetBIOS gateway (or IP, IPX routers) to project the client  (add NetBIOS names or IP, IPX addresses).

When the line gets disconnected it tells the NetBIOS gateway to un-project the client and reposts listen waiting for new clients.

When the RAS server is stopped,  it disconnects the lines.

### 2.2.	Design Goals

·	Achieve a modular, maintainable and robust component.

·	Minimize system resources usage (e.g. event objects) by using a message based synchronization scheme.

### 2.3.	Design Exclusions

## 3.	Platform Support

### 3.1.	Systems

Windows NT.

### 3.2.	Down-level Support

Describe to what extent previous versions of the component and it's clients/providers are supported and how this support will be achieved.

### 3.3.	Up-level Support

Describe any plans for supporting future expansion of the product.  Describe any non-obvious disciplines that must be followed to avoid creating future expansion problems.

## 4.	Development Tools

### 4.1.	Build Environment

List the system and tools needed to build the component, how it fits into the product source trees, etc.

### 4.2.	Language Tools

Describe the compiler/assembler or other language tools used to build the component.

### 4.3.	<Another Toolset...>

Add a section for each  major toolset you will be using, e.g. Presentation Tools to describe the UI libraries used,  Device Driver Tools to describe DevHelp libraries used, etc.   List the owner and location of source code and/or a descriptive document for each.

Note these are development tools.  Tools written/used specifically for testing should be listed in the Testing section.

### 4.4.	Prototype

If you plan to do a prototype, describe it here.

## 5.	The Solution

### 5.1.	Overview

The Supervisor is implemented as a set of event driven state machines. Every dialin device has an associated state machine. The whole set of state machines is implemented via a unique thread which waits on a set of events. There is an event associated with the connection state of every dialin device and a message mechanism for communication with the NetBIOS gateway and the authentication threads. See the block diagram below.

### 5.2.	Major Data Structures

The Supervisor is implemented as a set of  state machines, one per remote client and all of them packaged in on thread. The major supervisor modules are event handlers  for this machines:

RasMan_Event_Handler;

Amb_Event_Handler;

NetBios_Gtwy_Event_Handler;

Service_Stop_Event_Handler.

Besides, there is an Supervisor_Init module which sets up the data structures and posts the initial listen on the dialin devices.

A Timer module.

The timer is implemented using a delta queue. The timer will have:

StartTimer

StopTimer

Timer() // timer handler

There will be a pool of nodes, one for each Device Control Block:

timer node struct:

link;

delta t;

callback function;

pointer (index) to the DCB, passed as argument to the Callback function.

A Close module which is invoked by different event handlers when the line gets disconnected and whose purpose is to clean up and insure that all resources are deallocated and  terminated  before reposting a listen.

The major data structures are:

service configuration data. This structure keeps all service configuration values (see the func spec).

service parameters table. This is an array of structures where each structure describes a configuration parameter. The entry has the parameter name, type, default, max, min and the address of the memory location to get the registry specified value.

a global variable keeping the state of the service:

ras_service_state : STARTING, ACTIVE, PAUSED, STOPPING, STOPPED.

global variables containing the configuration parameters.

an array of Device Control Block structures, one per dialin device:

client_service_state: see below

Note:

At creation time, all ports are initialized in the stopped state.

Then, as listen is posted on each port, the state is changed to disconnect_listen. It will never be stopped until the service gets stopped.

authentication_timer_active: ON/OFF - this flag is ON as long as the auth timer is kept active. The auth passed & projection request or auth passed & callback request from the auth module resets the flag and stops the timer.

dialin_device_state:  CONNECTED, DISCONNECTED

Authentication_module_information

·	Authentication package index. This indexes into a table of authentication packages attached to the server. Each table is an array of pointers to the authentication API. The initial value of the index is established at connection time, when a frame is received and passed along  to each authentication package until one recognizes it.

·	Authentication_module_state: ACTIVATED,  STOPPING, STOPPED

·	Authentication transport - NBF, AUTH_ASYNC_XPORT.

·	NBF routing status - DEALLOCATED,  ALLOCATED, ACTIVATED

·	NBF lana - this field is valid only if  NBF routing status is  ALLOCATED or ACTIVATED.

·	transport address - used later if Direct connection is required.

Direct_connection_information:

·	Direct connection status - INACTIVE, REQUESTED, ACTIVE

·	NBF routing status - DEALLOCATED, ALLOCATED, ACTIVATED

·	transport address - used in NetServerTransportAdd, NetServerTransportDel for direct connection;

NetBios_service_information:

·	NetBIOS_service_state:  INACTIVE, PROJECTION_STARTED, PROJECTION_DONE, ACTIVE

·	RasManager protocol handle (maybe, to unroute);

·	NBF  routing status -  DEALLOCATED,  ALLOCATED,  ACTIVATED

·	NBF lana - valid only if the routing status is not DEALLOCATED.

IP_service_information:

·	IP status -  INACTIVE, REQUESTED, ACTIVE - this will be colapsed into routing status

·	IP routing status - DEALLOCATED, ALLOCATED, ACTIVATED

IPX_service_information:

·	IPX_status - INACTIVE, REQUESTED, ACTIVE - will be colapsed into routing status

·	IPX routing status - DEALLOCATED, ALLOCATED, ACTIVATED

timer:  There are  three different delays that this timer should keep:

·	When the authentication is started for this client (client_service_state == authentication_started) the timer is used for the authenticatetime control.

·	In a hardware error situation (client_service_state == hw_error) the timer is used to know when to repost listen if a previous listen has failed. The timer is for 5 sec usually.

·	In callback, before reconnecting with the remote client (client_service_state == callback_disconnecting), to wait before reconnecting. This is necessary because the remote client's modem may have a long disconnection time.

The timer nodes are kept in the :

free_timer_nodes_pool. There is one node per DCB.

StartTimer puts the DCB ptr in the timer node and inserts the node in the delta queue.

StopTimer or timeout returns the node to pool.

### 5.3.	Pseudo-code and/or Block Diagrams

First, we will present the interactions between the supervisor and the other components of the system. The Supervisor interracts with:

·	Ras Manager;

·	Authentication  (AMB ) component;

·	Registered Service Control Handler;

·	NetBIOS Gateway.

The interaction is achieved as follows:

The Supervisor calls the respective module at an entry point. The call is asynchronous or returns immediately with the result. When the asynchronous call has been completed, the invoked module signals the supervisor. There are two siganling mechanisms employed by the Supervisor:

RasManager  signals via event objects, one per port. The Service Contro handler also uses an event to signal. The Authentication and the NetBIOS gateway use a common message.dll. This is a mechanism which allows the signaler to pass a message to the siganled entity. This mechanism minimizes the number of events to be waited upon by the Supervisor thread and provides the information (message) and the signal.

Note: A useful debugging tool would be to have a log of messages passed through the message.dll.

Interraction Specifics:

Supervisor -> Ras Manager:

RasEnum

RasConnect

RasDisconnnect

RasListen

RasGetInfo

All the calls are asynchronous. Synchronization is achieved via an event object passed as a parameter in RasConnnect/RasDisconnect calls.

RasManager -> Supervisor:

When the event gets signaled by the RasManager, the Supervisor submits a RasGetInfo for the signaled device. This call returns immediately  reporting the device state (connected/disconnected) and the reason (remote disconnected, hw error, etc.)

The Supervisor uses the same event for all the DCBs.

To find out which DCBs are affected when the event is signaled, the RasMan_Event_Handler scans all the DCBs for each signal.

To have a more efficient processing here we will implement the scanning as follows:

The function map_port_state_to_index gets the status information about a port from RasGetInfo and then decides if this can be mapped to CONNECTED, DISCONNECTED or don't care.

CONNECTED -> 0

DISCONNECTED -> 1

all others -> -1

The  DCB states values are used as indexes.

The table RasMan_Event_Handlers_Table is a 2D array of function pointers to sub-handlers of the RasMan_Event_Handler.

FUNCPTR	RasMan_Event_Handlers_Table[MAX_DCB_STATES][2] = {

NULL ,    // stopped - connected

NULL,     // stopped -disconnected

start_auth_handler,  // disconnect_listen - connected

start_hw_err_handler, // disconnect_listen -disconnected

etc.

};

The code is efficiently written as:

RasMan_Event_Handler ()

{

for(each DCB )  {

map_port_state_to_index(DCB);

if(port_state_index = map_port_to_index(port_state) == -1)

break;

if(handler = RasMan_Event_Handlers_Table [DCB state][port_state_index])

(*handler)( this DCB )

}

}

Supervisor -> Authentication:

AuthStart(device_id, nr_of_retries, frame_buffp, frame_bufflen)

AuthStop(device_id)

AuthCallbackDone(device_id)

AuthProjectionDone(device_id,  projection_result)

Authentication -> Supervisor:

This is done via the message.dll subcomponent. The following messages are posted by the Authentication:

authentication_failure ( reason = nonauthenticated, xport_error, etc.)

authentication_stop_completed

projection_request (and projection information provided. The projection info includes protocol name and protocol data to project. For instance: NetBIOS + names, IP + address, etc.)

callback_request (this includes the phone nr to call back and the callabck delay (if other than  the default delay parameter used by the supervisor).)

authentication_done.

The first authentication request after AuthStart will stop the auth timer (StopTimer).

Supervisor -> NetBIOS Gateway

The Supervisor issues requests to the NetBIOS Gateway via the same message.dll.

The requests are:

projection_request

start_client (device_id,  NetBIOS lana for this client)

terminate_client (device_id)

terminate_gateway

NetBIOS Gateway -> Supervisor

disconnect_request

projection_done

client_started

client_terminated

gateway_started

gateway_terminated - gateway can be terminated because ordered so by the supervisor or because it had a fatal configuration param problem or a fatal run time problem. When  a fatal problem happens, the gateway logs the error.

Service Controller Handler -> Supervisor

There is an event reserved for this interraction. When the event gets signaled the Supervisor must stop the service.

Client_Service_State.

The  client_service_state is the main state variable of the machine. It is maintained on a per port basis.

In can take the followin values:

disconnected_listenning

hw_error

paused

stopped

authentication_active

projection_started

active

callback_disconnecting

callback_connecting

closing

µ §Supervisor Main Modules:

Supervisor_Init()

{

Start the service, register the Service Control Handler and update a global memory area with the service parameters. The service parameters are read from the registry using the registry APIs.

Setup table of authentication packges entry points.

RasEnum to get information on all the RAS System devices.

Post listen on each device;

If NetBIOS gateway will be used :

SetGtwyConfiguration in message.dll;

Start the gateway process;

Wait for the gateway to send back a gateway_started message . The Supervisor uses a timeout in waiting for the gateway to start (in the wait on event from message.dll). If no message is received from the gateway, the gateway process is killed and instalation is aborted.

Call RasProtocolEnum (or use params ?) and determine if IP, IPX will be used. For each of them used,  load the library module with their entry points.

Inform the service controller that the servcie has been started.

}

The DCB state machine transitions diagram:

µ §

Notes:

The event handler modules implement the above state machine.

Each event handler module makes the transition to the next state and invokes appropriate functions as per the transition diagram

RasMan Event Handler Module

This module handles the: CONNECTED and DISCONNECTED events.

Service Control Event Handler Module

This module handles the following events from the Service Controller:

SERVICE_STOPPED

SERVICE_PAUSED

SERVICE_CONTINUE.

SERVICE_PAUSED event handling:

If the state is disconnected_listening or hardware error, then

the line is disconnected (RasDisconnect) and the next state will be paused. Else, the event is ignored. The close module will take care to change the state to paused on close complete.

SERVICE_CONTINUE handling:

If the state is paused, the next state will be disconected_listening and alisten is posted. Else, this is ignored.

Close Module

A special case is the transition in the closing state. When this occurs, there may be subcomponents which are still active for this device and we have to synchronize with their closing. For instance, the line gets disconnected but the client has been already projected and authentication is still active.

In the case of such an event (line disconnected above, handled by the Ras Manager Event Handler)  the event handler which detects the closing condition changes the state to closing and calls the CloseInit function. The  function will initiate all actions to stop all the subcomponents still running for that client.

In closing state, all event handlers will call CloseExec function when they receive termination messages from the respective modules.

When all services activated for the client are in the OFF state,  the Close module will call CloseComplete which then switches the state to stopped  (if the service is stopping) or post a new listen and switch to disconnect_listening state.

Timer Module

This module is used to schedule different actions after a delay. It provides an API of the form:

StartTimer(device_id, timeout, callback)

StopTimer(device_id).

The data structure used will be probably a delta queue, or we may use something simpler.

### 5.4.	Modular Decomposition

## 6.	Testing

### 6.1.	Test Tools and Hooks

Describe any test/monitoring tools you will be using, counting on someone to write, etc.

### 6.2.	Unit Testing

Supervisor unit testing will be done in the following steps:

·	A basic Win32 service framework will be built and tested;

·	The config. parameters reading and checking using Win32 API will be added and tested;

·	Three emulators will be built for further tests:

1.	A Ras Manager emulator  -> generates Connect/Disconnect events;

2.	An authentication emulator -> generates auth events in the message.dll

3.	A Netbios gtwy emulator -> generates gtwy events and dequeues Superviosr messages.

·	Using these emulators, further unit tests are done.

·	Final integration.

### 6.3.	Test Problems

List any features that will be difficult to test or that cannot be fully tested.

## 7.	Dependencies

The component's complete unit test is dependent upon the availability of:

·	RasManager

·	Authentication package - client and server

·	NetBios Gateway

·	Client module to connect and authenticate (may be clien UI or some test stuff).

## 8.	Work Estimates

| Days | Task |
| --- | --- |
| 2.0 | Setup build and debugging environments |
| 2.0 | Code NT service frame |
| 2.0 | Test NT service frame |
| 4.0 | Code config parameters read and check using the Win32 Registry API |
| 6.0 | Code main event handler modules and data structures |
| 1.5 | Code the Close module |
| 2.0 | Code the Timer module |
| 2.0 | Code the message.dll |
| 4.0 | Code test modules: event emulators for RasManager, Authentication and NetBios Gateway |
| 4.0 | Unit test the Supervisor with emulated events |
| 3.0 | Integrate the Supervisor with the real RasManager - Note 1 |
| 2.0 | Integrate the Supervisor with the real NT RAS Server Authentication - Note 1 |
| 3.0 | Integrate the Supervisor with the real NetBios Gateway - Note 1 |
| 3.0 | Final unit test - the whole system: Supervisor, RasManager, Authentication, Gateway. |

Note 1:	These integrations are done function of the availability of the real components. The complete unit test will be done with all components tested and the whole system client/server completed.

## 9.	Open Issues

This is a bulleted list of unresolved issues.  At the time of the CDD review, the items should be mainly minor details and external unresolved issues.  Significant design issues are better listed in the pseudo-code section.