Windows NT Remote Access

RASPHONE

Component Design

Revision History

| Revision | Date | Author | Comment |
| --- | --- | --- | --- |
| 0.1 | 03/01/92 | Steve Cobb | Original draft |
| 1.0 | 04/07/92 | Steve Cobb | Review comments |

Table Of Contents

µ1.  Audience	3

1.1.  Prerequisites	3

1.2.  References	3

2.  The Problem	3

2.1.  Purpose	3

2.2.  Design Goals	3

2.3.  Design Exclusions	4

3.  Platform Support	4

3.1.  Systems	4

3.2.  Down-level Support	4

3.3.  Up-level Support	4

4.  Development Tools	4

4.1.  Build Environment	4

4.2.  Language Tools	4

4.3.  Presentation Tools	5

4.4.  RASFILE	5

5.  The Solution	5

5.1.  Overview	5

5.2.  Major Data Structures	6

5.3.  Pseudo-code and/or Block Diagrams	6

5.4.  Modular Decomposition	8

6.  Testing	9

6.1.  Test Tools and Hooks	9

6.2.  Unit Testing	9

6.3.  Test Problems	9

7.  Dependencies	9

8.  Work Estimates	9

9.  Open Issues	11

# 1.  Audience

## 1.1.  Prerequisites

The reader is assumed to be familiar with the following:

·  NT RAS product goals and terminology

·  RAS 1.0 RASPHONE for DOS/OS2 functionality.

·  The Windows RASPHONE prototype.

·  RAS Manager APIs functional specifications.

·  RAS External APIs functional specifications.

## 1.2.  References

NT RAS Windows RASPHONE Functional Specification describes the functional requirements of Windows RASPHONE.  See \\testy\consys\src\ras\doc\rasphofs.doc.

Windows RASPHONE Visual Basic prototype demonstrates the operation and appearance of Windows RASPHONE on dummy data.  See \\testy\consys\src\ras\proto\rasphone\rasphone.exe.

NT RAS Architecture is an architectural overview of the entire NT RAS product.  See \\testy\consys\src\ras\doc\rasarch.doc

NT RAS External API Functional Specification and Component Design describes the external APIs which provide developers with dial/hangup/status functionality from within a program.  See \\testy\consys\src\ras\doc\funcspec\rasapifs.doc and \\testy\consys\src\ras\doc\design\rasapicd.doc.

NT RAS RAS Manager Functional Specification and Component Design describes the RasXxx API interface.  See \\testy\consys\src\ras\doc\funcspec\rasmanfs.doc and \\testy\consys\src\ras\doc\design\rasmancd.doc.

NT RAS RASFILE Functional Specification and Component Design describes the configuration file editing library.  See \\testy\consys\src\ras\doc\funcspec\rasfilfs.doc. and \\testy\consys\src\ras\doc\design\rasfilcd.doc.

# 2.  The Problem

## 2.1.  Purpose

Windows RASPHONE provides a graphical user interface to the RAS phone book and connection management functionality.  Windows RASPHONE is expected to be the most commonly used RAS UI tool (followed closely by DOS RASPHONE), and as such, will have great impact on the customers perception of the product.

DOS RASPHONE provides a character window interface identical to Windows RASPHONE in functionality.

## 2.2.  Design Goals

·	All standard UI objectives apply, e.g. command sequences will be intuitive, feedback will be provided to all user input, screen layout will be visually pleasing, etc.

·	Full support for both mouse and keyboard operation will be provided.

·	Being a "daily" tool, the interface must minimize keystrokes and mouseclicks for common operations.  The most common operations are expected to be dialing and hanging up an existing phone book entry.

·	Since a large percentage of RASPHONE users will be non-technical, techno-babble should be avoided  and, where necessary, should be presented in a non-threatening way.  Concise contextual and outline help should be available at all times.

·	Since Windows is the company's critical client platform , this interface must be at least as fast as DOS/OS2 RASPHONE.

## 2.3.  Design Exclusions

·	The new command line support provides Program Manager shortcuts using a RASBATCH-like interface, but the Windowed UI still appears.  As a Windows program it does not does not provide RASBATCH functionality from the DOS command line.

·	The *terminal macro which would have allowed an interactive terminal session to be embedded in a larger switch script in not included.  However, terminal mode is available as an alternative to a canned switch script.  (This should be sufficient for the Defender box case).

# 3.  Platform Support

## 3.1.  Systems

Win16 RASPHONE will run on Sparta, Win32 RASPHONE on Windows NT, and COW RASPHONE on Slalom (DOS).  The Windows programs are based on a single source.

## 3.2.  Down-level Support

RAS 1.0 phone books will be automatically updated by NT RAS RASPHONE, but the phone book produced by Windows RASPHONE will not be readable by RAS 1.0 DOS/OS2 RASPHONE.

## 3.3.  Up-level Support

The phone book format produced will be extensible to support the future protocol stacks and non-modem devices listed in the architecture document.

# 4.  Development Tools

## 4.1.  Build Environment

The program will be built in the standard NT build environment using the BUILD command rather than makefiles.  The MS SLM program will be used for source code management.

The source trees will be set up with the intent of building both Win16 and Win32 versions of the program.  The anticipated source tree location of this component is \\testy\consys\ras\src\ui\client\{win,cow}.

ISSUE: Verify that NT build environment supports Win16 build using Win 3.1 SDK tools.

## 4.2.  Language Tools

Windows RASPHONE is a C++ program, based on the BLT Windows classes.  The same compiler used for BLT will be used.  At this time, that means Cfront.  At some point, possibly before this release conversion to MS C7 will occur.  (Even if this occurs, it should not be too painful, since BenG has been on top of what's going on with C7 for some time).

COW RASPHONE will be based on MS C 7.0.

## 4.3.  Presentation Tools

Windows RASPHONE will be based on the BLT windows classes being used by the NetUI group.  The advantages of BLT for RASPHONE are not as great as for RASADMIN, since uniformity of appearance and the availability of the LMOBJ support libraries are less of an issue.  However...

Visual Basic was considered but judged unfit because:

·	There is currently no support for Win32 or UNICODE.  Since our use of NT-WoW is in question, this is a serious problem.

·	Basic does not scale to large programs well.  I consider RASPHONE a medium-sized program, but it would push VB's scalability right to the edge (and beyond?).

·	As an interpreted language it's performance is not as good as C-based packages.

·	It requires use of the integrated development environment and does not support include files both of which are a pain for localization.  As far as I know, it doesn't support DBCS either.

·	The integrated development environment is not well suited to SLM.

·	There is no precedent for serious MS-systems applications in VB, so we're on our own.

The commercial nature of other C-based windows class libraries is attractive, since BLT offers no real chance of buying what you need.  But other C-based windows class libraries (I briefly considered MS MFC/AFX) are disadvantaged because:

·	Windows libraries are large, involved, and take time to master.  (Not that I wouldn't like to try one, mind you, but we're already pinched for time).  Our group has already paid the up-to-speed time on BLT and the product already contains another BLT-based program.

·	BLT already addresses localization concerns such as LAN Manager error strings and UNICODE support.  With BLT, RasUI (meaning me) gets to leverage off the NetUI (huge) group for support on common issues like this.

·	There are several BLT-based classes that can be taken directly from RASADMIN, which would have to be re-invented with a new package.

·	We know for a fact that BLT will work on NT.  With everything else, we're hoping.

ISSUE:  It would be nice if we could get NetUI to build the 16-bit version of BLT in their tree so we can avoid creating a private BLT snapshot.  Not that they're likely to support the 16-bit version, just let us borrow space in their build tree.

DOS RASPHONE will use the same COW package as RAS 1.0 RASPHONE.

## 4.4.  RASFILE

The RASFILE library (based on the Sparta CFG file editor) will be used to manipulate the PHONE.INF file contents.  A C++ wrapper class (RASFILE) will be used in Windows RASPHONE.  DOS RASPHONE will use the library directly.

# 5.  The Solution

## 5.1.  Overview

The general structure of Windows RASPHONE is dictated by BLT, and will be very similar to the existing RASADMIN code.  Given this and the existence of the VB prototype, there's no point in doing involved pseudo-code descriptions of most of the top-end (user interface) functionality.

A PHONEINF class derived from RASFILE will be created to manipulate the phone book.

At the bottom-end, RASPHONE talks to the RAS Manager and Authentication DLLs using the same APIs provided for external developers.

DOS RASPHONE will simply be extensions of the RAS 1.0 RASPHONE code.

## 5.2.  Major Data Structures

class PHONEINF : public RASFILE

{

public:

PHONEINF::PHONEINF();

/* Set global CALLBACK number to 'pszNumber'

*/

VOID SetCallbackNumber( CHAR* pszNumber );

/* Finds entry 'pszEntry' and sets caller's '*pwlkrsectEntry' to it.

** Returns TRUE if found, FALSE otherwise.

*/.

BOOL FindEntry( CHAR* pszEntry, WLKR_SECT* pwlkrsectEntry );

/* The following "Set" commands creates/modifies the associated subsection with the parameter

** values indicated.  The SetSerialMedia, SetModem, and SetPad methods also set the

** "old" style parameters in the entry's main section.  In each case, 'pwlkrsect' points to

** the entry section.  If 'pwlkrsect' is NULL, the associated subsection is deleted.

*/

VOID SetSerialMedia( WLKR_SECT* pwlkrsectEntry, CHAR* pszPort, ULONG ulBaud );

VOID SetPreconnectSwitch( WLKR_SECT* pwlkrsectEntry, CHAR* pszName );

VOID SetPostconnectSwitch( WLKR_SECT* pwlkrsectEntry, CHAR* pszName );

VOID SetModem( WLKR_SECT* pwlkrsectEntry, CHAR* pszPhoneNumber,

CHAR*pszDescription, BOOL fErrorControl, BOOL fCompression,

BOOL fHwFlowControl );

VOID SetPad( WLKR_SECT* pwlkrsectEntry, CHAR*pszPadType, CHAR* pszAddress,

CHAR* pszUserData, [BOOL fFacility1...] );

}

## 5.3.  Pseudo-code and/or Block Diagrams

APP_START class (bootstrapped automatically by BLT)

OnInit()

Verifies that workstation is started, exits if not

Initializes globals and construct APP_WINDOW

Registers main window class

Creates main window

Main()

Loads accelerator table

Starts message loop

OnTerm()

Writes phone book, if dirty

deletes APP_WINDOW

APP_WINDOW

Application window callback procedure

Reads phone book into memory and  uses it to fill list and read options and callback number

The two dispatchers invoke subdialogs by calling the XxxDlg stub functions:

OnMenuCommand() dispatcher

OnCallback does CALLBACK_DIALOG

OnMinimizeOnDial/Hangup

Sets MINONDIAL and MINONHANGUP to YES/NO in PHONEINI global file section.

OnExit ends the application

OnHelp calls WinHelp with INDEX or Context Number

OnAbout does ABOUT_DIALOG which does not use external data

OnCommand()  dispatcher for Phone Book list and Button clicks

OnAddEntry does ENTRY_DIALOG(EditMode=FALSE)

OnEditEntry does ENTRY_DIALOG(EditMode=TRUE)

OnRemoveEntry

Removes entry from phone book following confirmation popup.

OnDial does DIAL_DIALOG

OnHangUp

Calls RasHangup() on the current connection handle.

OnStatus does PORTSTATUS_DIALOG

CALLBACK_DIALOG

Retrieves callback number from PHONEINI global file section

OnOK()

Writes callback number back to PHONEINI global file section

ENTRY_DIALOG

If EditMode

Retrieve PhoneBook entry from PHONEINI

Else

Create Default entry

OnModemSettings does MODEMS_DIALOG

OnX25Settings does X25 DIALOG

OnSwitchSettings does SWITCH_DIALOG

OnOK()

If EditMode

Remove previous entry from PHONEINI

Create new entry in PHONEINI

MODEMS_DIALOG

Ports, Modems, and Baud lists are read from COMDEVINI object

Current port and baud are read from PHONEINI serial media section

EC, ECC, and HWFLOW checkbox settings are read from PHONEINI entry

OnOk()

Current port, baud, and EC, ECC, and HWFLOW checkboxes are written back to PHONEINI entry

X25_DIALOG

All settings are read from PHONEINI PAD device subsection

OnOk()

All settings are written back to PHONEINI PAD device subsection

SWITCH_DIALOG

Both settings are read from respective PHONEINI SWITCH device subsections

OnOk()

Both settings are written back to the subsection

DIAL_DIALOG

Loads Phone Number from PHONEINI entry

Loads username with LUI_GetDefUsername

OnOk()

If USERNAME or PASSWORD parameters returned by RasGetInfo on X.25 device

Do CONNECTX25_DIALOG

Do DIALSTATUS_DIALOG

ISSUE: Find out how LUI_GetDefUsername works.

DIALSTATUS_DIALOG

Call RasDialCfg()

Enable 1-second timer

OnCancel

CallRasHangup()

Exit

OnTimer()

Call RasGetInfo()

If status has changed

Update list of events

If status is error

Display error

If status is done

Change Cancel button to Ok

OnOk()

Exit

PORTSTATUS_DIALOG

Get statistics with RasGetStatistics()

OnTimer

Refreshes statistics with RasGetStatistics()

OnZeroCounts

CallsRasClearStatistics()

## 5.4.  Modular Decomposition

Windows RASPHONE will be modularized by common dialogs.  Each dialog set consists of the files module.cxx, module.hxx, module.rch, and module.dlg which contain the methods, class definitions and constants, resource constants, and dialog resource definitions respectively.

| Module | Contents |
| --- | --- |
| rasphone | Application initialization and termination routines (the WinMain is actually in BLT) |
| appwin | Application window and menu dispatch routines |
| misc | About, Callback Number, and Port Status dialog routines |
| entry | Add/Edit/Remove phone book entry dialog routines |
| settings | Modem, X.25, and Switch Settings dialog routines |
| dial | Dial and Connect To X.25 dialog routines |
| status | "Status Log" dialog routines |

In addition to the dialog modules, the following files are included:

| File | Contents |
| --- | --- |
| rasphone.rc | Master resource file which contains string tables, menu definitions, and includes the module.dlg and module.rch files. |
| phoneinf.?xx | CFGFILE-derived phone.inf file manipulation class |
| errmsg.?xx | Multi-level error message routines |
| util.?xx | General utility classes and routines |

DOS RASPHONE will simply logically extend the RAS 1.0 organization.

# 6.  Testing

## 6.1.  Test Tools and Hooks

RASPHONE will support the GUILTT hooks supported by RASADMIN, if they are needed.

ISSUE:  Are they still needed?

## 6.2.  Unit Testing

Since most of what UI's do is visible, unit testing consists mainly of simply running down the feature list and seeing if everything works.  Specifically:

·  Check all control values for correct validation and storage/retrieval.

·  Check end-cases for all fields.

·  Verify Tab order, keyboard accelerators, and double-clicks.

·  Verify that phone book entries can be added, deleted, and edited.

## 6.3.  Test Problems

·	Bizarre error conditions are initially tested by twiddling bits with the debugger.  This is, however, time consuming and typically gets left out of regressions.

# 7.  Dependencies

·	BLT and COW

·	RAS Manager (Win16, Win32, and DOS)

·	External RAS APIs

·	RASFILE library

# 8.  Work Estimates

Windows RASPHONE:

| Days | Task Description |
| --- | --- |
| 2.0 | Set up Windows build environment |
| 2.0 | Review changes to BLT since 1.0 in detail |
| 2.0 | Code menu and application window |
| 2.0 | Code main window listbox |
| 3.0 | Code main window toolbar |
| 3.0 | Code main window status bar |
| 1.0 | Code entry toolbar |
| 1.0 | Code modem settings dialog |
| 2.0 | Code X.25 settings dialog |
| 1.0 | Code switch settings dialog |
| 2.0 | Code callback number dialog |
| 2.0 | Code logon retry dialog |
| 4.0 | Code dial dialog and hangup confirm box |
| 4.0 | Code status list mechanism |
| 4.0 | Code terminal mode |
| 1.0 | Port ERRORMSG class from RASADMIN class |
| 2.0 | Code port status dialog |
| 4.0? | SNA entry stuff (TBD) |
| 4.0? | SNA non-entry stuff (TBD) |
| 1.0 | Code about dialog |
| 1.0 | Code help hooks |
| 5.0 | Unit test Sparta version |
| 1.0 | Set up Win NT environment |
| 2.0 | Add Win NT specific ifdefs |
| 1.0 | Build Win NT version |
| 2.0 | Unit test Win NT version |
| 1.0 | Code review |
| 2.0 | Code review changes |
| ----- |  |
| 62.0 |  |

DOS RASPHONE:

| Days | Task Description |
| --- | --- |
| 2.0 | Set up DOS build environment |
| 2.0 | Review COW operation in detail |
| 1.0 | Code menu and status bar changes |
| 1.0 | Code communication settings changes |
| 1.0 | Code modem settings dialog |
| 2.0 | Code X.25 settings dialog |
| 1.0 | Code switch settings dialog |
| 1.0 | Code dial dialog and hangup confirm box |
| 2.0 | Code status list mechanism |
| 2.0 | Code terminal mode |
| 2.0 | Port ERRORMSG class to C form |
| 4.0? | SNA entry stuff (TBD) |
| 4.0? | SNA non-entry stuff (TBD) |
| 3.0 | Unit test DOS version |
| 1.0 | Code review |
| 2.0 | Code review changes |
| ----- |  |
| 31.0 |  |

# 9.  Open Issues

See ISSUES: comments in above sections.