####################################################### 90

From v-simonf Thu Sep 26 10:48:44 1991

To: daveh

Subject: vdd spec

Date: Thu Sep 26 17:37:00 1991

I don't know how well this is going to transmit - there's a large lightning

storm about and it's affecting the lines...

VDD interfaces for NTVDM

(rev 1)

Jargon Note: In 'Insignia Speak' this document would be entitled

'Adapter Interfaces for SoftPC'. Thus read references to

'adapters' as VDD's and references to 'SoftPC' as NTVDM! Your

'target' in this document refers to the hardware or subsystem that

you wish to emulate. Feedback is very welcome on areas of the

document that are confusing be they due to jargon or technical

practice differences. Note also that spelling is English rather

than American!

The following Insignia defined data types are used in this

document:

half_word   - unsigned 8 bits.

word                  - unsigned 16 bits.

io_addr     - unsigned 16 bits.

double_word - unsigned 32 bits.

sys_addr    -r further details.

There are three main ways in which a VDD for a new card or

subsystem / driver can interface with the underlying 'PC': IO Port

access, Interrupts and Memory Accesses.

Note that this revision of the document does not consider how to

dynamically install VDD's.

IO Ports

=========

When port IN or OUT instructions are encountered in the emulated

Intel instruction stream, the cpu routes the request through to

the 'in' or 'out' routine of the appropriate nature as below:

out  -  outb()        byte write

-  outw()   word write

-  outs()   byte string write

-  outw()   word string write

in   -  inb() byte read

-  inw()   word read

-  ins()   byte string read

-  inw()   word string read

These routines perform table lookup to obtain a function pointer

to the appropriate adapter function for that port and then call

it.

To support a new adapter you should provide functions of the

form xxx_outb, xxx_inb etc (see below) as required for your

emulated target. All empty ports are initialised to act as 'sinks'

for written data and to return 'FF's for reads.

VOID  xxx_outb(IN io_addr port, IN half_word value)

VOID  xxx_outw(IN io_addr port, IN word value)

VOID  xxx_outsb(IN io_addr port, IN half_word *valarray,

IN word count)

VOID  xxx_outsw(IN io_addr port, IN word *valarray,

IN word count)

- where

port is the port value (or initial port value) specified

in the OUT instruction.

value is the value written to that port.

valarray is a series of values written to the port.

count is the number of values written to the port.

VOID  xxx_inb(IN io_addr port, OUT half_word *value)

VOID  xxx_inw(IN io_addr port, OUT word *value)

VOID  xxx_insb(IN io_addr port, OUT half_word *valarray,

IN word count)

VOID  xxx_insw(IN io_addr port, OUT word *valarray,

IN word count)

- where

port is the port value (or initial port value) specified

in the IN instruction.

value is a pointer to a memory location that should be

updated by the routine with the value to be read from

the port.

valarray is a pointer to the start of a memory

location that should be updated by the routine with

the values to be read from the port.

count is the number of values to be read from the port.

To link your new routines into the IO port you must first choose

a new 'adapter index' for your target. This is used by the IO

system for function lookup. Typically you will need an adapter

index for every port you define where the functionality of that

port differs greatly from the other ports in your target. As the

port value is available as a parameter to the io routine then it

is possible to group ports together into single entry points for

convenience. The existing indices are held in 'base\inc\ios.h'.

Their range is from EMPTY_ADAPTOR (0) to IO_MAX_NUMBER_ADAPTORS.

(Yes, the spelling does conflict!!) You must increase the value of

IO_MAX_NUMBER_ADAPTORS by the number of adapters you are adding.

This file is also used as a central location for the port ranges

of adapters. You will see the adapter ranges nominated as #defines

in this file.

To attach your new adapter to the IO system you should call the

following routines as approprate in your adapter initialisation.

VOID io_define_in_routines(

IN half_word adapter_index,

IN VOID (*inb_func)(),

IN VOID (*inw_func)(),

IN VOID (*insb_func)(),

IN VOID (*insw_func)() )

- where

adapter_index is the new index defined above.

the functions are the xxx_in routines defined above.

VOID io_define_out_routines(

IN half_word adapter_index,

IN VOID (*outb_func)(),

IN VOID (*outw_func)(),

IN VOID (*outsb_func)(),

IN VOID (*outsw_func)() )

- where

adapter_index is the new index defined above.

the functions are the xxx_in routines defined above.

Should your target have a simple interface where all types of port

IO can be expressed in terms of the byte case, you can call the

simpler interfaces:

VOID io_define_inb(IN half_word adapter_index, IN void (*func)())

VOID io_define_outb(IN half_word adapter_index, IN void (*func)())

- where

adapter_index is the new index defined above.

func is the xxx_inb or xxx_outb routine defined above.

In this case, should a word write occur, the xxx_outb routine

would automatically be called twice: once with the passed port

address and the low byte of the word and then again with the

passed port address + 1 and the high byte of the word. String byte

writes would make repeated calls to the xxx_outb routine with

successive values from the string. Reads are handled analagously.

Once the adapter functions are in place, it is necessary to attach

the relevant ports to the adapter. This is achieved with the

following interface.

VOID io_connect_port(IN io_addr port,

IN half_word adapter_index,

IN half_word mode);

- where

port is the port index.

adapter_index is as used to define the IO routines.

mode is one of IO_READ, IO_WRITE or IO_READ_WRITE which

specifies the nature of the port for INs and OUTs.

To 'unplug' your adapter if you wish to indicate it's

unavailability, the following interface is used:

VOID io_disconnect_port(IN io_addr port,

IN half_word adapter_index)

- where

port is the port index.

adapter_index is as used to define the IO routines.

This will redirect further IO to this port to the empty adapter

routines.

Interrupts

===========

Should you need to generate interrupts from your target then use

the following interface:

void ica_hw_interrupt(IN int ms, IN half_word line, IN int count)

- where

ms is ICA_MASTER or ICA_SLAVE as appropriate.

line is the interrupt line to raise.

count allows a batch of interrupts to be delivered but will

usually be 1.

Should you need to lower an interrupt line (assuming the CPU has

not already been interrupted) use the following interface:

void ica_hw_interrupt_cancel(IN int ms, IN half_word line)

- where

ms is ICA_MASTER or ICA_SLAVE as appropriate.

line is the interrupt line to lower.

Memory

=======

All acceses to the PC memory are done via the 'sas' (System

Address Space) interface functions. These functions are used as

they perform checking on the memory changed/accessed that is

important for maintaining SoftPC operation consistency. The 'sas'

interfaces are as follows:

Memory Reads:

half_word sas_hw_at(IN sys_addr addr)

- return half_word from Intel address 'addr'.

void sas_load(IN sys_addr address, OUT half_word *value)

- load half_word from Intel address 'addr' into 'value'

location.

word sas_w_at(IN sys_addr addr)

- return word from Intel address 'addr'.

void sas_loadw(IN sys_addr address, OUT word *value)

- load word from Intel address 'addr' into 'value' location.

double_word sas_dw_at(IN sys_addr addr)

- return double_word from Intel address 'addr'.

void sas_loads(IN sys_addr src,

OUT host_addr dest,

IN double_word count);

- load 'count' bytes from Intel address 'src' into host

memory at 'dest'

Memory Writes:

void sas_store(IN sys_addr addr, IN half_word value)

- store half_word 'value' at Intel address 'addr'.

void sas_storew(IN sys_addr addr, IN word value)

- store word 'value' at Intel address 'addr'.

void sas_storedw(IN sys_addr addr, IN double_word value)

- store double_word 'value' at Intel address 'addr'.

void sas_stores(IN sys_addr addr,

IN host_addr valarray,

IN double_word count);

- store 'count' bytes from host memory 'valarray' into

Intel memory at 'addr'.

void sas_fills(IN sys_addr addr,

IN half_word value,

IN double_word count);

- fill 'count' bytes of Intel memory starting at 'addr' with

the byte 'value'.

void sas_fillsw(IN sys_addr addr,

IN word value,

IN double_word count);

- fill 'count' words of Intel memory starting at 'addr' with

the word 'value'.

Memory Moves.

void sas_move_bytes_forward(IN sys_addr src,

OUT sys_addr dest,

IN double_word count);

- copy 'count' bytes from the Intel memory that starts at 'src'

into the Intel memory that starts at 'dest'.

void sas_move_bytes_backward(IN sys_addr src,

OUT sys_addr dest,

IN double_word count);

- copy 'count' bytes from the Intel memory that starts at 'src'

into the Intel memory that starts at 'dest'. 'src' and 'dest'

are the highest Intel locations affected and the memory below

those locations is used (hence 'backwards').

void sas_move_words_forward(IN sys_addr src,

OUT sys_addr dest,

IN double_word count);

- copy 'count' words from the Intel memory that starts at 'src'

into the Intel memory that starts at 'dest'.

void sas_move_words_backward(IN sys_addr src,

OUT sys_addr dest,

IN double_word count);

- copy 'count' words from the Intel memory that starts at 'src'

into the Intel memory that starts at 'dest'. 'src' and 'dest'

are the highest Intel locations affected and the memory below

those locations is used (hence 'backwards').

Debugging

==========

SoftPC has a built in debugger 'Yoda'. The following are some

notes on the use of Yoda.

Yoda is a useful tool but has some very rough edges as it is an

accumulation of useful functions that have been accumulated over a

number of SoftPC ports and versions. Some commands will have no

relevance to you and you will probably find some areas lacking in

commands.

How to get into Yoda:

If you set an environment variable YODA then when you start up

SoftPC it will present you with a disassembly of the first CPU

instruction, a register & flag dump and the 'yoda> ' prompt on

your debugging screen. Note you can give the command 'h' to this

prompt to print a list of commands but beware the somewhat

haphazard nature of the list. You can also enter Yoda at any time

during a SoftPC session by hitting the 'Break' key on the PC

keyboard which is temporarily wired to call Yoda. This produces a

disassembly of the current instruction and the yoda> prompt. The

prompt is given before the instruction is executed giving the user

the opportunity to watch the exact effect of the instruction. Yoda

assumes all numbers given to him are hex - 0x prefixes or 'h'

postfixes will probably just confuse.

Program control:

Yoda uses a pseudo interrupt between each instruction to single

step. To enable this interrupt give the yoda command 'yint'. This

enables the interrupt. SoftPC will then run in one of two modes

controlled by the commands 'fast' (run at near normal pace) or

'slow' (check state between each instruction). In order to single

step the application or to use breakpoints you must be in 'slow'

mode so give that command after the 'yint' command. The 'fast'

mode is very useful if you want to maintain your breakpoint state

but run quickly through an app. perhaps to get back to the start

of a dubious section. You can then control the program with the

following commands:

s - single step.

j - jump over call or int.

c - continue execution.

? - where am I.

u seg:off len - unassemble memory. NB 'u .' means unassemble from

current location.

t - enable verbose tracing of every instruction executed.

nt - disable    "     "    "     "      "          "

If you discover, whilst tracing through an application that you

seem to be spending all your time in the timer interrupt code then

issue the command 'toff' - this will prevent the timer interrupt

being delivered to the application. To re-enable the interrupt

issue the command 'ton'.

Breakpoints:

For breakpoints to work, Yoda must be running in slow mode. Yoda

can breakpoint on a range of conditions.

Executing a certain address - bi seg:off len type

where type==1 means break but type== 0 means trace and continue.

Executing a certain opcode (8, 16 or 32 bit) - bo opcode

Access to a certain port - ba port

A change to a PC memory location - bw seg:off len type

A change to a host memory location (good for spotting the dreaded

memory scribble) - bh hostaddr len type.

Executing any interrupt - bint

Executing a specific interrupt for a given AH - bintx intno ah

For each bX command there are equivalent 'p' and 'c' commands. eg

bo breaks on opcodes. 'po' prints the active opcode breakpoints

and 'co' deletes all the opcode breakpoints (sorry, there is no

way of deleting an individual opcode breakpoint).

A very useful command to know about is 'pla' (print last address).

This  prints the addresses of the last few instructions executed

before the one you have breakpointed on. Not very exciting if you

are stepping through a sequence of code, but invaluable if the app

has just jumped to this code location from another part of the

app. Beware though, 'pla' is not infallible, it sometimes can be

fooled.

Memory:

If the instruction unassembled references a memory location,

Yoda will attempt to show what that memory location currently

holds. Memory can be displayed with the following commands:

db seg:off len - display bytes

dw seg:off len - display words (16 bit)

da seg:off len - display memory in both hex & ascii.

To modify memory use the following commands:

byte seg:off value

word seg:off value

Sorry, no general memory edit command.

Registers:

The standard Yoda display will give a register and flag dump.To

get a dump of the current registers use the 'r' command. Some of

the 'protected mode' registers are missing from this display as

they are too unwieldy and these have their own print commands (eg

pgdt to print the global descriptor table). See the last dozen or

so entries in the help listing.To modify the current value of a

register then you enter a command that is the name of the register

followed by the new value. eg to set AX to ff00 you would use

ax ff00

Note that changing CS & IP will probably not have effect for the

current instruction.

VDD/Adapter tracing.

The current activity of various adapters can be determined by

setting the appropriate bit in the 'io_verbose' or

'sub_io_verbose' variables. If 'their' bit is set the adapters

will give various debugging information about their activities.

See 'base\inc\trace.h' for the appropriate bit settings used by

various adapters. To change the settings of the 'verbose'

variables use the yoda commands:

it value - change io_verbose to 'value'.

sit value - change sub_io_verbose to 'value'.