Installable Virtual Device Driver (VDD) Support For NTVDM

==========================================================

Problem:

--------

There is a class of DOS applications which run on their

own custom hardware. Generally these applications will

have a pluggable card and a 16bit device driver for their

card. As these cards are beyond the scope of normal PC

architecture, NTVDM cannot virtualize them in a secure

manner. So all such applications are not supported under

NTVDM.

Solution:

---------

If an ISV is writing an NT native device driver for such a

card, it is technically quite simple to provide the support

such that the DOS application runs unmodified and in a secure

fashion. The vendor has to write a Virtual Device Driver (VDD)

which will virtualize the card for the DOS application by calling

the native device driver.

-----------------------

|			  |

|	DOS Application	  |	  V86 mode

|			  |

-----------------------

I/O-map |	 ^     |Mem-Map	| DMA

IO      |	 |     |IO	|

-------------|-----------------------------

|  |     |	|

V  |     V	V

-----------------------

|		   | DMA  |

|	    NTVDM  -------|	  NT User mode code

|			  |

-----------------------

^     |

|     | Dispatches the event to VDD

|     V

-----------------------

|			  |

|	     VDD	  |	  VDD is a DLL attached to NTVDM

|			  |

-----------------------

^     |

|     | Call the real driver

-------------|-----------------------------

|     |

|     V

-----------------------

|			  |

|  NT Device Driver	  |	  Kernel mode

|			  |

-----------------------

^     |

|     | Carries out the operation with its card

|     V

-----------------------

|			  |

|	 Plugged Card	  |

|			  |

-----------------------

Following are the main work items to achieve the above solution:

a. Loading the VDD

b. Support for I/O mapped I/O

c. Support for Memory mapped I/O

d. Support for DMA operations

e. Register manipulation services

f. Memory accessing services

g. Interrupt simulation services

a. Loading the VDD:

The system administrator will add the command lines in the WOW

section of the registry for all the VDDs to be loaded in the

VDM process. The command line format will be:

[WOW]

VDD = <full-path of the VDD1.DLL>

VDD = <full-path of the VDD2.DLL>

VDD ACTION :

NTVDM will load all these VDDs from the registry at the VDM process

initialization time and call their initialization routine. VDDs should

make sure at this time that their respective NT device driver is

present and make all the resource allocations. The VDD handle passed

in this initialization routine will be the Id of the VDD when calling

the VDD services as described below.

b. Support for I/O mapped I/O:

Following services will be provided for VDDs to deal with  IO

ports:

BOOL VDDInstallIOHook (HANDLE, IO_PORT_RANGE, IO_PORT_HANDLERS);

BOOL VDDDeInstallIOHook (HANDLE, IO_PORT_RANGE);

The HANDLE is the one passed to the VDD in its init routine. The earlier

IO hook will be overwritten. On DeInstalling, the default IO hook will be

placed, which means no one is hooked on those IO ports. IO_PORT_HANDLERS

should atleast provide a byte read and a byte write handler. In addition,

word and string handlers can also be provided. In the absense of word

or string handlers, they will be emulated using byte handlers.

A port has to be hooked for both read and write. VDDs should not hook

DMA ports as these are virtualized by NTVDM and services are provided

for VDDs to access DMA.

VDD Action:  On an IO access (or on a series of IO accesses) the VDD

can check if it needs to start the DMA. If so it can call

the DMA services given in the following section. If it

does'nt require the DMA or if the DMA has transfered the

buffer, then the VDD can call its NT device driver to

complete the desired request. (Its also possible that the

VDD first requests the device driver and than using DMA

services copies the contents to the VDM buffer).

c. Support for Memory mapped I/O:

Following services will be  provided for VDDs to deal with their

memory mapped addresses:

BOOL VDDInstallMemoryHooks (HANDLE, ADDR_RANGE, MEMORY_HANDLER);

BOOL VDDDeInstallMemoryHooks (HANDLE, ADDR_RANGE);

The addr_range should be a valid range i.e. above RMSIZE

and below system rom. The earlier memory hook will be overwritten.

On deinstalling, the range will map to invalid memory and any access

will result in an unhandled page fault. Memory_handler will just tell

on which address the fault occured (it wont say whether it was read or write

operation or what was the operand value etc.). On its return from the

memory handler it will be assumed that the fault was handled.

A port-range will actually result in a whole page to be reserved. That

means the page will not remain available to EMM for EMM page frames and

UMBs.

VDD Action: It can map the normal memory and let the app write to it.

Later they can use the WIN32 API's or its device driver

as per the case to pass the whole memory range.

d. Support for DMA operations

Following DMA service will be provided for the VDD:

BOOL VDDRequestDMA (HANDLE, DMA_CHANNEL, BUFFER, TRANSFER_COUNT);

BOOL VDDQueryDMA (HANDLE, DMA_CHANNEL, DMA_INFO_BUFFER);

BOOL VDDSetDma (HANDLE, DMA_CHANNEL, INDEX, DMA_INFO_BUFFER);

NTVDM will control all the DMA ports and will maintain all the

information on per channel basis. There are two flavors in which

a VDD can carry-on the DMA operations. It can call VDDRequestDMA

and this service will interpret the DMA registers and do the

DMA transfer. It should be clear at this point that this will

involve two buffer copyings. For example, the VDD will ask the device

driver to trasnfer some data in buffer (allocated by VDD) then it will

call this service to transfer this buffer to the address DOS application

has asked for (through DMA programming).

On the other hand, a VDD can collect all the DMA registers using

VDDQueryDMA and figure out where the DOS application	has asked the

DMA to take place, in what mode and how much to transfer. Then it can

call the NT device driver with same address as asked by DOS app. In this

case there will be only one copying. VDD should use VDDSetDMA after

such an operation to update the DMA state.

e. Register manipulation services:

ULONG	getEAX(VOID);

USHORT getAX(VOID);

UCHAR	getAL(VOID);

UCHAR	getAH(VOID);

ULONG	getEBX(VOID);

USHORT getBX(VOID);

UCHAR	getBL(VOID);

UCHAR	getBH(VOID);

ULONG	getECX(VOID);

USHORT getCX(VOID);

UCHAR	getCL(VOID);

UCHAR	getCH(VOID);

ULONG	getEDX(VOID);

USHORT getDX(VOID);

UCHAR	getDL(VOID);

UCHAR	getDH(VOID);

ULONG	getESP(VOID);

USHORT getSP(VOID);

ULONG	getEBP(VOID);

USHORT getBP(VOID);

ULONG	getESI(VOID);

USHORT getSI(VOID);

ULONG	getEDI(VOID);

USHORT getDI(VOID);

ULONG	getEIP(VOID);

USHORT getIP(VOID);

USHORT getCS(VOID);

USHORT getSS(VOID);

USHORT getDS(VOID);

USHORT getES(VOID);

USHORT getFS(VOID);

USHORT getGS(VOID);

ULONG	getCF(VOID);

ULONG	getPF(VOID);

ULONG	getAF(VOID);

ULONG	getZF(VOID);

ULONG	getSF(VOID);

ULONG	getIF(VOID);

ULONG	getDF(VOID);

ULONG	getOF(VOID);

USHORT getMSW(VOID);

VOID setEAX(ULONG);

VOID setAX(USHORT);

VOID setAH(UCHAR);

VOID setAL(UCHAR);

VOID setEBX(ULONG);

VOID setBX(USHORT);

VOID setBH(UCHAR);

VOID setBL(UCHAR);

VOID setECX(ULONG);

VOID setCX(USHORT);

VOID setCH(UCHAR);

VOID setCL(UCHAR);

VOID setEDX(ULONG);

VOID setDX(USHORT);

VOID setDH(UCHAR);

VOID setDL(UCHAR);

VOID setESP(ULONG);

VOID setSP(USHORT);

VOID setEBP(ULONG);

VOID setBP(USHORT);

VOID setESI(ULONG);

VOID setSI(USHORT);

VOID setEDI(ULONG);

VOID setDI(USHORT);

VOID setEIP(ULONG);

VOID setIP(USHORT);

VOID setCS(USHORT);

VOID setSS(USHORT);

VOID setDS(USHORT);

VOID setES(USHORT);

VOID setFS(USHORT);

VOID setGS(USHORT);

VOID setCF(ULONG);

VOID setPF(ULONG);

VOID setAF(ULONG);

VOID setZF(ULONG);

VOID setSF(ULONG);

VOID setIF(ULONG);

VOID setDF(ULONG);

VOID setOF(ULONG);

VOID setMSW(USHORT);

f: Memory Accessing services:

/** GetVDMPointer - Findout the linear address of a given VDM address

*

*   INPUT

*	Address - seg/sel:off (hi word has segment or selector and loword

*		  is offset)

*	Size	- Range of the pointer

*	ProtectMode - If protecmeode == TRUE its sel:off

*		      If protecmeode == FALSE its seg:off

*

*   OUTPUT

*	Returns Linear address.

*/

PVOID GetVDMPointer(ULONG Address, ULONG Size, BOOL ProtectedMode);

/** FlushVDMPointer - Flushes the contents (required because of emulator)

*

*   INPUT

*	Address - seg/sel:off (hi word has segment or selector and loword

*		  is offset)

*	Size	- Range of the pointer

*	Buffer	- Address returned by GetVDMPointer.

*	ProtectMode - If protecmeode == TRUE its sel:off

*		      If protecmeode == FALSE its seg:off

*

*   OUTPUT

*	Returns Linear address.

*/

BOOL  FlushVDMPointer(ULONG Addr, ULONG Size, PVOID Buffer, BOOL ProtectedMode);

/** FreeVDMPointer - Frees a pointer previously returned by GetVDMPointer

*

*   INPUT

*	Address - seg/sel:off (hi word has segment or selector and loword

*		  is offset)

*	Size	- Range of the pointer

*	ProtectMode - If protecmeode == TRUE its sel:off

*		      If protecmeode == FALSE its seg:off

*

*   OUTPUT

*	None

*/

BOOL  FreeVDMPointer(ULONG Address, ULONG Size, BOOL ProtectedMode);

g: Interrupt simulation services

/** VDDSimulateInterrupt - Simulates an interrupt to the VDM.

*

*   INPUT

*	ms    -	Is either ICA_MASTER or ICA_SLAVE as appropriate

*	line  - Interrupt line

*	count - allows a batch of interrupts to be delivered but will usually

*		be 1.

*

*   OUTPUT

*	None

*/

VOID VDDSimulateInterrupt (BYTE ms, BYTE line, WORD count);

----------------VDD SERVICES INTERFACES-------------------------

/** Basic typedefs of VDD IO hooks **/

typedef VOID (*PFNVDD_INB)   (WORD iport,BYTE * data);

typedef VOID (*PFNVDD_INW)   (WORD iport,WORD * data);

typedef VOID (*PFNVDD_INSB)  (WORD iport,BYTE * data,WORD count);

typedef VOID (*PFNVDD_INSW)  (WORD iport,WORD * data,WORD count);

typedef VOID (*PFNVDD_OUTB)  (WORD iport,BYTE data);

typedef VOID (*PFNVDD_OUTW)  (WORD iport,WORD data);

typedef VOID (*PFNVDD_OUTSB) (WORD iport,BYTE * data,WORD count);

typedef VOID (*PFNVDD_OUTSW) (WORD iport,WORD * data,WORD count);

/**  Array of handlers for VDD IO hooks. **/

typedef struct _VDD_IO_HANDLERS {

PFNVDD_INB	 inb_handler;

PFNVDD_INW	 inw_handler;

PFNVDD_INSB	 insb_handler;

PFNVDD_INSW	 insw_handler;

PFNVDD_OUTB	 outb_handler;

PFNVDD_OUTW	 outw_handler;

PFNVDD_OUTSB outsb_handler;

PFNVDD_OUTSW outsw_handler;

} VDD_IO_HANDLERS, *PVDD_IO_HANDLERS;

/**  VDD handler for memory mapped addresses. **/

typedef VOID (*VDD_MEMORY_HANDLER) (PVOID)

/** Buffer for returning DMA information **/

typedef struct _VDD_DMA_INFO {

WORD    addr;

WORD    count;

WORD    page;

BYTE    status;

BYTE    mode;

BYTE    mask;

} VDD_DMA_INFO, *PVDD_DMA_INFO;

/** Index numbers for querying the DMA information **/

#define VDD_DMA_ADDR	0

#define VDD_DMA_COUNT	1

#define VDD_DMA_PAGE	2

#define VDD_DMA_STATUS	3

#define VDD_DMA_MODE	4

#define VDD_DMA_MASK	5

#deine	VDD_DMA_ALL	0xff

/*** VDDInstallIOHook - This service is provided for VDDs to hook the

*			IO ports they are responsible for.

*

* INPUT:

*	hVDD	: VDD Handle

*	iPort	: Starting Port Number

*	cPort	: Number of ports

*	IOhandler : VDD handler for the ports.

*

* OUTPUT

*	SUCCESS : Returns TRUE

*	FAILURE : Returns FALSE

*		  GetLastError has the extended error information.

*

* NOTES:

*	1. The last one to hook a port will get the control. There

*	   is no concept of chaining the hooks.

*

*	2. IOHandler must atleast provide a byte read and a byte write

*	   handler. Others can be NULL.

*

*	3. If word or string handlers are not provided, their effect

*	   will be emulated using byte handlers.

*

*	4. VDDs should not hook DMA ports. NTVDM manages it for all

*	   the clients and services are provided to perform DMA

*	   operations and to access and modify DMA data.

*

*	5. VDDs should not hook video ports as well. Such a hooking

*	   will succeed but there is no gurantee that the IO handler will

*	   get called.

*/

BOOL VDDInstallIOHook (

HANDLE hVDD,

WORD   iPort,

WORD   cPort,

PVDD_IO_HANDLERS IOhandler

);

/*** VDDDeInstallIOHook - This service is provided for VDDs to unhook the

*			  IO ports they have hooked.

*

* INPUT:

*	hVDD	: VDD Handle

*	iPort	: Starting Port Number

*	cPort	: Number of ports

*

* OUTPUT

*	None

*

* NOTES

*

*	1. On Deinstalling a hook, the defult hook is placed back on

*	   those ports. Default hook  returns 0xff on reading

*	   and ignores the write operations.

*

*/

VOID VDDDeInstallIOHooks (

HANDLE hVDD,

WORD  iPort,

WORD  cPort

);

/*** VDDInstallMemoryHook - This service is provided for VDDs to hook the

*			    Memory Mapped IO addresses they are resposible

*			    for.

*

* INPUT:

*	hVDD	: VDD Handle

*	addr	: Starting linear address

*	count	: Number of bytes

*	MemoryHandler : VDD handler for the memory addresses

*

*

* OUTPUT

*	SUCCESS : Returns TRUE

*	FAILURE : Returns FALSE

*		  GetLastError has the extended error information.

*

* NOTES

*	1. The last one to hook an address will get the control. There

*	   is no concept of chaining the hooks.

*

*	2. Memory handler will be called with the address on which the

*	   page fault occured. It wont say weather it was a read operation

*	   or write operation or what were the operand value. If a VDD

*	   is interested in such information it has to get the CS:IP and

*	   decode the instruction.

*

*	3. On returning from the hook handler it will be assumed that

*	   the page fault was handled and the return will go back to the

*	   VDM.

*

*	4. Installing a hook on a memory range will result in the

*	   consumption of a full page. After this, the	page will not be

*	   available to EMM.

*/

BOOL VDDInstallMemoryHook (

HANDLE hVDD,

DWORD  addr,

DWORD  count

VDD_MEMORY_HANDLER MemoryHandler

);

/*** VDDDeInstallMemoryHook - This service is provided for VDDs to unhook the

*			      Memory Mapped IO addresses.

*

* INPUT:

*	hVDD	: VDD Handle

*	addr	: Starting linear address

*	count	: Number of addresses

*

* OUTPUT

*	None

*

* NOTES

*	1. On Deinstalling a hook, the memory range becomes invalid.

*	   VDM's access of  this memory range will cause a page fault.

*

*/

VOID VDDDeInstallMemoryHook (

HANDLE hVDD,

PVOID  addr,

DWORD  count

);

/*** VDDRequestDMA - This service is provided for VDDs to request a DMA

*		     transfer.

*

* INPUT:

*	hVDD	 VDD Handle

*	iChannel DMA Channel on which the operation to take place

*	Buffer	 Buffer where to or from transfer to take place

*	length	 Transfer Count (in bytes)

*

* OUTPUT

*	SUCCESS : Returns TRUE

*	FAILURE : Returns FALSE

*		  GetLastError has the extended error information.

*

*

* NOTES

*	1. This service is intended for those VDDs which do not want to

*	   carry on the DMA operation on their own. Carrying on a DMA

*	   operation involves understanding all the DMA registers and

*	   figuring out what has to be copied, from where and how much.

*

*	2. This service will be slower than using VDDQueryDMA/VDDSetDMA and

*	   doing the transfer on your own.

*

*/

BOOL VDDRequestDma (

HANDLE hVDD,

WORD  iChannel,

PVOID Buffer,

WORD  length

);

/*** VDDQueryDMA -   This service is provided for VDDs to collect all the DMA

*		     data.

*

* INPUT:

*	hVDD	 VDD Handle

*	iChannel DMA Channel for which to query

*	Buffer	 Buffer where information will be returned

*

* OUTPUT

*	SUCCESS : Returns TRUE

*	FAILURE : Returns FALSE

*		  GetLastError has the extended error information.

*

*

* NOTES

*	1. This service is intended for those VDD which are doing

*	   performance critical work. These VDD can do their own DMA

*	   transfers and avoid one extra buffer copying which is a

*	   overhead in using VDDRequestDMA.

*

*	2. VDDs should use VDDSetDMA to properly update the state of

*	   DMA after carrying on the operation.

*

*/

BOOL VDDQueryDMA (

HANDLE hVDD,

WORD iChannel,

PVDD_DMA_INFO Buffer

);

/*** VDDSetDMA - This service is provided for VDDs to set the DMA data.

*

* INPUT:

*	hVDD	 VDD Handle

*	iChannel DMA Channel for which to query

*	index	 DMA data fields which are to be set

*		    VDD_DMA_ADDR

*		    VDD_DMA_COUNT

*		    VDD_DMA_PAGE

*		    VDD_DMA_STATUS

*		    VDD_DMA_MODE

*		    VDD_DMA_MASK

*		    VDD_DMA_ALL

*	Buffer	 Buffer with DMA data

*

* OUTPUT

*	SUCCESS : Returns TRUE

*	FAILURE : Returns FALSE

*		  GetLastError has the extended error information.

*

*

* NOTES

*

*/

BOOL VDDSetDMA (

HANDLE hVDD,

WORD iChannel,

WORD index,

PVDD_DMA_INFO Buffer

);