Windows NT 4.0 RasDD Modifications to Support Minidriver Custom UI

Version 1.0

July 29, 2026

Jason Staczek

Amanda Nguyen

Daniel Chou

Ganesh Pandey

Table of Contents

1. Introduction

2. Conventions

3. Custom UI Support at Driver Setup Time

3.1 Locating The Minidriver UI DLL

3.2 Minidriver UI DLL Capabilities

3.2.1 DEVMODE Support

3.2.2 Adding Treeview Nodes in Device Options and Advanced Tabs

3.2.3 UI Processing

3.3 Other Minidriver UI APIs

3.3.1 DevQueryPrintEx

3.3.2 DrvDeviceCapabilities

3.3.3 DrvUpgradePrinter

4. Print Time Support

4.1 Public PDEV fields

4.2 Minidriver Exports

4.2.1 OEMEnableDriver

4.2.2 OEMEnablePDEV

4.2.3 OEMDisablePDEV

4.2.4 OEMResetPDEV

4.2.5 OEMDisableDriver

4.2.6 OEMDevMode

4.2.7 OEMCommandCallback

4.2.8 OEMFilterGraphics

4.2.9 OEMDump

4.3 RasDD Helper Functions

4.3.1 RasDDAlloc

4.3.2 RasDDFree

4.3.3 RasDDPDEVStateChange

4.3.4 RasDDItemInfo

4.3.5 RasDDWriteSpoolBuf

4.3.6 RasDD OEMDDI Functions

4.4 Execution Path

5. Interoperability

6. Bi-directional Support / JetAdmin

7. Items Not Covered

8. Issues/Questions

9. Reference

9.1 Minidriver UI DLL Entry Points (user-mode)

9.2 Minidriver  DLL Entry Points (kernel-mode)

# Introduction

This document describes the modifications required for RasDD and RasDDUI to support a minidriver supplied custom user interface under Windows NT 4.0.

Currently, RasDD handles all user interaction for the minidriver. The minidriver’s behavior is described through data only, located in the GPC. There is no mechanism for a minidriver to add its own user interface code or modify RasDD’s user interface presentation. These modifications extend RasDDUI to allow the minidriver to add user interface code for printer features which are not supported in the GPC. Note that these modifications do not require any changes to existing minidriver data. All existing minidrivers will continue to operate without modification.

To complete support for printer features not described in the GPC, RasDD must also allow the minidriver to modify the print stream at print time. This will be accomplished through command callbacks, the same mechanism used by the Windows 95 Unidriver. Again, these changes will not affect existing minidrivers.

# Conventions

To reduce the need to add additional APIs (…Ex) as minidriver UI requirements change, the minidriver APIs will be constructed to take a single parameter, a pointer to a structure which contains a size field and the arguments to the function. In future versions, the size of the argument structure may grow, but existing fields will not be removed. See OEMDevMode in section 3.2.1 for example. Exceptions are granted where an API reasonably expects only one or two arguments.

# Custom UI Support at Driver Setup Time

The minidriver custom UI will be controlled through a new minidriver UI DLL that interacts with RasDDUI and the Common Driver UI  APIs.

## Locating The Minidriver UI DLL

The minidriver DLL will contain a resource of type RC_OEMUIDLL at ID 1 that names the minidriver UI DLL. The resource should contain the null-terminated DLL name only, for example “hp5siui.dll”. RasDDUI will assume that the UI DLL is located in the same directory as the minidriver DLL. For example:

1	   RC_OEMUIDLL PRELOAD MOVEABLE

BEGIN

"hp5siui.dll\0\0"

END

Note that this is not a string resource in order to avoid interference with Unitool’s management of the string table.

## Minidriver UI DLL Capabilities

The minidriver UI DLL will be able to add new nodes to the Device Options treeview in the Print Properties sheet and to the Advanced treeview in the Document Properties sheet. When selected, each node may offer additional control through an area at the bottom of the dialog.

RasDDUI will interact with the minidriver UI DLL through four entry points, OEMDevMode, OEMCommonUI, and two UI callbacks for processing printer properties UI messages and document properties UI messages.

### DEVMODE Support

To support the minidriver custom UI, RasDDUI will allow the minidriver to add private data to the DEVMODE structure. The entire DEVMODE, including public, RasDD extra and minidriver extra will look like this:

To the spooler, it appears that all of the extra data is owned by RasDD. RasDD will call the minidriver to:

- Get the minidriver’s private DEVMODE size

- Get the minidriver’s default private DEVMODE

- Convert an existing minidriver private DEVMODE from a different version

- Validate an existing DEVMODE

This happens through OEMDevMode:

#define OEMDM_SIZE					0x01

#define OEMDM_DEFAULT				0x02

#define OEMDM_CONVERT 			0x03

#define OEMDM_VALIDATE			0x04

typedef struct _OEM_DEVMODEPARAM {

DWORD 			cbSize;					/* size of OEM_DEVMODEPARAM */

DWORD 			fMode;					/* DEVMODE operation */

LPWSTR 			pPrinterModel;

PDEVMODE		pPublicDMIn;

PDEVMODE 		pPublicDMOut;

PVOID 			pOEMDMIn;

PVOID 	 		pOEMDMOut;

DWORD			cbBufSize;			/* size of pOEMDMOut */

LPDWORD			pcbNeeded;

} OEM_DEVMODEPARAM, *POEM_DEVMODEPARAM;

BOOL OEMDevMode(

POEM_DEVMODEPARAM pOEMDevModeParam

);

#### OEMDM_SIZE

To get the size of the minidriver’s extra data, RasDD will call OEMDevMode with pOEMDM->fMode == OEMDM_SIZE. The minidriver should return the size required for its extra data in pOEMDM->pcbNeeded and TRUE to indicate success. If the minidriver returns FALSE, RasDD will assume that the minidriver requires no extra data.

| Field | In | Out |
| --- | --- | --- |
| cbSize | sizeof(OEM_DEVMODEPARAM) | - |
| fMode | OEMDM_SIZE |  |
| pPrinterModel | Printer model name | - |
| pPublicDMIn | - | - |
| pPublicDMOut | - | - |
| pOEMDMIn | - | - |
| pOEMDMOut | - | - |
| cbBufSize |  | - |
| pcbNeeded | - | Size of OEM extra data |

#### OEMDM_DEFAULT

To retrieve default values for the minidriver’s extra data, RasDD will call OEMDevMode with pOEMDM->fMode == OEMDM_DEFAULT.

| Field | In | Out |
| --- | --- | --- |
| cbSize | sizeof(OEM_DEVMODEPARAM) | - |
| fMode | OEMDM_DEFAULT |  |
| pPrinterModel | Printer model name | - |
| pPublicDMIn | - | - |
| pPublicDMOut | Pointer to public DEVMODE, may be 0 if RasDD requires only default OEM extra data. | - |
| pOEMDMIn | - | - |
| pOEMDMOut | Pointer to OEM extra data | - |
| cbBufSize | size of OEM extra data buffer (pOEMDMOut) | - |
| pcbNeeded | - | count of bytes copied to pOEMDMOut |

The minidriver should return the count of bytes copied to pOEMDMOut in pcbNeeded and return TRUE for success. If the output buffer is not sufficiently large to contain the new version, the minidriver should set pcbNeeded to the required size, SetLastError( ERROR_INSUFFICIENT_BUFFER )and return FALSE. Note that the minidriver may modify the public portion of the DEVMODE if pPublicDMOut != 0. pPublicDMOut may be 0 in situations where RasDD requires only default extra data.

Important: The minidriver should never assume that it can locate its extra data from the top of the public DEVMODE. All access to its extra data should happen through pOEMDMIn and pOEMDMOut.

#### OEMDM_CONVERT

RasDD will call OEMDevMode with fMode == OEMDM_CONVERT to allow the minidriver to convert a DEVMODE to the current driver DEVMODE version. The minidriver will be asked to convert only its own extra data.

| Field | In | Out |
| --- | --- | --- |
| cbSize | sizeof(OEM_DEVMODEPARAM) | - |
| fMode | OEMDM_CONVERT |  |
| pPrinterModel | Printer model name | - |
| pPublicDMIn | Pointer to old public DEVMODE | - |
| pPublicDMOut | Pointer to new public DEVMODE | - |
| pOEMDMIn | Pointer to old OEM extra data | - |
| pOEMDMOut | Pointer to new OEM extra data buffer | - |
| cbBufSize | size of OEM extra data buffer (pOEMDMOut) | - |
| pcbNeeded | - | count of bytes copied to pOEMDMOut, or size of buffer required to hold new OEM extra data if > cbBufSize |

The minidriver should convert extra data in pOEMDMIn and copy it to the appropriate location in the output buffer pOEMDMOut. The minidriver should return the count of bytes in the new minidriver extra in pcbNeeded. If the output buffer is not sufficiently large to contain the new version, the minidriver should set pcbNeeded to the required size, call SetLastError(ERROR_INSUFFICIENT_BUFFER) and return FALSE.

#### OEMDM_VALIDATE

RasDD will occasionally require the minidriver to validate that the minidriver extra data in a DEVMODE is correct. RasDD will call OEMDevMode with a pointer to the extra data in question. The minidriver should validate and return TRUE if the extra data is correct and usable, FALSE otherwise. The minidriver should not modify any DEVMODE data during this call.

| Field | In | Out |
| --- | --- | --- |
| cbSize | sizeof(OEM_DEVMODEPARAM) | - |
| fMode | OEMDM_CONVERT |  |
| pPrinterModel | Printer model name | - |
| pPublicDMIn | - | - |
| pPublicDMOut | - | - |
| pOEMDMIn | Pointer to OEM extra data | - |
| pOEMDMOut | - | - |
| cbBufSize | size of OEM extra data buffer (pOEMDMIn) | - |
| pcbNeeded | - | - |

#### RasDDUI’s DRIVEREXTRA

In order to keep the size of RasDD’s DRIVEREXTRA the same as the shipping beta, two unused fields will be converted to accommodate the OEM’s extra data.

typedef struct {

short sVer;

WORD dmSize;

} DMEXTRAHDR;

typedef struct {

short sVer;

WORD dmSize;					/* sVer and dmSize are DMEXTRAHDR */

WORD dmOEMDriverExtra; /* was short dmTextQuality */

WORD dmFields;

WORD wOEMVer;

short dmBrush;

short sCTT;

short dmNumCarts;

short rgFontCarts[MAXCART];

short dmMemory;

short rgindex[MAXHE];

short sFlags;

short Padding;

COLORADJUSTMENT ca;

} DRIVEREXTRA;

Note that dmOEMDriverExtra is the total size of all OEM extra data. Also note that minidrivers must include a DMEXTRAHDR structure as the first field in their extra data:

### Adding Treeview Nodes in Device Options and Advanced Tabs

The minidriver will use Common UI structures to add nodes to the treeviews on the Device Options page and the Document Properties Advanced page.

#### OEMCommonUI

RasDDUI will call the minidriver to get a list of OPTITEMs to append to each treeview. The list may contain any number of nodes and subnodes.

#define OEMUI_DOCPROP			0x01

#define OEMUI_PRNPROP			0x02

typedef struct {

DWORD 					cbSize;

DWORD 					fMode;					// OEMUI_xxxPROP

HPRINTER 				hPrinter;

LPWSTR 					pModelName;

PDEVMODE 				pPublicDMIn;		// public devmode in

PDEVMODE 				pPublicDMOut;		// public devmode out

PVOID					pOEMDMIn;				// OEM extra data in

PVOID					pOEMDMOut;			// OEM extra data out

DWORD					cbBufSize;			// size of pOEMDMOut

DWORD 					cRasddOptItems;

POPTITEM				pOptItemList;

LPDWORD 				pcbNeeded;

DWORD					UIVersion;

DWORD					cOEMOptItems;

DWORD 					OEMDriverUserData;

PCPSUICALLBACK		pfnUICallback;

} OEMPROPERTYHEADER, *POEMPROPERTYHEADER;

WORD OEMCommonUI(

POEMPROPERTYHEADER		pOEMPropertyHeader

};

To build the list, RasDD calls the minidriver twice. Once to get the number of items required then again to get the items. The behavior is the same for Document Properties items and Printer Properties items.

First call gets OEM item count (OEMUI_PRNPROP):

| field | In | Out |
| --- | --- | --- |
| cbSize | sizeof(OEMPROPERTYHEADER) | - |
| fMode | OEMUI_PRNPROP | - |
| hPrinter | Handle to current printer | - |
| pModelName | Printer model name | - |
| pPublicDMIn | Pointer to public DEVMODE | - |
| pPublicDMOut | used during Common UI callbacks | - |
| pOEMDMIn | Pointer to OEM extra data | - |
| pOEMDMOut | used during Common UI callbacks | - |
| cbBufSize | size of pOEMDMIn | - |
| cRasddOptItems | - | - |
| pOptItemList | NULL | - |
| pcbNeeded | - | - |
| UIVersion | - | - |
| cOEMOptItems | - | count of Common UI items minidriver wishes to add to treeview |
| OEMDriverUserData | - | - |
| pfnUICallback | - | - |

Second call gets OEM items and callback:

| field | In | Out |
| --- | --- | --- |
| cbSize | sizeof(OEMPROPERTYHEADER) | - |
| fMode | OEMUI_PRNPROP | - |
| hPrinter | Handle to current printer | - |
| pModelName | Printer model name | - |
| pPublicDMIn | Pointer to public DEVMODE | - |
| pPublicDMOut | used during Common UI callbacks | - |
| pOEMDMIn | Pointer to OEM extra data | - |
| pOEMDMOut | used during Common UI callbacks | - |
| cbBufSize | size of pOEMDMIn | - |
| cRasddOptItems | count of Common UI items in RasDD’s OPTITEM list | - |
| pOptItemList | top of RasDD’s OPTITEM list | - |
| pcbNeeded | - | - |
| UIVersion | - | UI DLL version (HIWORD = major version, LOWORD = minor version) |
| cOEMOptItems | number of item slots available to minidriver at the end of RasDD’s OPTITEM list | number of OEM Common UI items copied to the end of RasDD’s OPTITEM list. |
| OEMDriverUserData | - | OEM provided private data. This value will be supplied to the minidriver during Common UI callbacks (see section 3.2.3). |
| pfnUICallback | - | OEM-supplied Common UI callback function. |

#### OPTITEMs

To identify its Common UI items as its own, the minidriver should set each item’s DMPubID to either standard common UI defined DMPUB_xxxx (must not conflict with RasDDUI’s DMPUB_xxx ID) or its own item ID with number greater than or equal to DMPUB_USER. Each item’s UserItemID should be set to a non-zero minidriver supplied ID. Any item with UserItemID zero (0) is the rasdd ui’s item (refer to compstui.h for more information).

Note: OPTITEMs now support help files on a per-item basis, and an HINSTANCE per item to indicate a resource DLL from which to load strings and icons. This allows the minidriver to use string ID’s and icon ID’s in OPTITEMs and let the Common UI handle all resource management.

### UI Processing

During dialog processing, the Common UI normally calls RasDDUI to process messages for node items. RasDDUI will determine whether a node item belongs to the minidriver UI and pass the call on to the minidriver through the minidriver-supplied callback. RasDDUI will set the UserData field in the CPSUICBPARAM structure passed to the callback to the OEMPROPERTYHEADER structure pointer previously supplied to the minidriver in OEMCommonUI. This allows the minidriver access to input and output DEVMODEs and its own private data.

typedef struct _CPSUICBPARAM {

WORD cbSize;         // size of this structure

WORD Reason;         // CPSUICB_REASON_XXXXX callback reason

HWND hDlg;           // handle of the dialog box

POPTITEM pOptItem;   // pOptItem field from COMPROPSHEETUI

WORD cOptItem;       // cOptItem field from COMPROPSHEETUI

WORD Flags;          // flags field from COMPROPSHEETUI

POPTITEM pCurItem;   // current selected item of callback

union {

LONG    OldSel;  // Last selection (index)

LPTSTR  pOldSel; // Last selection (pStr)

} DUMMYUNIONNAME;

DWORD UserData;      // Minidriver’s POEMPROPERTYHEADER

DWORD Result;        // OUT parameter for the APPLYNOW

} CPSUICBPARAM, *PCPSUICBPARAM;

typedef LONG (APIENTRY *_CPSUICALLBACK)(

PCPSUICBPARAM pComPropSheetUICBParam);

During callbacks, the minidriver will have access OPTITEMs as shown below. Note that the minidriver will have access to Rasdd’s items and may show or hide them if necessary.

The minidriver may export two callbacks, one for Printer Properties processing and one for Document Properties processing. The minidriver is required to process all callback reasons defined in compstui.h. For general purpose callback reasons CPSUICB_REASON_APPLYNOW and CPSUICB_REASON_ITEMS_REVERTED, RasDDUI will always call the minidriver supplied callback.

## Other Minidriver UI APIs

Following spooler APIs in the mini driver will have same spooler APIs interface, except the RasDDUI will call mini drivers exported function with same API parameters, the meaning of returning values from mini driver determined the action which RasDDUI will take. If a mini driver does not want to support any of these functions, it simply just not export the function name in the .DEF file.

See the spooler API documentation in the DDK for more detailed descriptions of each function.

Following are each spooler API which mini driver can exported and will called by the RasDDUI if these function exported by the mini driver.

### DevQueryPrintEx

This function allowed spooler to managed mismatch job.  RasDDUI will first check if the job is printable or not printable based on the DEVMODE and device option setting from the GPC file, if the job is not printable then it returned back to the spooler without calling mini driver’s function.   It only call mini drivers DevQueryPrintEx() if the job is printable from RasDDUI point of view, and let the mini driver checking other mini driver’s own features’ job mismatch.

### DrvDeviceCapabilities

This function return device capabilities, all device capabilities are in public DEVMODE.  Most of time this will handle by RasDDUI using GPC and DEVMODE settings.   If a mini driver exported this function, RasDDUI will call mini driver with validated pDevMode first.  If return value from mini driver is GDI_ERROR then RasDDUI will assume that the minidriver does not want to handle that particular device capability.

If mini driver handle a particular device capability, it must fully handle that caps, either caller asked for output size or to fill in the output buffer.  Mini driver is responsible for handling all appropriate DC_xxx reasons related to the capabilities it reported for DM_FIELDS and for providing an appropriate UI for those elements. For instance, if the minidriver wishes to handle UI for duplexing, it would clear the MD_DUPLEX bit in fGeneral in the GPC, but set the DM_DUPLEX bit when processing DM_FIELDS in DrvDeviceCapabilities.

RasDDUI will always handle DC_FIELDS.  It will call mini driver first and then OR in the device capabilities bits which RasDDUI want to handle.

### DrvUpgradePrinter

This API allowed driver has a chance during upgrade to update its device options stored in the registry from one version to the next.   This function only needed if any new registry information need to be added or old registry information needed to be validated or updated. RasDDUI will always process its own upgrade first and then call minidriver’s DrvUpgradePrinter.

# Print Time Support

At print time, RasDD interacts with kernel-mode code in the minidriver DLL. The minidriver may request direct access to DDI calls, or rely on command callbacks from RasDD as in Windows 95.

## Public PDEV fields

RasDD’s PDEV structure will be extended to include public fields which the minidriver can access during callbacks or processing of DDI calls. Specifically, the PDEV will now include a field for the minidriver’s PDEV, an HPRINTER for the current printer and tables of RasDD and minidriver entry points.

typedef struct _RASDDPDEV {

DWORD RasDDID;						// RasDD ID

WORD	cbSize;

WORD wReserved;

DHPDEV OEMPDev;					// Minidriver’s PDEV

HANDLE hPrinter;					// Spooler printer handle

PRASDDFNTBL pRasDDFnTbl;	// RasDD’s entry points

POEMFNTBL	pOEMFnTbl;			// Minidriver entry points

DWORD dwReserved[8];			// for the future, do not touch

..

Other readable fields...

..

} RASDDPDEV, *PRASDDPDEV;

This PDEV will be passed to all minidriver kernel-mode rendering APIs to give the minidriver access to its private data and RasDD’s helper functions.

## Minidriver Exports

### OEMEnableDriver

The minidriver must export OEMEnableDriver. Rasdd will call this at DDI DrvEnablePDEV time to determine which functions the minidriver exports.

#define INDEX_OEMEnablePDEV				INDEX_OEMFirst

#define INDEX_OEMDisablePDEV				INDEX_OEMFirst + 2

#define INDEX_OEMResetPDEV					INDEX_OEMFirst + 3

#define INDEX_OEMDevMode					INDEX_OEMFirst + 4

#define INDEX_OEMFilterGraphics			INDEX_OEMFirst + 5

#define INDEX_OEMDump							INDEX_OEMFirst + 6

#define INDEX_OEMCommandCallback		INDEX_OEMFirst + 7

typedef struct _OEMDRVEABLEDATA {

DWORD	cbSize;

DWORD	DriverVersion;

DRVENABLEDATA* pDeD;

} OEMDRVENABLEDATA, *POEMDRVENABLEDATA;

BOOL OEMEnableDriver(

OEMDRVENABLEDATA	*pOEMDrvEnableData

);

The minidriver should set pDED->pdrvfn to a function table as described in the documentation for DrvEnableDriver. The minidriver must include entries for at least INDEX_OEMEnablePDEV and INDEX_OEMDisablePDEV. It may include entries for any other DDI functions it wishes to handle.

### OEMEnablePDEV

Before RasDD returns its PDEV to GDI, it will call the minidriver’s OEMEnablePDEV to allow it to construct its own PDEV. At this time, the mindriver has access to all RasDD helper functions through the supplied RASDDPDEV. In particular, the minidriver may use RasddAlloc to allocate any storage required for the PDEV.

typedef struct {

DWORD 		cbSize;				// size of this structure

DEVMODEW *pdm,					// start: DDI DrvEnablePDEV parameters

LPWSTR    pwszLogAddress,

ULONG     cPat,

HSURF    *phsurfPatterns,

ULONG     cjCaps,

ULONG    *pdevcaps,

ULONG     cjDevInfo,

DEVINFO  *pdi,

HDEV      hdev,

LPWSTR    pwszDeviceName,

HANDLE	   hDriver,			// end: DDI DrvEnablePDEV parameters

PVOID 		 pOEMDevMode,	// OEM’s extra DEVMODE data

} OEMENABLEPDEVPARAM, *POEMENABLEPDEVPARAM;

DHPDEV OEMEnablePDEV(

PRASDDPDEV pRasddPDEV,

POEMENABLEPDEVPARAM pOEMEPDParam

);

Note that at the time this function called, the minidriver can use information in the DEVMODE to modify the entries in its function table (stored in RasDD’s PDEV). For instance, if the minidriver does not wish to handle DrvTextOut in landscape mode, it may zero the INDEX_DrvTextOut entry in its function table. The minidriver may also do this at during DrvEnableSurface, if it hooks the function, but it should not modify the table at any other time.

### OEMDisablePDEV

VOID OEMDisablePDEV(PRASDDPDEV pDev);

RasDD will call the minidriver’s OEMDisablePDEV to allow the minidriver to free any memory associated with its PDEV. The minidriver should free all allocated resources using RasddFree.

### OEMResetPDEV

BOOL OEMResetPDEV(

PRASDDPDEV pdevOld,

PRASDDPDEV pdevNew);

OEMResetPDEV transfers the state of the driver from the old PDEV to the new PDEV when an application calls ResetDC.

### OEMDisableDriver

VOID OEMDisableDriver(VOID)

OEMDisableDriver is used by RasDD to notify a minidriver that it no longer requires the driver and is ready to unload it.

### OEMDevMode

If the minidriver adds private data to the DEVMODE, it must export a kernel-mode version of the OEMDevMode routine described in section 3.2.1, DEVMODE Support. RasDD will use this at print time to validate, convert and generate default DEVMODEs as necessary. The minidriver behavior should be exactly as described in section 3.2.1. This code can be implemented in a library and shared by the minidriver UI DLL and the minidriver kernel-mode DLL.

### OEMCommandCallback

Support for minidriver modification of the print steam will be implemented through command callbacks as spec’ed in the Windows 95 Unidriver. The minidriver sets MD_CMD_CALLBACK in MODELDATA and then indicates in the GPC that it wishes to be called to implement particular commands. Rather than supplying a PCL string for that command, the GPC contains “%ID”, where ID is a unique identifier to be passed to the minidriver during the callback. At print time, if RasDD determines that a printing feature is to be handled by the minidriver, it will call the minidriver’s exported command callback routine with the ID and parameter list for the feature. The callback form is similar to the Win95 OEM callback (see Appendix B of the GPC spec, version 3.00), with the exception of the first parameter, pdv as RASDDPDEV pointer.  The callback function can use pRasDDPDEV to retrieve its own OEMDrvDHPDev field.

VOID OEMCommandCallback(

PRASDDPDEV	pRasDDPDEV,

DWORD			wCmdCbID,

LPDWORD			pdwParams

);

### OEMFilterGraphics

### OEMDump

## RasDD Helper Functions

RasDD will export a set of functions to allow the minidriver to do kernel-mode allocation, make queries on RasDD’s PDEV attributes and notify RasDD of PDEV changes. The minidriver can call these functions through a set of macros which extract RasDD’s entry points from its PDEV.

### RasDDAlloc

### RasDDFree

### RasDDPDEVStateChange

The minidriver will use this API to inform RasDD of changes in PDEV state.

### RasDDItemInfo

The minidriver will use this API to extract GPC data from RasDD’s PDEV (paper size, paper type, etc.).

### RasDDWriteSpoolBuf

### RasDD OEMDDI Functions

## Execution Path

The minidriver will never call or be called directly by engine. All DDI calls go directly to RasDD. RasDD will call the minidriver if it has exported the DDI in its function table. The minidriver may call RasDD back to handle the DDI if it only wishes to pre- and/or post-process the call.

# Interoperability

| client | server | operation | result |
| --- | --- | --- | --- |
| NT4.0 | NT3.51 | print | OK |
| NT4.0 | NT3.51 | remote admin | Not possible. 4.0 settings made on client will be lost when DEVMODE is converted back to 3.51. |
| NT3.51 | NT4.0 | print | OK. Client can access only features available in 3.51 driver. |
| NT4.0, driver < server | NT4.0 | all | OK. Client will pull down new driver. |
| NT4.0, driver > server | NT4.0, driver < client | print | OK. |
| NT 4.0, driver > server | NT4.0, driver < client | remote admin | Public DEVMODE settings will stick, but driver extra settings will default to server defaults. |

# Bi-directional Support / JetAdmin

Registry keys TBD, but these are the PML objects we’d like to support:

Group 1 (required)

OT_PERIPHERAL_DETAILS

OT_PERIPHERAL_INFO

OT_PERIPHERAL_CAPABILITIES

OT_PERIPHERAL_DISK

OT_PERIPHERAL_INPUT_TRAYS

OT_PERIPHERAL_HCI

OT_PERIPHERAL_HCO

OT_PERIPHERAL_CONFIG_CHANGE

OT_PERIPHERAL_INSTALLED_PHD

OT_PERIPHERAL_OUTPUT_BINS

OT_PERIPHERAL_ENVL_FEEDER

OT_PERIPHERAL_ENABLED_MEDIA

Group 2 (if time permits)

OT_PERIPHERAL_FONT_LIST2

OT_PERIPHERAL_FONT_INFO

OT_PERIPHERAL_DOWNLOAD_FONT

OT_PERIPHERAL_DELETE_FONT

OT_PERIPHERAL_MS_CHANGE

OT_PERIPHERAL_MASS_STORAGE

OT_PERIPHERAL_RPC_BOUND

# Items Not Covered

Modifications to RasDD will specifically not include the following:

- the ability for the minidriver to supply its own property sheet page tabs

- per-user document defaults

- These may be supported by Microsoft in a future OS release.

# Issues/Questions

- Add stapling to public DEVMODE?

- Not at this time. Beta has been out too long already.

- Does the minidriver need to know about RasDD extra data?

- No.

- Will RasDD handle multiple installable UI modules for 3rd-party hardware options?

- Not at this time.

- How do we handle custom font SIMM support in the 5Si driver?

- Ganesh suggests that we port the HP font installer to NT 4.0 and make it registry-based in the process. What else needs to change to make this work correctly? Do we need a separate document on this piece? Is it work that can reasonably get done in this timeframe?

# Reference

## Minidriver UI DLL Entry Points (user-mode)

- OEMDevMode (Required)

- OEMCommonUI (Required)

- DevQueryPrintEx (Optional)

- DrvDeviceCapabilities (Optional)

- DrvUpgradePrinter (Optional)

- DocumentPropertiesCommonUICallback (minidriver internal)

- PrinterPropertiesCommonUICallback (minidriver internal)

## Minidriver  DLL Entry Points (kernel-mode)

- OEMEnableDriver (Required)

- OEMDisableDriver (Required)

- OEMEnablePDEV	 (Required)

- OEMDisablePDEV	(Required)

- OEMResetPDEV	(Required)

- OEMDevMode (Optional)

- OEMCommandCallback (Optional)

- OEMFilterGraphics	(Optional)

- OEMDump (Optional)

- Any other DDI Functions (Optional)