WINDOWS NT 5.0 USER INTERFACE MODULE

DESIGN SPECIFICATION

VERSION 0.46

7/29/26Table of Contents

1. INTRODUCTION

1.1. BACKGROUND

1.2 GENERAL OVERVIEW

2. ARCHITECTURE OVERVIEW

3. INTERFACE REQUIREMENTS

3.1  SPOOLER and APPLICATIONS

3.2 COMPSTUI

3.3 OEM CUSTOM UI

4. DATA REQUIREMENTS

4.1 Internal Binary Data Structure.

4.1.1 RESOURCES

4.1.2 INFOSTRUCT

4.1.3 INFOHEADER

4.1.4 UIINFO

4.1.5 UICONSTRAINT

4.1.6   INVALIDCOMBO

4.1.7  UI GROUP

4.1.8 FEATURE

4.1.9   OPTION

4.1.11   INVOCATION

4.1.10  FEATURE IDENTIFICATION

4.2 PRINTERDATA - PRINTER STICKY INFORMATION

4.3 DEVMODE - DOCUMENT STICKY AND DRIVER BUILTIN INFO

4.3.1 PUBLIC DEVMODE

4.3.2 PRIVATE DRIVEREXTRA DEVMODE

4.3.3   DISTINGUISHING UNIDRV DRIVEREXTRA FROM POSTSCRIPT DRIVEREXTRA

4.4 OTHER DRIVER INFORMATION STORED IN REGISTRY

4.4.1 FONT SUBSTITUTION TABLE

4.4.2 FORM TO TRAY TABLE

4.4.3  DEVHTINFO

4.4.4  MODELNAME

4.4.5   COUNTRY

4.4.6  FREE MEMORY

4.4.7  FONT CARTRIDGE

4.4.8  UNIDRV FLAGS

4.4.9   PRINTER HALFTONE

4.4.10  FORM?

4.5 INTEROPERABILITY

4.51 DEVMODE CheckSum == Binary Data File CheckSum

4.52 DEVMODE CheckSum != Binary Data File CheckSum

4.5.3 CLIENT/SERVER SCENARIOS

5.  BI-DI SUPPORT

6. CUSTOM UI SUPPORT

7. PROPERTY SHEET UI SUPPORT

8. EXTENDED FEATURES FOR NT 5.0

9. IMPLEMENTATION DETAILS

9.1  Caching the Binary Data File

9.2  Devmode Handling

9.3 DeviceCapabilities

9.4  HANDLING OF PRINTERDATA/INFORMATION IN THE REGISTRY

9.4.1  PRINTER STICKY FEATURES

9.4.2  OTHER INFORMATION IN REGISTRY

10.  OPEN ISSUES

# 1.	INTRODUCTION

This document describes the design specification and implementation details of the UI module used for all system printer drivers in Windows NT 5.0.  It includes description of interfaces and data structures shared between the UI module and the text data parsers (GPD/PPD parser), the common UI module (compstui.dll), OEM plug in custom UI, applications and the spooler.   In addition, it will also outline some extended features planned for standard UI.  This is intended to be a working document and will get updated as the project progresses.

## 1.1.	BACKGROUND

Currently, we have three UI DLLs that handle the printer/plotter user interface for NT, one for Postscript one for Unidrv devices (PCL, inkjets, TTY etc) and one for plotter devices.  These modules have a lot of common features and functionality.  Both have the same exports for the spooler/applications as well as their common use of the compstui.dll to display the UI.  Internally,  their main difference is the data structure that is used to drive the UI.  One is PPD driven and the other is GPC driven.

The objective of this effort is to have one UI module for Postcript,Unidrv and plotter devices.  Some of the advantages of having a single UI DLL are as followed: No duplication of code for OEM custom UI, future DDI/spooler changes would require changes in one module, reduction in testing effort, reduction in code footprint, and extendatable support for all devices.

In addition, a common UI module would simplify the installation of multi personality devices.  An example of a confusing installation scenario is the HP laserJet 4MP, which supports PCL5 and Postcript.  At printer installation time, the user would see two device names for the printer.  This was confusing and received a lot of complaints.  With one UI module for all system printer drivers, the choice of Postcript/PCL would be in the user interface.  Once the user change the default, the UI module would call the spooler to update printer driver file.

[ There are two possiblities for handling multi-personality devices:  Dynamically switching the UI and driver based on the user selection (Postscript/Unidrv) in Printer Properties or the printer installation automatically creates multiple printer queues, one for each language the printer can support]

## 1.2 GENERAL OVERVIEW

The user interface module handles UI requests from the spooler/applications for Unidrv, Postscript and plotter drivers.  Its features included but not limited to handling of all device capabilities querying, displaying the graphical user interface, providing support functions for OEM plug-in UI, allowing users to set printer/document properties as well as providing the setting information to the rendering modules.

# 2.	ARCHITECTURE OVERVIEW

The following figure illustrates the interaction of the UI module with other components in the system.

DEVMODE/PRINTERDATA - contains document/printer sticky information for the print job.

INTERNAL BINARY DATA - structure containing all information necessary for displaying the printer features ,rendering , and sending the print job to the printer.

PPD/GPD PARSER - PPD and GPD parser that parse PPD/GPD into INTERNAL BINARY DATA.

WINSPOOL - Spooler client that loads the UI module and call compstui to collect propertysheet pages and get option items to display.

COMPSTUI - This module calls the UI module to collect propertysheet to display.  It collects the option items to display and handles all the displaying. It calls back to the UI module to update DEVMODE, PRINTERDATA per user’s setting.

OEM CUSTOM UI - OEM plug-in UI.  This module is developed by the OEM and is supported by the UI module to allow OEMs to extend their user interface to support new printer features.

RENDERING DRIVERS - Poscript , Unidrv and plotter rendering drivers.  They handles all the rasterizing and downloading.

# 3.	INTERFACE REQUIREMENTS

This section lists the data requirements and interface requirements for the UI module.

## 3.1 	SPOOLER and APPLICATIONS

The UI Module must export the following functions (as a minimum).  Function prototypes and details are available in the Win32 SDK.

PrinterProperties - This function displays a printer properties dialog box for the specified printer.

DllInitialize - This function is called when the system loads/unloads the module.  It creates/deletes critical section objects ( private UI data).

DrvDocumentProperies - This function deals with DEVMODE settings(copy, update etc.) as well as displaying the document properties dialog.

DevQueryPrintEx - This function checks whether the job can be printed with the corresponding DEVMODE passed in.

DrvDeviceCapabilities - This function support the querrying of device capabilities.

DrvUpgradePrinter - This function is called by the spooler everytime a new driver is copied to the system.  It handles the upgrading of information in the registry to new driver format.

DrvDevicePropertySheets - This function is called to display the Device Property Sheet.

DrvDocumentPropertiesSheets - This function is called to display the Document Property Sheet.

DrvConvertDevMode - This function is coverts the devmode between different versions of the operating system.

DrvAdvancedDocumentProperties - This function is the old API kept for compatibility.  It’s the same as DrvDocumentProperties.

DrvDocumentEvents - This function enabled GDI to call the UI module for special processing during the process of printing (CreateDC, StartDoc, StartPage, EndPage, EndDoc, Escape etc.)

## 3.2	COMPSTUI

There are two possible interaction scenarios between the UI module and COMPSTUI.DLL -  Either the UI is initiated from Print Folder or the UI is initiated from the applications.

Print Folder:

The spooler loads the UI module and call COMPSTUI’s CommonPropertySheetUI API to collect either device or document property sheet. Then COMPSTUI calls the UI module DocumentPropertySheets or DevicePropertySheets API to collect the option items to display in the respective property sheet.  The UI module builds up all the appropriate OPTITEMs and call COMPSTUI back for display. COMPSTUI.DLL notifies the UI module when any changes is made by the user through callbacks for DocumentProperties and DeviceProperties

Applications:

Whenever the applications call UI module’s PrinterProperties or DrvDocumentProperties API to display the dialogs, the UI module loads COMPSTUI and calls its CommonPropertySheetUI function (providing it with the appropriate callbacks - DrvDocumentPropertySheets or DrvDevicePropertySheets).   Then Comptui.dll calls the UI module DocumentPropertySheets or DevicePropertySheets to collect the option items to display in the respective property sheet.  The UI module builds up all the appropriate OPTITEMs and call COMPSTUI back for display. COMPSTUI.DLL notifies the UI module when any changes is made by the user through callbacks for DocumentProperties and DeviceProperties

See compstui.h or DDK for more details on Compstui.

## 3.3	OEM CUSTOM UI

The UI module interacts with the OEM custom UI module through four entry points. For more details see NTGDI\PRINTERS\SPECS\CUSTOM\OEMCUST.DOC.

OEMDevmode - UI module calls this function to get the OEM private devmode size, private default devmode, and to  convert/validate the OEM private devmode.

OEMCommonUI - The UI module calls this function when it completed building its own OPTITEMs list to allow OEM UI to add new OPTITEMs to the Document Property Sheet or Device Property Sheet.

OEMPrinterPropertiesCallback - Callback for any changes to the OEM printer sticky items.

OEMDocumentPropertiesCallback - Callback for any changes to OEM document sticky items.

# 4.	DATA REQUIREMENTS

The following is a clarification of the definitions used in this section:

Built-In Capabilities - These are features that are either built-in driver features available for all printers(example: host halftoning) OR features that could be simulated by the driver if the printer device does not support it (example: the driver will simulate rotation if the printer hardware does not have support for it).

Generic Features - These are features that are particular to the given hardware device and would not require any knowledge, interaction or handling from the driver. (example: stapling)

Predefined Features - These are features that the driver and PPD/GPD parser are aware of through predefined GIDs.

## 4.1	Internal Binary Data Structure.

The internal binary structure is required to have all the information necessary to display the user interface, handle device querrying and render the print job.  The binary data file is divided into two sections, the ui section and the driver section.  This document covers only the ui section (i.e. items  related to device querying and displaying of dialogs).  The following figure illustrates how the internal binary data structure is generated.

### 4.1.1	RESOURCES

Resources (icons, strings, bitmaps etc.) use by the UI module can be stored in the binary data heap or in a resource DLL.

The following sections refer to the term RESOURCE REFERENCE used for icons, strings, and bitmaps.  Therefore, a clarification of the term is warranted.  A resource reference is always a DWORD, and its most significant bit is used to distinguish an offset to the internal binary heap from a resource ID.  If the MSB is set, it means that the resource is in a resource DLL, the UI module would need to load the resource DLL to get the resource, and the remaining bits represent a resource ID.  If the MSB is not set, then the resource reference is an offset into the internal binary heap.  Note that all offset are relative to the beginning of the binary data file.

### 4.1.2	INFOSTRUCT

PPD and GPD parsers are required to support the following structure.  UIINFO is the struct encapsulating all the information the UI module needs to handle device querying and displaying of graphical interface.  DRIVERINFO is the rendering information struct.

The GPD/PPD organization of binary file is really up to the GPD/PPD parser designer.  The only requirement for UI module is that the parser organizes the INFOHEADER, UIINFO and heap as specified below.

BINARY FILE

INFOHEADER	infoheader;		// header for binary file, contains offsets

UIINFO		uiinfo;			// UI information

DRIVERINFO	driverinfo;		// render information

BYTE[ ]		heap;			// heap for binary data

….

….

### 4.1.3	INFOHEADER

This is the header for the binary file supported by GPD/PPD parser.  The header contains offsets to UIINFO and DRIVERINFO as well as version information.  All heap related data are kept in contiguous memory.  Note: All offsets are relative to the beginning of the binary data.

typedef struct _INFOHEADER {

DWORD	dwSize;			// sizeof(INFOHEADER)

DWORD	dwFileSize;		// sizeof BINARY DATA FILE

DWORD	dwVersion;		// parser’s version

DWORD	dwCheckSum;		// 32 bit CRC checksum of ASCII

// text file GPD/PPD.

DWORD	dwSignature;		// DRIVER_SIGNATURE_POSTSCRIPT

// DRIVER_SIGNATURE_UNIDRV

DWORD	dwUIInfoOffset;		// byte offset to UIINFO struct

DWORD	dwDriverOffset;		// byte offset to DRIVERINFO struct

} INFOHEADER, *PINFOHEADER;

### 4.1.4	UIINFO

The following structure is used to describe the graphical user interface as well as device capabilities.  For definition of RESOURCE REFERENCE, refer to section 4.1.1

typedef struct _ARRAYREF{

DWORD	dwCount;		\\ count of items in array

DWORD	dwOffset;		\\ byte offset to the array.  If the dwOffset

\\ is NULL, then the array is not valid.

} ARRAYREF, *PARRAYREF;

typedef struct  _UIINFO{

DWORD	dwSize;			// sizeof (UIINFO)

// version, compatibility

DWORD	dwResourceName;	// offset to the name of the Resource DLL.

// dwResourceName == 0 means that

// there is no resource DLL.

DWORD	dwFilename;		// RESOURCE REFERENCE to PPD/GPD

// filename (Unicode string)

DWORD	dwNickname; 		// RESOURCE REFERENCE to device

// nickname (Unicode string)

DWORD	dwUIINFOVersion;	// uiinfo version number

DWORD	dwSpecVersion;	// PPD/GPD spec version

DWORD	dwTechnology		// technology ID for UNIDRV devices

// TECH_PCL4, TECH_CAPSL,

// TECH_PPDS, TECH_TTY, TECH_DBCS

// There is no Technology bit in GPD, the

// GPD parser would have to perfom the checking

// to initialize this field.

// list of  features

// Due to the definition of OPTION struct and predefinedIndex[ ] , the number of total feature // (generic & predefined) are limited to 256, with each feature containing up to 64K options.

// cDocStickyFeatures is the total number of

// document sticky features (generic + predefined)

// cPrinterStickyFeatures is the total number of

// printer stikcy features (generic + predefined)

DWORD	cDocStickyFeatures;

DWORD	cPrinterStickyFeatures;

DWORD	dwFeatureListOffset;	// byte offsets to array of FEATURES

// This array contains printer and

// document sticky  features.

// default font substitution table

// table layout: TT font name followed by corresponding device font name

// font names are null terminated unicode string

ARRAYREF	FontSubstitutionTable;	// count and offset to font sub table

// ui constraints This is present but the UI would not need to use it if helper functions

// are available.

// There are a maximum of 65536 UI contraints available.

ARRAYREF	UIConstrainsts;		// count and offset to UICONSTRAINTS array

// the parser will give the UI module a list of items that require notification to the parser.

// Once any of the items changed, the UI module calls the parser to update the

// binary data struct. This might be removed if helper functions are available.

ARRAYREF	UpdateRequiredItems;	// count and offset to update required array

// invalid combination.  This might be removed if helper functions are available.

ARRAYREF	InvalidCombinations;	// count and offset to INVALIDCOMBO array

// UI group.  In the case where UIGroups is not present, the UI module always refer to

// the FEATURE array first.  Otherwise, it uses the UIGroups to sort the features first then

// go back to the FEATURE list to handle the left over ones.

ARRAYREF	UIGroups;		// count and offset to UIGROUP array

// miscellaneous information about the Postscript devices.

// This could be encapsulate to a PS specific structure.

LONG		lLanguageEncoding	// language encoding

DWORD	dwLangLevel;		// Postscript language level

INVOCATION	password;		// for downloading header permanently

INVOCATION	exitServer;		// cmds for job loop

DWORD	dwProtocols		// protocol supported by the device

DWORD	dwFreeMem;		// default amount of free VM

DWORD	dwJobTimeOut;		// default job timeout value

DWORD	dwWaitTimeOut;	// default wait timeout value

LONG		lTTRasterizer;		// TT options. What rasterizer

// is on the system.

// Flags for UI Module. Applicable for Postscript and Unidrv

// These flags are used to notify the UI module whether these features are supported

// for the device.

// FLAG_RULESABLE - device support RECTFILL.  For enabling rules support.

// FLAG_DOWNLOADXXX can be removed here and make into a

// Predefined feature.

// FLAG_PRINTPSERRORS - whether to print PS errors

DWORD	dwFlags;		// flags for UI module internal handling

// The following is a resource reference to the printer icon.  Each printer may have

// a unique icon.

DWORD	dwPrinterIcon;		// RESOURCE REFERENCE to printer icon.

// RECTL rcMaxExtend and RECTL rcMinExtend are the maximum and minimum

// dimensions for the printer.

RECTL		rcMaxExtend;

RECTL		rcMinExtend;

// font cartridge information.  This is not applicable for Poscript5.

DWORD	cCartridgeSlotCount;	// number of cartridge slots, MAX_NUM_CART=4

// softfont installer

DWORD	dwFontInstallerName;	//RESOURCE REFERENCE for font installer.

// the predefinedIndex array is used to index into the Feature list.

// it’s present for optimization since it’s inefficient to grovel through the Feature list

// for every predefined item.

// the parser is responsible for initializing each field in the predefinedIndex[ ]  with the

// coressponding index into the feature list.

// MAX_GID is 64

BYTE		predefindedIndex[MAX_GID];

} UIINFO, *PUIINFO;

The following figure illustrates the FEATURE array with respect to the UIINFO struct.

### 4.1.5	UICONSTRAINT

This structure is used to specify which features cannot be supported together.  This allows the UI to prevent the selection from the user interface.

typedef struct  _UICONSTRAINT{

WORD		wReserved;		// DWORD alignment.

WORD		wNextConstraintID;	// index of the next UICONSTRAINT in the list

// if wNextConstraintID is 0xFFFF, then

// there is no next constraint.

WORD		wFeatureIndex;		// index of printer feature being constrained

WORD		wOptionIndex;		// index of the feature option being constrained

// if wOptionIndex is 0xFFFF, then this

// contrainst applies to all options of the feature.

} UICONSTRAINT, *PUICONSTRAINT;

### 4.1.6   INVALIDCOMBO

This structure is used to define the invalid combination of group of FEATUREs or OPTIONs.  Invalid Combination means that a group of features or options cannot be selected together, although subsets are valid.

typedef struct _INVALIDCOMBO{

WORD		wFeature;		// specifies the feature index

WORD		wOption;		// specifies the option index

WORD		wNextElement;		// specifies the index to another INVALIDCOMBO

// 0xFFFF indicates end of the list.

WORD		wNewCombination;	// specifies the index of the first element of

// another INVALIDCOMBO list.

// 0xFFFF indicates no more INVALIDCOMBO list

} INVALIDCOMBO, *PINVALIDCOMBO;

### 4.1.7  UI GROUP

This structure is used to allow the UI module to group related FEATUREs together.  In the case where UIGROUP is defined in UIINFO, the UI module will display the features according to the specification in the UIGROUP first then it display the rest of the items in the FEATURE list.  If there are no grouping construct, UIINFO.UIGroup.dwOffset is NULL.

typedef struct _UIGROUP{

STRINGREF	GroupName;		// string reference to the group name

WORD		wNextGroup;		// specifies the index of the next group in

// the same level to display

WORD		FirstSubGroup;		// specifies the index of the first sub group

DWORD	wFlags;			// UIGROUP_NOSUBGROUP.  If this flag is set

// then there is no subgroup.  The FirstSubGroup is

// an index to a FEATURE list for the group.

} UIGROUP, *PUIGROUP;

### 4.1.8	FEATURE

This structure specifies all the information necessary to represent a feature in the graphical user interface.  The PPD/GPD parser is required to fill out all the information relevant to a particular feature.  For defintion of RESOURCE REFERNCE, refers to section 4.1.1

typedef struct _FEATURE{

DWORD	dwReserved;		// reserved

DWORD	dwKeyWord;		// RESOURCE REFERENCE to feature

// keyword.

DWORD	dwDisplayName;	// RESOURCE REFERENCE to feature

// display name Unicode string

DWORD	dwFlags;		// printer feature flag.  FF_INSTALLABLE is used

// for installable option.  This flag is cleared for

// non-installable options

// The following three fields are present for reference purposes.  The UI module

// will not need to use these fields if there are helper functions to support constraint and

// building option array.

DWORD	dwDefaultOptIndex;	// index of default option for feature

DWORD	dwNoneFalseOptIndex;	// index of NONE or FALSE option

ARRAYREF	UIConstraintList;	// count and offset to list of options constrained

// by the feature.  If the offset is NULL, then there

// is no contraint list.

DWORD	dwFeatureID;		// specifies the feature ID

// GID_UNKNOWN is used for generic feature.

// The rest are predefined GIDs.

// GID_RESOLUTION

// GID_PAGESIZE

// GID_PAGEREGION

// GID_CUSTOMSIZE

// GID_DUPLEX

// GID_INPUTSLOT

// GID_MANUALFEED

// GID_MEDIATYPE

// GID_MEMOPTION

// GID_DUPLEX

// GID_COLORMODE

// GID_COPIES

// GID_ROTATION

// GID_PAGEPROTECTION

// GID_COLORMODE

// GID_FONTSUB

// GID_FONTCART

// GID_PRINTERHT

DWORD	dwUIType;		// specifies type of feature option list.

// UITYPE_PICKONE

// UITYPE_PICKMANY

// UITYPE_BOOLEAN

DWORD	dwFirstOrderIndex;	// specifies the first option to be emitted if the

// feature type is UITYPE_PICKMANY

// BIDI support. Postscript.

// The invocation string is defined in the PPD file for Postscript.

// For more information, refer to the bi-di section.

INVOCATION	queryInvoc;		// invocation string used for local bi-di.

// BIDI support. Unidrv

// There are two set of {bidi ID, data format} per feature.  The EnumXXX set is

// used to enumerate all installed options.  The CurrentXXX set is used to find

// the current valid option for the feature.  For more details , refer to the bi-di section.

WORD		wEnumID;

WORD		wEnumFormat;

WORD		wCurrentID;

WORD		wCurrentFormat;

DWORD	dwOptionSize;		// size of option data structure of the feature

ARRAYREF	Options;		// count and offset to the array of OPTIONs

// available for this feature.

// Note: dwResourceIcon, dwHelpString, dwPromptMessage are optional.  These

// items might need to be updated based on the user’s selection,i.e they have dependencies

DWORD	dwResourceIcon;	// RESOURCE REFERENCE to the icon

// corresponded with the feature

DWORD	dwhelpstring;		// RESOURCE REFERENCE to the help string

// of the feature.

DWORD	dwPromptMessage;	//  RESOURCE REFERENCE to the message to

// be prompted at selection. Like Feed paper.

// The following two fields are not valid for non-installable FEATURE.  They will be set

// 0xFFFF for non-installable feature.

// For installable feature, we need to have a mechanism to link installable features to the

// installable section (where a feature is marked just installed or not installed).  For example,

// RET might have a couple of options (smooth, pattern ect.).  However, selection of smooth

// option is tied to whether RET is installed or not.  In this case, dwInstallableFeatureIndex

// is the index to the installable list, indicating whether RET is installed or not.

// dwInstallableOptionIndex is used to link an option dependent on an installable feature.

// For example, evelop feeder selection of input slot feature is dependent on whether

// envelop feeder is installed or not.

WORD		wInstallableFeatureIndex;

WORD		wInstallableOptionIndex;

DWORD	dwPriority;		// specifies the priority of the feature.  If the GPD

// writer doesn’t specify the priority, the parser will

// assign the priority.

} FEATURE, *PFEATURE;

### 4.1.9   OPTION

This structure specifies all the information necessary to represent an option of a feature.

typedef struct _OPTION{

DWORD	dwReserved;		// reserved

DWORD	dwKeyWord;		// RESOURCE REFERENCE to option keyword

// ASCII string

DWORD	dwDisplayName;	// RESOURCE REFERENCE to  option

// display name Unicode string

// UI Module will not need to look at invocation.

INVOCATION	invocation;		// invocation string for option

DWORD	dwOrderIndexNext;	// specifies the next option to be emitted if

// feature.uiType is PICKMANY.

DWORD	dwUIConstraintList;	// list of options constrained by the option.

// 0xFFFFFFFF indicates no constraint.

// The following two fields are kept for reference.  The UI module will not need to

// access them if the parser provides the helper functions to resolve invalid

// combination and initialization.

WORD		wInvalidComboIndex;	// specifies the index into UIINFO

// invalid combination list.

WORD		wInstallableOptionIndex;// set to 0xFFFF for non-installable feature.

// otherwise, it’s an index to the FEATURE

// list for dependency on a installable feature.

// Note: dwResourceIcon, dwHelpString, dwPromptMessage are optional.  These

// items might need to be updated based on the user’s selection,i.e they have dependencies

DWORD	dwResourceIcon;	// RESOURCE REFERENCE to icon

// for the option

DWORD	dwHelpString;		// RESOURCE REFERENCE to help

// string for the option.

DWORD	dwPromptMessage;	// RESOURCE REFERENCE to a message

// to be prompted at selection.

// Like Feed Envelop.

} OPTION, *POPTION;

### 4.1.11	  INVOCATION

This structure describes the format of a command to the printer device.

typedef struct _INVOCATION{

DWORD 	dwLength;		// size of invocation string in bytes

DWORD	dwDataOffset;		// byte offset to the invocation string data

} INVOCATION, *PINVOCATION;

### 4.1.10  FEATURE IDENTIFICATION

There are two types of feature ID.  The first one is for the features that the UI module need not have any knowledge of, they are generic printer feature IDs and are represented by GID_UNKNOWN.

The second type is used to represent features that the driver understands and possibly have to handle due to extra requirements in the system.  These features are considered predefined because they don’t fit into the generic option structure.  The following is a list the predefined feature identifiers followed by their corresponding option structures.  Note:  Render specific information are not defined in these predefined features.  They will either be in the Global or State variables tables in the Control module.

In addition, each option structure defined below has a pointer that points to a render driver defined structure that encapsulate the rendering information needed per option.  Postscript and Unidrv drivers may have different “EXTRA” structure.

GID_RESOLUTION

typedef struct _RESOLUTION{

OPTION	genericOption;

short		xdpi;

short		ydpi;

PSFIX		screenAngle;

PSFIX		screenFreq;

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

} RESOLUTION, *PRESOLUTION;

GID_CUSTOMSIZE

typedef struct _CUSTOMSIZE{

OPTION	genericOption;

DWORD	wFlags;			// CUSTOMSIZE_CUTSHEET

LONG 		lMaxMediaWidth;

LONG		lMaxMediaHeight;

LONG		lMinMediaWidth;

LONG		lMinMediaHeight;

RECTL		rectHWMargins;

struct {

LONG	lorder;

LONG	lminVal;

LONG	lmaxVal;

} params[CUSTOMPARAM_MAX];

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

} CUSTOMSIZE, *PCUSTOMSIZE;

GID_DUPLEX

typedef struct _DUPLEX{

OPTION	genericOption;

DWORD	dwDuplexID;		// ID of pre-defined Duplex option.

// DMDUP_HORIZONTAL

// DMDUP_VERTICAL

// DMDUP_NONE (SIMPLEX)

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

} DUPLEX, *PDUPLEX;

GID_COLLATE

typedef struct _ COLLATE {

OPTION	genericOption;

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

} COLLATE; *PCOLLATE;

GID_PAGEPROTECTION

typedef struct _PAGEPROTECTION{

OPTION	genericOption;

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

} PAGEPROTECTION;*PPAGEPROTECTION;

GID_COPIES

typedef struct _COPIES{

OPTION	genericOption;

DWORD	dwMaxCount;

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

} COPIES, *PCOPIES;

If GID_COLORMODE is not present, it implies the device is monochrome.

GID_COLORMODE

typedef struct _COLORMODE{

OPTION	genericOption;

// wPlanes # of color planes supported.

// wBitPixel, # of bits per pixel per plane.

WORD		wPlanes;

WORD		wBitsPixel;

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

} COLORMODE, *PCOLORMODE;

GID_ORIENTATION

typedef struct _ORIENTATION{

OPTION	genericOption;

// wLanscapeAble is TRUE or FALSE.  It’s set to TRUE if the printer can do rotation.  If

// set to FALSE, then it cannot.

// wRotationAngle can be one of the following

// ROTATE_90

// ROTATE_270

// ROTATE_NONE.

// If wLanscapeAble is FALSE, wRotationAngle is ignored.

WORD		wLandscapeAble;

WORD		wRotationAngle;

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

} ORIENTATION, *PORIENATION;

GID_PAGESIZE

typedef struct _PAGESIZE{

OPTION	genericOption;

SIZEL		szPaperSize;

// Two possible printable area, one for each orientation.

RECTL		rectImageableAreaP;

RECTL		rectImageableAreaL;

// GPD file will contain a predefined keyword for each paper size.  This will be used by the // UI to map paper size with the one in the form database.  sPaperSizeID < 256, Windows

// predefined ID. sPaperSizeID >= 257, it’s driver defined.

DWORD	dwPapersizeID;			// ID of predefined paper size

// PAGESIZE_ROTATE - X & Y dimensions are exchanged by UI module for DevCaps.

// PAGESIZE_MAXPRINTABLEAREA - Max Printable area in microns

// PAGESIZE_MINLEFTMARGIN - Min left margin in microns.

DWORD	dwPSFlags;

// dwPageProtectionMemory define the amount of protection memory used for this page.

// If dwPageProtectionMemory value is 0xFFFFFFFF, then page protection is not ON for

// this paper size.

DWORD	dwPageProtectionMemory;

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

} PAGESIZE; *PPAGESIZE;

GID_INPUTSLOT

typedef struct _INPUTSLOT{

OPTION	genericOption;

DWORD	dwFlags;			// INPUTSLOT_REQPAGERGN

// GPD has predefined ID for paper source. InputSlotID < 256, it’s Windows predefined ID.

DWORD	dwIPaperSourceID;		// ID of predefined input slot

// UI need this for DeviceCapabilities. sbinAdjust is set to one of the DCBA_XXX option

DWORD	dwBinAdjust;			// DCBA_XXX option

// Maximum and Minimum Extend for the input slot.

RECTL		rcMaxExtend;

RECTL		rcMinExtend;

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

} INPUTSLOT, *PINPUTSLOT;

GID_OUTPUTBIN

typedef struct _OUTPUTBIN{

OPTION	genericOption;

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

} OUTPUTBIN;*POUTPUTBIN;

GID_MEDIATYPE

typedef struct _MEDIATYPE{

OPTION	genericOption;

// GPD has predefined ID for paper quality. mediatypeID < 256, it’s Windows predefined

// ID.

DWORD	dwMediatypeID;			// ID of predefined paper quality

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

}MEDIATYPE, *PMEDIATYPE;

GID_MEMOPTION

typedef struct _MEMOPTION{

OPTION	genericOption;

DWORD	dwInstalledMem;

DWORD	dwFreeMem;

// The following pointer points to a structure defined by the rendering module to hold

// render specific information.  If dwRenderInfoOffset is 0xFFFFFFFF, there is no

// corresponding render info.

DWORD	dwRenderInfoOffset;

// UI module would have to create a name for memory option using the

// value in dwInstalledMem.

} MEMOPTION, *PMEMOPTION;

The following 3 IDs are defined in order for the UI module to track and saved equivalent data currently stored in the registry.

GID_FONTSUB

GID_FONTCART

GID_PRINTERHT

## 4.2	PRINTERDATA - PRINTER STICKY INFORMATION

PRINTERDATA struct encapsulates all printer sticky information.  PRINTERDATA struct defined below is a superset of the printer sticky data items for Postcript and Unidrv devices.  The code path for handling printer sticky information will be different for Unidrv and Postscript.  Not all of the items in PRINTERDATA applied to Unidrv.  The common fields are identified are italicized below and will have common code path for all devices.

PRINTERDATA is stored in the registry.  It’s read from the registry when the UI need to initialize the DevicePropertySheet dialog and is written to the registry when the user made changes to the printer sticky features.

REGISTRY KEY = REGKEY_PRINTERDATA

typedef struct _PRINTERDATA{

WORD 		wVersion;			// PRINTERDATA version

WORD		wSize;				// size of PRINTERDATA

DWORD	dwFlags;			// flags:

// PD_METRIC - specifies country/unit

// PD_HOST_HALFTONE - use host HT

// PD_IGNORE_DEVFONT- ignore

// devfont

// PD_FONTSUB - use font substitution

// PD_JOBSEP - use job separation

// PD_PAGEPROT - use page protection

DWORD	dwFreeVM;			// available VM on printer

DWORD	dwJobtimeout;			// job timeout

DWORD	dwWaitimeout;			// wait timeout

DWORD	Reserved[4];			// reserved

WORD		wCheckSum			// old checksum

PROTOCOLS	iProtocol;			// Poscript output protocol

// PROTOCOL_ASCII

// PROTOCOL_BCP

// PROTOCOL_TBCP

// PROTOCOL_BINARY

DWORD	dwCheckSum32;		// 32 bits check sum

// array of paper margins

RECT		rectPaperMargins[MAXPAPERMARGINS];

DWORD	dwOptionCount;			// number of printer sticky features

// OPTSELECT is a structure containing the index of the currently selected options

// for the corresponding features and the index of the next option selected for PICKMANY

// MAX_PRN_OPTIONS is 256.

OPTSELECT	options[MAX_PRN_OPTIONS];

} PRINTERDATA; * PPRINTERDATA;

[ As a side note, we should consider going with an ID based keys to represent printer sticky items.  This could be modeled after the open devmode architecture ]

typedef struct _OPTSELECT{

WORD		wCurrentSelectOption;		// index of selection for the feature

WORD		wNextSelection;			// index of next selection for PICKMANY

// 0xFF for no next option selected

} OPTSELECT, *POPTSELECT;

The following figures illustrates the mapping of printer features in binary data structure with option selected in PRINTERDATA.  It’s a one-to-one mapping between printer sticky features and option selection.

## 4.3	DEVMODE - DOCUMENT STICKY AND DRIVER BUILTIN INFO

The UI module is required to handle DEVMODE for both Postcript and Unidrv devices.  The public devmode is defined and is common for all devices.  Since the private driver extra devmode is different for Postcript and Unidrv, the UI module will handle the devmode differently depending on the device.  Document sticky items and driver built-in items are stored in the private driver extra devmode.  Each private driver extra devmode is required to include a DRIVER_SIGNATURE which should be initialize to either :

DRIVER_SIGNATURE_POSTCRIPT

DRIVER_SIGNATURE_UNIDRV.

### 4.3.1	PUBLIC DEVMODE

Public DEVMODE is defined in Win32 SDK.

### 4.3.2	PRIVATE DRIVEREXTRA DEVMODE

The UI module will have different code path for private devmode handling.  In all cases, it will use the EXTDEVMODE defined below.

POSTCRIPT DRIVEREXTRA

typedef struct _DRIVEREXTRA{

DWORD		dwSignature;			// DRIVER_SIGNATURE

DWORD		dwFlags;			// general flags

// PSDM_EPS

// PSDM_EHANDLER

// PSDM_MIRROR

// PSDM_BLACK

// PSDM_NEG

// PSDM_FONTSUBST

// PSDM_COMPRESSBMP

// PSDM_ENUMPRINTERFONT

// PSDM_LSROTATED

// PSDM_NO_LEVEL2

// PSDM_CRTLD_BEFORE

// PSDM_CTRLD_AFTER

// PSDM_METAFILE_SPOOL

// PSDM_USE_PRN_DEF_HAF

// PSDM_USE_ICM

// PSDM_PRINT_BORDER

// PSDM_NO_MARGINS

// PSDM_USE_PRINTER_CALI

// PSDM_INTENT_DONTCARE

WCHAR		wstrEPSFile[MAX_EPS_FILE];	// EPS file name buffer

COLORADJUSTMENT	ColorAdjust;			// system halftone info

WORD			wReserved1;			// old checksum, not used

// wOptionCount is the number of doc-sticky and pre-defined items

// MAX_DOC_OPTION is 256

// bOptions [ ] is an array of doc-sticky and pre-defined items.  The pre-defined

// items are followed by doc-sticky items.

// see definition for OPTSELECT above.

WORD			wOptionCount;

OPTSELECT		options[MAX_DOC_OPTION];

WORD			dmSize;			// size of private devmode

WORD			dmOEMDriverExtra;		// size of OEM private

// devmode

LAYOUT		iLayOut;			// Nx page option layout

PSFIX			psfixScreenFrequency;		// halftone screen freq.

PSFIX			psfixScreenAngle;		// halftone screen angle

DIALECT		iDialect;				// PS output format

FONTSUB		iTTSubFmt;			// font substitution

TT_DLFMT		iTT_DLFontFmt;		// TT font download fmt

PS_DLFMT		iPS_DLFontFm;			// PS font format

WORD			sPSLevel;			// PS Level (1, 2, 3)

WORD			wMinOutlinePPEM;		// threshold of outline 1

DWORD		dwCheckSum32;		// 32 bit checksum

} PSDRIVEREXTRA, *PPSDRIVEREXTRA;

UNIDRV DRIVEREXTRA

typedef struct _DRIVEREXTRA{

short			sVer;				// DRIVEREXTRA version

// dmSize was dmDefaultSource.

WORD			dmSize;			// size of DRIVEREXTRA

// dmOEMDriverExtra was dmTextQuality

WORD			dmOEMDriverExtra;		// total size of OEM

//DRIVEREXTRA

short			sMiniVer;			// GPC version

short			dmBrush;			// type of dithering brush (0)

short			sCTT;				// CTT index

short			reserved[36];			// reserved

// reserved[36] cam from eliminating the following fields

//short			dmNumCarts;			// Not used, in registry

//short			rgFontCarts[MAXCART]	;	// Not used, in registry

//short			dmMemory;			// Not used, in PRINTERDATA

// Basically, rgindex [ ] is replaced by options [ ] array.

//short			rgindex[MAXHE];		// Not used, in PDEVICE

short			sFlags;				// general flags

// DXF_NORULES (ON/OFF)

// DXF_TEXTASGRAPHICS

// DXF_JOBSEP

// DXF_PAGEPROT

// DXF_VECTOR

// DXF_DOWNLOADTTBITMAP

// DXF_DOWNLOADOUTLINE

short			Padding;			// alignment

COLORADJUSTMENT	ca;				// system halftone information

DWORD		dwDriverSignature;		// DRIVER_SIGNATURE

DWORD		dwCheckSum32;		// 32 bit checksum

// wOptionCount is the number of doc-sticky and predefined features.

// options [ ] is an array of doc-sticky and predefined options

// MAX_DOC_OPTION is 256

WORD			wOptionCount;			// number of doc-sticky options	OPTSELECT		option [MAX_DOC_OPTION];	// array of doc-sticky and

// predefined options

} UNIDRIVEXTRA, *PUNIDRIVEXTRA;

typedef union _DRIVEREXTRA{

PSDRIVEREXTRA	psDriverExtra;

UNIDRIVEXTRA	uniDriverExtra;

} DRIVEREXTRA, *PDRIVEREXTRA;

typedef struct _EXTDEVMODE{

DEVMODE		dm;			// public devmode

DRIVEREXTRA		dx;			// driver devmode

} EXTDEVMODE, *PEXTDEVMODE;

The following figure illustrates the mapping between option list and binary data struct.

### 4.3.3   DISTINGUISHING UNIDRV DRIVEREXTRA FROM POSTSCRIPT DRIVEREXTRA

Given a EXTDEVMODE, we need a mechanism to distinguish the Unidrv devmode from Postscript devmode.  A field in the public devmode can be used - dmDriverVersion.

Currently dmDriverVersion is used to mark the version of the driver.  Postscript and Unidrv have use different base for this version.  Postcript driver version starts from 0x350.  Unidrv driver version starts from 0x301.

The UI module will distinguish one devmode from another based on the following logic.

#define DRIVER_SIGNATURE_POSTSCRIPT 	0x350

#define DRIVER_SIGNATURE_UNIDRV	0x301

if ( pEDM->dmDriverVersion < DRIVER_SIGNATURE_POSTSCRIPT)

…Unidrv devmode

else

Postscript devmode.

## 4.4	OTHER DRIVER INFORMATION STORED IN REGISTRY

In addition to the driver features available in the DEVMODE , the UI module is required to store some other information in the registry.  These features are stored in the registry instead of DEVMODE in order to prevent DEVMODE from becoming unnecessary large and inefficient.  Furthermore, in order to maintain backward compatibility, all currently defined registry keys must be kept.   This also means that  some Postscript and Unidrv registry keys have to be handled separately.  For more details on the handling, see the section 9.0.

### 4.4.1	FONT SUBSTITUTION TABLE

Font substitution table is used to map TrueType font to printer’s device font. Font substitution handling is the same for Postcript and Unidrv devices.  The default table is stored in the UI internal binary structure used for all devices.  Subsequent changes are saved in the registry.  The table consists of  N font mapping entries, each entry is as defined below.  Unidrv and Postscript will use the same registry keyword.  This table is applicable on a per printer instance.

REGISTRY KEY = L”TTFontSubTable”

REGISTRY KEY = L”TTFontSubTableSize”

typedef struct _TTFONT_MAPPING{

WSTR		wstrTTFont[ ];			// TT font name string

WSTR		wstrDevFont[ ] ;			// device font name string

} TTFONT_MAPPING, *PTTFONT_MAPPING;

### 4.4.2	FORM TO TRAY TABLE

Form to tray table is used to specify the current form selected for each paper source available.  The table is stored in the registry in a multi-string format.  The handling of FORM TO TRAY assignment is the same for Postcript and Unidrv devices.  For driver version 4.0 or greater, the following registry keys are used to access the table.  For driver version 3.51 or smaller, the REGKEY_PAPER_SRC is used to find a match for the form selected.

REGISTRY KEY = L”TrayFormTable”

REGISTRY KEY = L”TrayFormTableSize”

UNIDRV

REGISTRY_KEY = REGKEY_PAPER_SRC (NT351)

typedef struct _FORMTOTRAY{

WSTR		wstrTrayName[ ];

WSTR		wstrFormName[ ];

WSTR		wstrPrinterFormName[ ];

WSTR		wstrDefaultTrayFlags[ ];

} FORMTOTRAY, *PFORMTOTRAY;

The following illustrates the format of the form to tray table

FORMTOTRAY FormToTrayTable [ ] {

“Auto”	, 		L“Letter”, 		L“Letter”, 	L“0”	,

“Envelop Feeder, 	L“Envelop Monarch”,	L“Monarch”, 	L“0”	,

……….									,

………									,

};

### 4.4.3  DEVHTINFO

DEVHTINFO is the struct defined in winddi.h for halftoning setup.  Postcript and Unidrv devices use this structure to get/set the halftone options. Handling and defaults for Postcript and Unidrv devices are the same for host halftoning.

REGISTRY KEY = L”CurDevHTInfo”

typedef struct _DEVHTINFO{

DWORD	HTFlags;

DWORD	HTPatternSize;

DWORD	DevPelsDPI;

COLORINFO	ColorInfo;

} DEVHTINFO, *PDEVHTINFO;

### 4.4.4	 MODELNAME

This is used for minidriver’s model name.  This is primarily used to verify the DRIVEREXTRA in the devmode.  The model name is in a REG_SZ format. Checksum should be adequate. Should this be in the registry still??

REG KEY = L”Model”

### 4.4.5   COUNTRY

This is used to store the country code in the registry.  The value stored are defined in SDK.

This should not be a private keyword.  The UI module should read the Country keyword from the

International key. This key will not be used by the UI module.  The UI module will read the country setting from the “intl” section and ‘”country” key.

REG KEY = L”Country”

### 4.4.6  FREE MEMORY

Postscript and Unidrv devices store the amount of memory available in the printer in the registry.

REG KEY = L”FreeMem”

### 4.4.7  FONT CARTRIDGE

This is used by Unidrv only.  The information stored in the registry is a list of font cartridge names.  The format the data in the registry is REG_MULTI_SZ.  The UI module reads from the list of cartridge name until a Unicode NULL is detected.

REG KEY = L”FontCart”

### 4.4.8  UNIDRV FLAGS

This is used by Unidrv only.  The flag stored in the registry is the same flags defined for sFlags in DRIVEREXTRA.  Why does it need to be in two places???

REG KEY = L”RasddFlags”

### 4.4.9   PRINTER HALFTONE

This is used only by Postscript to remember whether host halftoning is enabled.  The format is either TRUE or FALSE.

REG KEY = L”PrinterHT”

### 4.4.10  FORM?

This is used only by Postscript to indicate whether printer forms have been added to the spooler form database.  The format is either TRUE or FALSE.

REG_KEY = L”Forms?”

## 4.5	INTEROPERABILITY

### 4.51	DEVMODE CheckSum == Binary Data File CheckSum

In the case where DEVMODE.dwCheckSum32 matches the UIINFO.dwCheckSum or PRINTERDATA.dwCheckSum32 matches the UIINFO.dwCheckSum, we know that the options [ ] items array is valid.  That means that the options in the array match up with the binary data feature list.

The following figure illustrates the mapping in the case where the checksums match.

In this particular example, once the user made the paper size selection to something else, let’s say Legal, the UI module will go to the binary data structure and get the predefinedID for paper size option 1.  DEVMODE.dmPaper will be updated to DMPAPER_LEGAL.

### 4.52	DEVMODE CheckSum != Binary Data File CheckSum

In the case where DEVMODE.dwCheckSum32 doesn’t match with UIINFO.dwCheckSum or PRINTERDATA.dwCheckSum32 doesn’t match with UIINFO.dwCheckSum, we know that the options [ ] items array is not valid and must grovel through the binary data file to update the options [ ] list.

In this case, the UI module must have support for translating public devmode settings to options list.  For example, given a public devmode ID for media type, the UI module goes through the option list for GID_MEDIATYPE feature and find the matching ID.  Once the matching ID is found, the return index is the option selected.  The option list is rebuild similarly for other features.  For features that don’t have corresponding public devmode fields, the default option is selected.

The following figure illustrates the mismatch of options [ ] list with binary data structure.

In the example shown above, the options [ ] would have to be rebuilt completely.  The UI module will use the public devmode fields to set the right option selection.  For options with no corresponding public devmode fields, the feature default option will be used.

### 4.5.3	CLIENT/SERVER SCENARIOS

The following table illustrates the possible interoperability scenario for DEVMODE conversion and validation.  In general, driver version should always be bumped for new releases.  If the version is no incremented, there is a possibility that the driver will be overwritten in a client/server scenario.

Client : 5.0 ; Server: 5.0

| Printing |  | Remote Admin |
| --- | --- | --- |
| EMF printing is OK RAW printing is OK | OK Client/Server drivers are the same. |  |

Client 5.0; Server 4.0

| Printing | Remote Admin |
| --- | --- |
| Client has a driver: - RAW printing is OK. - EMF printing doesn’t work. - Client/Server drivers are different. Client does not have a driver: - RAW printing is OK. - EMF printing is OK | - Client has no driver, Server driver is copied down. Remote Admin is OK because drivers are the same on client/server. - Client has driver, and the drivers don’t match, Remote Admin doesn’t work. The DRIVEREXTRA devmode settings are not preserved on the server side. |

Client 4.0; Server 5.0

| Printing |  | Remote Admin |
| --- | --- | --- |
| EMF printing is OK RAW printing is OK | Remote Admin is OK if the driver version are different. The Server driver is copied down to the client. Client/Server drivers are the same. |  |

Client 5.0; Server 3.51

| Printing |  | Remote Admin |
| --- | --- | --- |
| RAW printing is OK (There is no EMF printing) | Remote Admin doesn’t work. The DRIVEREXTRA devmode settings are not preserved. Client/Server drivers are different |  |

Client 3.51; Server 5.0

| Printing |  | Remote Admin |
| --- | --- | --- |
| RAW printing is OK (There is no EMF printing) | Remote Admin doesn’t work. The DRIVEREXTRA devmode settings are not preserved. Client/Server drivers are different. |  |

# 5.  BI-DI SUPPORT

In an ideal scenario, the user should not have to manually go through all the installable options and select the currently installed options and their corresponding defaults.  This is very user un-friendly and leads to all sort of confusions.

In order to support bi-di querrying, the GPD/PPD and the Language Monitor must communicate through a set of predefined identifiers.  These IDs are private among GPD/PPD  and Language Monitor.  The UI module does not any pre-knowledge of these IDs.

For each feature in GPD/PPD file, there are two set of {bidi ID, format of data}, each with a different purpose as described below.  The parser parses this information and initializes the FEATURE structure accordingly.  When it’s time to querry the printer for defaults and installable options, the UI module call GetPrinterData with the bi-di ID and the expected data format.  The spooler will call the language monitor to querry the information and return the information to the UI module in the format it expected.

There are two types of querries required by the UI module.  The first is to enumerate all installed options for a feature (example: all paper tray installed for paper source feature).  The second is to find the default option of a feature (example: upper tray for paper source).  In the first scenario, if the buffer returned is NULL, that means the feature is not available.  Otherwise, the buffer contains all the options installed for the particular feature.  In the second scenario, the default option is returned to the buffer.

# 6.	CUSTOM UI SUPPORT

Custom UI support is describes in NTGDI\PRINTERS\SPECS\CUSTOMUI.DOC

# 7.	PROPERTY SHEET UI SUPPORT

Currently, we are using the tree view model to represent the user interface.  However, we want to extend this to support the property sheet model also.  The followings are necessary in order to support property sheet driven UI.

- Templates for Standard User Interface (dialog resources)

- COMPSTUI allows dynamic property sheet pages to be added to the property sheet list.

- COMPSTUI exposing the interfaces to specify where to draw the item, the type of items (COMBOBOX, RADIOBUTTON etc.).

- Handling of the items will be through callbacks ( i.e. current implementation)

# 8.	EXTENDED FEATURES FOR NT 5.0

The followings are items that are being considerd for NT 5.0 user interface.

FEATURE						REQUIREMENTS

Reverse Page Ordering					Spooler/Print Processor support.

Halftoning Option					UI module, Render driver module.

(error diffusion, large pattern dithering,

OEM custom UI )

Document Type Option					UI module, Render Driver module.

(Drawing, Text, Photographs, Presentation)		PPD/GPD new keyword.

Watermarks (SW simulated)				UI module, Spooler/Print Processor

support.

Power Saver						UI module, Render Driver module.

Color matching						UI module, Render Driver module.

OEM icons for Feature					UI module.

OEM logos in Printer Properties sheet			UI module

Duplexing (SW simulation)				UI module, Render Driver module.

(Booklet, Binding options)

Rotation(90 or 270 rotation when printing in landscape)	UI module, Render Driver module.

Task bar containing explanation of the option selected.	UI module

Constrainst						UI module

# 9.	IMPLEMENTATION DETAILS

The following sections discuss the different code path necessary to handle the differences between Unidrv and PostScript.

## 9.1  Caching the Binary Data File

The binary data file is memory mapped.  However, to increase speed of accessing data, the binary data file will be cached in memory.

When the file is memory mapped, it returns a handle to the mapped file, a pointer to the starting address of the file and a size of the file.  Memory from the local heap is allocated and the binary data file is copied to allocated memory.

## 9.2  Devmode Handling

Due to the differences between Unidrv and Postscript DRIVEREXTRA devmode, the following functions are necessary in the UI module.

BOOL GetDefaultDevmode (

PEXTDEVMODE pEDM, 		\\ pointer to EXTDEVMODE

PWSTR pDeviceName, 		\\ pointer to Device Name

PINFOHEADER pInfoHeader,		\\ pointer to INFOHEADER

BOOL bMetric				\\ Use Metric system if TRUE

);

This function gets the defaults of public and private devmodes and put them in pEDM.  UIINFO struct will be available via offset in pInfoHeader.  The function set the public fields according to the defaults.  Then it checks the driver signature in UIINFO in order to determine which private devmode function it should call.  If the driver signature is DRIVER_SIGNATURE_UNIDRV, it will call GetUnidrvDefaults otherwise it will call GetPSDefaults to fill out the private devmode.

Returns: TRUE for success, FALSE or failure.

BOOL SetDefaultDevmode (

PEXTDEVMODE pEDMDest, 		\\ pointer to DestEXTDEVMODE

PEXTDEVMODE pEDMSrc,		\\ pointer to Src EXTDEVMODE

PINFOHEADER pInfoHeader		\\ pointer to INFOHEADER

);

This function merges the devmode settings from the Src devmode with the destination devmode.  The public part of EXTDEVMODE is merged then the driver signature is checked.  This function will call either SetDefaultUnidrv or SetDefaultPS depending on the driver signature to merge the private devmode.

Returns: TRUE for success, FALSE or failure.

BOOL ValidateDevmode (

PEXTDEVMODE pEDM

);

This function checks whether the devmode passed in is valid for the device.  It validates the devmode based on the checksum32 value in private devmode, this checksum should match with the checksum in the binary data file.  There is no need for separate Unidrv and PostScript validation code.

Returns: TRUE for success, FALSE or failure.

BOOL GetPSDefault (

PDRIVEREXTRA pDMDX, 		\\ pointer to DRIVEREXTRA

PWSTR pDeviceName, 			\\ pointer to Device Name

PINFOHEADER pInfoHeader,		\\ pointer to INFOHEADER

BOOL bMetric				\\ Use Metric system if TRUE

);

This function get the default private Postscript devmode.

Returns: TRUE for success, FALSE or failure.

BOOL GetUnidrvDefaults (

PDRIVEREXTRA pDMDX, 		\\ pointer to DRIVEREXTRA

PWSTR pDeviceName, 			\\ pointer to Device Name

PINFOHEADER pInfoHeader,		\\ pointer to INFOHEADER

BOOL bMetric				\\ Use Metric system if TRUE

);

This function gets the default private Unidrv devmode.

Returns: TRUE for success, FALSE or failure.

BOOL SetPSDefault  (

PDRIVEREXTRA pDXDest, 		\\ pointer to Dest. DRIVEREXTRA

PDRIVEREXTRA pDXSrc,		\\ pointer to Src DRIVEREXTRA

PINFOHEADER pInfoHeader		\\ pointer to INFOHEADER

);

This function merges the Postcript destination private devmode with the src private devmode.

Returns: TRUE for success, FALSE or failure.

SetUnidrvDefaults  (

PDRIVEREXTRA pDXDest, 		\\ pointer to Dest. DRIVEREXTRA

PDRIVEREXTRA pDXSrc,		\\ pointer to Src DRIVEREXTRA

PINFOHEADER pInfoHeader		\\ pointer to INFOHEADER

);

This funtion merges the Unidrv destination private devmode with the src private devmode.

Returns: TRUE for success, FALSE or failure.

## 9.3 DeviceCapabilities

DeviceCapabilities - All DC_XXX querrying should be the same for Unidrv and Postscript with the following exceptions:

DC_ORIENTATION - Postscript reads the angle out of dmDriverExtra->dwFlags and Unidrv reads it out of the binary data structure through GID_ORIENTATION.

## 9.4  HANDLING OF PRINTERDATA/INFORMATION IN THE REGISTRY

### 9.4.1  PRINTER STICKY FEATURES

Handling of printer sticky information is the same for Postscript and Unidrv.  Both devices would use the OPTSELECT array in PRINTERDATA to display the printer sticky features and currently selected options.

### 9.4.2  OTHER INFORMATION IN REGISTRY

Handling of other information in the registry can varies.  The followings outlined three lists, one common to both Unidrv and Postscript, one that is Unidrv specific and the other is Postscript specific.

COMMON

Font Substitution Table

Form To Tray Table

Host Halftoning Info

Free Memory

UNIDRV

ModelName

Country

Font Cartridge

Rasdd Flags

POSTSCRIPT

Printer Halftone

Forms?

# 10.  OPEN ISSUES

- Compstui.dll can tell the UI module which items are expanded/contracted.  Need new OPTITEM flag.

- Compstui.dll can tell the UI module which items are in the window view.  Need new OPTITEM flag.

- Constrainst/Invalid combination.  The UI module must allow the user to save invalid combination and constrainst in the DEVMODE.  The UI module only need to notify the user of the constrainst but all combination of settings are saveable in devmode.  This only applies to INSTALLABLE UI constrainst and invalid combination.  The UI must allow this because formatting of the document should not rely on the current printer selected.

- Print Text as Graphics and Rules are not planned as UI options for NT 5.0.  These features could still be supported but will be hidden from the users.